ICE INFORMATION

FOR ELECTRONIC NAVIGATION CHARTS (ENC)

data file naming convention and structure

Version 01.12.2009

Content:

File naming convention	2

Table 1 – Naming conventions for ice chart purpose	2

Table 2 – Naming conventions for Region, Month, Day, Hour and Forcastal time of ice chart	3

Data file structure	4

# File naming convention

Ice chart data file naming convention in S-57 format should be based on the following scheme:

SSPRMDHF.000, where

SS – code of issuing service (TBD in cooperation with the OGC);

(P)urpose in accordance with table 1;

(R)egion, (M)onth, (D)ay, (H)our and (F)orcastal time of ice chart in accordance with table 2.

## Table 1 – Naming conventions for ice chart purpose

| Symbol |  | ice chart specification | Type of chart |
| --- | --- | --- | --- |
| in file name | code |  |  |
| V | 100 | overview | diagnostic ice chart |
| D | 101 | detailed |  |
| S | 102 | special |  |
| C | 103 | concentration | prognostic ice chart |
| H | 104 | hummocking |  |
| P | 105 | compacting |  |
| W | 106 | waves height of 3% significance | prognostic hydrometeorological chart |
| U | 107 | waves predominant direction |  |
| L | 108 | sea level denivelation |  |
| A | 109 | Icing (ice accretion) |  |

## Table 2 – Naming conventions for Region, Month, Day, Hour and Forcastal time of ice chart

| Symbol | R (region) | M (month) | D (day) | H (hour) | F (forecastal time in hours) |
| --- | --- | --- | --- | --- | --- |
| 1 | Baltic Sea | 1 | 1 | 00 | 00 |
| 2 | Northern Sea | 2 | 2 | 01 | 03 |
| 3 | Greenland Sea | 3 | 3 | 02 | 06 |
| 4 | Norwegian Sea | 4 | 4 | 03 | 09 |
| 5 | Barents Sea | 5 | 5 | 04 | 12 |
| 6 | White Sea | 6 | 6 | 05 | 15 |
| 7 | Kara Sea | 7 | 7 | 06 | 18 |
| 8 | Laptev Sea | 8 | 8 | 07 | 21 |
| 9 | East Siberian Sea | 9 | 9 | 08 | 24 |
| 0 | Chukchi Sea | 10 | 10 | 09 | 48 |
| A | Beaufort Sea | 11 | 11 | 10 | 72 |
| B | Canadian Archipelago | 12 | 12 | 11 | 96 |
| C | Baffin Sea |  | 13 | 12 | 120 |
| D | Devis-Labrador |  | 14 | 13 |  |
| E | Hudson Bay |  | 15 | 14 |  |
| F | St.Lawrence Gulf |  | 16 | 15 |  |
| G | Great Lakes |  | 17 | 16 |  |
| H | Arctic Basin |  | 18 | 17 |  |
| I | Black - Azov Seas |  | 19 | 18 |  |
| J | Caspian Sea |  | 20 | 19 |  |
| K | Japan Sea |  | 21 | 20 |  |
| L | Okhotsk Sea |  | 22 | 21 |  |
| M | Bering Sea - Alaska Gulf |  | 23 | 22 |  |
| N | Weddell Sea |  | 24 | 23 |  |
| O | Atlantic sector (of Antarctic) |  | 25 |  |  |
| P | Indian sector (of Antarctic) |  | 26 |  |  |
| Q | Pacific sector (of Antarctic) |  | 27 |  |  |
| R | Ross Sea |  | 28 |  |  |
| S | Amundsen Sea |  | 29 |  |  |
| T | Bellingsgauzen Sea |  | 30 |  |  |
| U | Drake Strait |  | 31 |  |  |
| V | Western Antarctic sector |  |  |  |  |
| W | Eastern Antarctic sector |  |  |  |  |
| X-Z | optional |  |  |  |  |

Note: Region indexes V - Z may be used by the issuing services for preparation of the products for areas not specified by A - U or for lower order and mixed type areas

# Data file structure

DDR

| Leader |  |
| --- | --- |
| Directory |  |
| Field Area | Fields enumeration |
|  | Detailed description of fields |

DR 1

| Leader |  |
| --- | --- |
| Directory |  |
| Field Area | Record number (1) (0) |
|  | DSID |
|  | DSSI – number of objects |

DR 2

| Leader |  |
| --- | --- |
| Directory |  |
| Field Area | Record number (2) (0) |
|  | DSPM |

DR 3 – Enumeration of all nodes (points)

| Leader |  |
| --- | --- |
| Directory |  |
| Field Area | Record number |
|  | VRID |
|  | SG2D – single coordinate ( Y – X ) |

DR 4 – Enumeration of all ice zones (polygones)

| Leader |  |
| --- | --- |
| Directory |  |
| Field Area | Record number |
|  | VRID |
|  | VRPT – pointer to initial (parent) node of the polygone |
|  | VRPT – pointer to final node of the polygone |
|  | SG2D – enumeration of coordinates ( Y – X ) |

DR 5 – Description of all ice zones (polygones)

| Leader |  |
| --- | --- |
| Directory |  |
| Field Area | Record number |
|  | FRID – class of ice object |
|  | ATTF – enumeration of attributes and their values |
|  | FSPT – internal or external polygone |

For DDR

0001  DSID  DSSI

0001  DSPM

0001  VRID  VRPT  SG2D

0001  FRID  ATTF  FSPT

For  DSID

[ EXPP ] = {1} – always new data set

[ INTU ] – (Intended usage) – Navigational purpose is

| 100 | V | overview | Diagnostic ice charts |
| --- | --- | --- | --- |
| 101 | D | detailed |  |
| 102 | S | special |  |
| 103 | C | concentration | Prognostic ice charts |
| 104 | H | hummocking |  |
| 105 | P | compacting |  |

[ PRSP ] – (Product Specification) – {100} – Ice Chart

[ PROF ] – (Application profile identification):

| 100 | Diagnostic ice chart |
| --- | --- |
| 101 | Prognostic ice chart |