JCOMM

Expert Team on Sea Ice

Electronic Chart Systems

Ice Objects Catalogue

Version 5.0.2

(DRAFT)

February 20, 2010

Not Yet Approved

Prepared by

Arctic and Antarctic Research Institute

38, Bering Street

St. Petersburg 199397

Russian Federation

Table of Contents

Record of Changes	iv

1.	Introduction	1

2.	ECDIS/ENC BACKGROUND	2

3.	APPROACH TO HARMONIZATION	3

4.	ICE OBJECT CLASSES	4

Notes to the draft of Version 5.0	4

5.	ICE OBJECT SUMMARY:	5

Ice Object Class:	 Sea Ice	6

Ice Object Class:	 Lake Ice	7

Ice Object Class:	 Iceberg Area	8

Ice Object Class:	 Fast Ice	9

Ice Object Class:	 Separate Giant Floe	10

Ice Object Class:	 Ice Edge	11

Ice Object Class:	 Iceberg Limit	12

Ice Object Class:	 Limit of Open Water	13

Ice Object Class:	 Limit of All Known Ice	14

Ice Object Class:	 Line of Ice Ridge	15

Ice Object Class:	 Line of Ice Lead	16

Ice Object Class:	 Line of Ice Fracture	17

Ice Object Class:	 Line of Ice Crack	18

Ice Object Class:	 Ice Compacting	19

Ice Object Class:	 Ice Lead	20

Ice Object Class:	 Iceberg	21

Ice Object Class:	 Floeberg	22

Ice Object Class:	 Ice Thickness	23

Ice Object Class:	 Ice Shear	24

Ice Object Class:	 Ice Divergence	25

Ice Object Class:	 Ice Ridge/Hummock	26

Ice Object Class:	 Ice Keel/Bummock	27

Ice Object Class:	 Ice Drift	28

Ice Object Class:	 Ice Fracture	29

Ice Object Class:	 Ice Rafting	30

Ice Object Class:	 Jammed Brash Barrier	31

Ice Object Class:	 Stage of Melt	32

Ice Object Class:	 Snow Cover	33

Ice Object Class:	 Strips and Patches	34

Ice Object Class:	 Grounded Hummock	35

6.	FEATURE ATTRIBUTES FOR ICE OBJECTS:	36

6.1.	ICE FEATURE ATTRIBUTE SUMMARY:	36

Ice Attribute:	 Total Concentration	38

Ice Attribute:	 Partial Concentration	40

Ice Attribute:	 Ice Stage of Development	42

Ice Attribute:	 Lake Ice Stage of Development	44

Ice Attribute:	 Floe Sizes	46

Ice Attribute:	 Melt Stage	48

Ice Attribute:	 Concentration of Strips and Patches	50

Ice Attribute:	 Number of Icebergs in Area	51

Ice Attribute:	 Level Ice	52

Ice Attribute:	 Compacting Strength	53

Ice Attribute:	 Ice Fracture Type	54

Ice Attribute:	 Ice Lead Status	55

Ice Attribute:	 Frequency of Leads or Fractures	56

Ice Attribute: 	 Orientation of Leads or Fractures	57

Ice Attribute:	 Ice Lead (or Fracture or Crack) Width	58

Ice Attribute:	 Ice Location Information	59

Ice Attribute:	 Iceberg Size	60

Ice Attribute:	 Ice Drift Direction	61

Ice Attribute:	 Ice Drift Speed	62

Ice Attribute:	 Ice Average Thickness	63

Ice Attribute:	 Maximum Ice Thickness	64

Ice Attribute:	 Minimum Ice Thickness	65

Ice Attribute:	 Ice Thickness Type	66

Ice Attribute:	 Snow Depth	67

Ice Attribute:	 Snow Cover Concentration	68

Ice Attribute:	 Direction of Sastrugi	69

Ice Attribute:	 Ice Ridge Concentration	70

Ice Attribute:	 Ice Ridge Classification	72

Ice Attribute:	 Ice Ridge Mean Height	73

Ice Attribute:	 Ice Ridge Frequency	74

Ice Attribute:	 Ice Ridge Maximum Height	75

Ice Attribute:	 Ice Keel Concentration	76

Ice Attribute:	 Ice Keel Frequency	77

Ice Attribute:	 Ice Keel Mean Depth	78

Ice Attribute:	 Ice Keel Maximum Depth	79

Ice Attribute:	 Ice Rafting Concentration	80

Ice Attribute:	 Brash Ice	82

Ice Attribute:	 Combination Ice Stage of Development and Floe Size for the 1st partial concentration	84

Ice Attribute:	 Combination Ice Stage of Development and Floe Size for the 2nd partial concentration	86

Ice Attribute:	 Combination Ice Stage of Development and Floe Size for the 3rd partial concentration	88

Ice Attribute:	 Ice Breccia for the first partial concentration	90

Ice Attribute:	 Ice Breccia for the second partial concentration	92

Ice Attribute:	 Ice Breccia for the third partial concentration	94

Ice Attribute:	 Snow cover	96

Ice Attribute:	 Stage of Melting	97

Ice Attribute:	 Contamination	98

Ice Attribute:	 Concentration of Hills	99

Ice Attribute:	 Fractures Concentration	100

Ice Attribute:	 Iceberg Concentration	101

Ice Attribute:	 Iceberg Form	102

Ice Attribute:	 Maximum Height of Iceberg Topside	103

Ice Attribute:	 Number of Ice Objects	104

Ice Attribute:	 Maximum Width of Ice Lead (or Fracture or Crack)	105

Ice Attribute:	 Minimum Width of Ice Lead (or Fracture or Crack)	106

Appendix A	108

Attribute Types	108

Appendix B	109

List of Non-ice S57 Attributes Referred to by Ice Objects	109

# Record of Changes

Version 4.0	Initial version approved 30 March 2007

Version 4.1	Draft

- S-57 Marine Ice Object (MOI) numeric codes have been assigned to all sea ice objects and attributes

- Amended definition of Code 91 for ICEACT and ICEAPC to include “9+/10” ice concentration

- Added Brash Ice Code 70 to Ice Objects ICESOD and ICESLO

- Added Ice Attribute ICEBRS

- Minor typographical and formatting errors fixed

- Amended references to more accurate citations

- “Remarks” updated for most ice features to indicate specific changes from Version 4.0

Version 5.0	Draft

- New Objects and Attributes added

- Various code tables amended for greater internal consistency

- Some attribute types changed from floating point to integer

ECDIS Ice Objects

# Introduction

A number of northern nations, particularly, Canada, the United States, Germany and those bordering on the Baltic Sea, maintain Ice Services, and issue ice charts on a regular basis during winter months. These ice charts are used on ships as an aid to navigation in ice infested waters, and as ECDIS becomes more widely available on ships navigating these northern waters, it will be important to provide ice data in a form that can be used in those systems.

Several workshops have been held in order to investigate how ice objects should be handled for ECDIS, An initial meeting held in Ottawa on June 26-29, 1995, addressed many of the potential issues. For that first meeting, a report was prepared by SevenC’s, a German ECDIS company, which proposed a potential set of new S-57 compliant ice objects. A second report was subsequently prepared by IDON Corporation, which included a review of the level of ice object support in the existing S-57 version 2, and the DIGEST Standard Feature and Attribute Catalogue (FACC), Version 1.2. This report was reviewed by the ice experts from those nations present at the initial meeting (Germany, the United States and Canada), following which IDON Corporation formatted the ice objects into the form required for S-57 and produced Version 1.0 of the Ice Object Catalogue.

A second workshop held in Hamburg, May 21-23, 1996 included participation of ice experts from most of the Baltic nations as well as the US, Canada, and Argentina. This group reviewed and approved the Ice Object Catalogue with minor changes that were incorporated into Version 2.0 of the specifications.

SevenC’s proposed further modifications to the ice objects at a third workshop held in St. John’s, Newfoundland, June 5, 6th, 2000. Terraqueous Technologies and IDON Technologies integrated these recommendations into Version 3.0.

Little work has been done on the Ice Objects Catalogue itself since the release of Version 3.0 in June, 2000. However, Dr. Paul Birkel, of the MITRE Corporation in the U.S., performed an analysis of the catalogue and identified specific issues and internal consistencies that should be addressed. Additionally, within the International Hydrographic Organization (IHO), the Digital Geospatial Information Working Group (DGIWG) has set up an ice register within the Feature and Attributes (FAD) Registry. The Expert Team on Sea Ice (ETSI) of JCOMM/WMO has been named “register owner”.

The Ice Register was presented to the 2005 meeting in Ottawa of the International Ice Charting Working Group (IICWG). At that meeting, it was recommended that a review by ice experts was needed to ensure harmonization with present ice community standards, including WMO Nomenclature and Symbology, and SIGRID-3 data transfer format. The U.S. National Ice Center took that action. The NIC presented the results of that review to the 2006 meeting of the IICWG in Helsinki, Finland. At the Helsinki meeting, an action was taken to prepare a new draft of the ECDIS Ice Objects for submission to the WMO and the IICWG. The Canadian Ice Service (CIS) took on this review, which has resulted in this updated Ice Object Catalogue – Version 4. Version 4 will be presented to the next meeting of ETSI in March, 2007.

Subsequent to adoption at the meeting of the Expert Team on Sea Ice (ETSI) in March, 2007 and final approval by the International Hydrographic Organization (IHO), Version 4.0 was officially released. Since that time, S-57 Marine Ice Object (MOI) numeric codes have been assigned to all sea ice objects and attributes, using the numeric range approved at the 4th meeting of the IHO-IEC Harmonizing Working Group on Marine Information Overlays (HGMIO4) in May, 2007.

# ECDIS/ENC BACKGROUND

An Electronic Navigational Chart (ENC) provides the data component of an Electronic Chart Display Information System (ECDIS), which is under review in many countries as an approved aid to navigation, and as the legal equivalent of a paper chart. However, a paper chart is not the only aid to navigation and in ice infested waters, Ice Charts are also used. It is therefore a natural extension to the ECDIS to incorporate ice data.

The presentation of ice data on an ECDIS is not necessarily a simple matter. One major difficulty is the potential clutter that the addition of ice data could cause on an ECDIS display. It may be that a chart display system can only operate as a true ECDIS without the ice information, and that the inclusion of an additional thematic group (layer) of ice information would downgrade an ECDIS to the lower status of an Electronic Chart System (ECS). However, it is still important for ice information to be fully compatible with an ECDIS system, because in certain waters it is vital for safety to be able to see the ice conditions integrated with the navigational chart data. This is an issue that requires further discussion by the ECDIS Colours and Symbols Working Group.

The second issue is the management of the frequent updates required for ice data. However, in order to begin to understand the scope of the problem, it is necessary to identify what are the potential ice objects and how they would be expressed in the S-57 standard.

The International Hydrographic Organization (IHO) has approved version 3.0 of the S-57 exchange standard for hydrographic information, with a minor upgrade to version 3.1 in November, 2000. This exchange standard consists of three primary parts. The first is the IHO S-57 Catalogue of Objects and Attributes. The second component is the specification of the S57 Exchange Format, and the third component is the Product Specification for Electronic Navigational Charts (ENCs). These three components, together with a mechanism for dynamic update, define an Electronic Navigational Chart Product. By designing the ice objects in a manner compatible with S-57, it should be possible to make use of the update mechanism designed for ECDIS for ice information.

The International Hydrographic Organization (IHO) and the Digital Geographic Information Working Group DGIWG, have worked on the harmonization of the DIGEST standard and the S-57 standard, and although the specification of ice objects is also of interest to the DGIWG the DIGEST Feature and Attribute Catalogue Committee (FACC) felt that the subject of ice objects was outside of their scope of interest.

The International Organization for Standardization (ISO) Technical Committee on Geographic Information and Geomatics is also examining object/feature coding. This standard (ISO 19110) is now at the draft international standard (DIS) stage. It establishes the method of describing an object catalogue such as the S-57 Object Catalogue in a universal manner. The IHO Transfer Standard Maintenance and Application Development Working Group (TSMADWG) is considering following this standard in future revisions of the IHO S-57 standard.

Finally, TSMADWG is presently developing the next IHO digital data transfer standard, to be called Special Publication S-100. Since this new standard is not yet finalized, the Features and Attributes described in this document are based upon S-57 Version 3.1.

# APPROACH TO HARMONIZATION

The Features and Attributes described in this document have, to the extent possible, been harmonized with the following ice community standards:

- "WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989 which includes:

  - Sea-Ice Nomenclature, Suppl. No. 5; and,

  - International System of Sea Ice Symbols, Suppl. No. 4

- “SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

WMO standards are quite outdated, and are internally inconsistent. The Nomenclature provides descriptive definitions designed to aid in visual observation of ice information. The System of Sea Ice Symbols was designed to standardize symbology that appears on ice charts which, at the time of publication, were primary produced in hardcopy. The main element of the WMO symbolization for ice information is the "Egg Code". Attributes are described in the form of an egg-shaped (oval) symbol that contains four horizontal rows to express up to 12 numerical values for different ice parameters. Other symbols are also included, to represent ice parameters at a particular location, either precisely or in the near vicinity.

SIGRID-3 is the vector data exchange format for ice chart information among ice services and with the Global Digital Sea Ice Data Banks for archival information. Polygons and associated attribute codes are supported, but line and point information is not. Thus, SIGRID-3 equates well to S-57 “area” Feature Objects, but not to “line” and “point” Feature Objects. However, since SIGRID-3 is a more recently developed standard than WMO, its attribute codes better represent the ice information (e.g.: ranges of concentrations, strips and patches) within the “Egg Code” used on International ice charts.

Thus, the harmonization of the Ice Objects Catalogue with WMO and SIGRID-3 standards, which resulted in this document, used the following approach:

- For the most part, WMO Symbology was used as a basis to develop S-57 Ice Feature Objects. This will enable Internationally-accepted symbology to be used for any ECDIS/ENC-displayed ice products.

- Because of internal inconsistencies within WMO ice symbol tables, and in order to support “Strips and Patches”, SIGRID-3 attribute codes are used as a basis for those Attributes related to the standard “Egg Code”.

- Since SIGRID-3 attribute codes do not support them, this version of the Ice Objects Catalogue does not support the “Double-Egg” sub-attributes which would be required with the Attribute “Strips and Patches” within the SEAICE and LACICE “area” Objects.

- For simplicity, Ice Feature Objects are defined as “area”, “point” or “line” only. For those point symbols in WMO Symbology that relate to either a “Specific Location”, or a “Presence in the Area”, a new “Ice Location” Attribute is defined.

- Neither WMO nor SIGRID-3 support line-type Objects. Additionally, there is no support for Iceberg information products produced by IIP or CIS, or for “Stage of Development” Attributes for Lake Ice (LACICE). These particular Ice Feature Objects have thus been harmonized with “MANICE – Manual of Standards Procedures for Observing and Reporting Ice Conditions”, 8th Edition, 1984, Canadian Ice Centre, Ottawa, Canada.

- During this harmonization process, some of the Attributes have been moved up to become Ice Feature Objects, so that the associated WMO attributes can be better accommodated without having to define “sub-attributes”. These include: Ice Fracture, Ice Compacting, Snow Cover, Stage of Melt, etc.

- Many of the Attributes associated with “point” Ice Feature Objects have also been added as Attributes to the SEAICE and LACICE “area” Feature Objects. This is done to provide S-57 support to future map-type/polygon-based ice information products, such as an ice lead product, ice drift products, etc.

- The following "regional" item has been included: "ICELVL (Level ice)" as an attribute to the SEAICE and LACICE objects. This is supported by WMO Symbology for use in the Baltic, for hatching or colouring of ice charts.

- Through discussion with Dr. Paul Birkel of The MITRE Corporation, and with Paul Seymour of NIC, Dr. Birkel’s comments from his analysis of October, 2003, have been included into this version of the Ice Objects Catalogue.

# ICE OBJECT CLASSES

The proposed Ice Feature Object Classes are described in accordance with the format specified in:

  - “IHO Transfer Standard for Digital Hydrographic Data”, Special Publication No. 57,  International Hydrographic Organization, Monaco, Edition 3.1 – Appendix A, IHO Object Catalogue, November 2000.

All Objects are of type “geo”, meaning “carries the descriptive characteristics of a real world entity.

# Notes to the draft of Version 5.0

- Objects and attributes that are not amended are not marked

- Amended objects are marked in blue

- New attributes for particular objects are marked in yellow

- New objects and attributes are marked in green

Some fields in this structure are absent due to different causes:

- Field ATTV is absent because we have no any attributes for vector data of ice charts

- Field SG3D is absent because we have only planar graph

- Field FOID is absent because we don’t use a long name and there are no any relationships between the Feature records

- Field NATF is absent because we don’t use any special attributes

- Field FFPT is absent because we have no necessity to define relationships between the Feature records

# ICE OBJECT SUMMARY:

| Ice Object Class | Acronym | Code |
| --- | --- | --- |
| Polygon |  |  |
| Sea Ice | SEAICE | 30 300 |
| Lake Ice | LACICE | 30 301 |
| Iceberg Area | BRGARE | 30 302 |
| Fast Ice | I_FAST | 30 303 |
| Separate Giant Floe | I_FLOE | 30 304 |
| Polyline |  |  |
| Ice Edge | ICELNE | 30 320 |
| Iceberg Limit | BRGLNE | 30 321 |
| Limit of Open Water | OPNLNE | 30 322 |
| Limit of All Known Ice | LKILNE | 30 323 |
| Line of Ice Ridge | I_RIDG | 30 324 |
| Line of Ice Lead | I_LEAD | 30 325 |
| Line of Ice Fracture | I_FRAL | 30 326 |
| Line of Ice Crack | I_CRAC | 30 327 |
| Point |  |  |
| Ice Compacting | ICECOM | 30 350 |
| Ice Lead | ICELEA | 30 351 |
| Iceberg | ICEBRG | 30 352 |
| Floeberg | FLOBRG | 30 353 |
| Ice Thickness | ICETHK | 30 354 |
| Ice Shear | ICESHR | 30 355 |
| Ice Divergence | ICEDIV | 30 356 |
| Ice Ridge/Hummock | ICERDG | 30 357 |
| Ice Keel/Bummock | ICEKEL | 30 358 |
| Ice Drift | ICEDFT | 30 359 |
| Ice Fracture | ICEFRA | 30 360 |
| Ice Rafting | ICERFT | 30 361 |
| Jammed Brash Barrier | JMDBRR | 30 362 |
| Stage of Melt | STGMLT | 30 363 |
| Snow Cover | SNWCVR | 30 364 |
| Strips and Patches | STRPTC | 30 365 |
| Grounded Hummock | I_GRHM | 30 366 |

- Ice Object Class:		Sea Ice

Acronym:	SEAICE

Code:	30300

subset ‘Attribute_A’: 	NOBJNM; OBJNAM; ICEACT; ICEAPC; ICESOD; ICEFLZ; ICESPC; ICELVL; ICECST; ICEFTY; ICEDSP; ICEDDR; ICERCN; ICERFQ; ICERMH; ICERXH; ICERDV; ICEKCN, ICEKFQ, ICEKMD, ICEKXD, ICEFCN; ICETCK; ICEMAX; ICEMIN; ICETTY; ICEMLT; ICESCN; ICESCT; ICEDOS; ICELST; ICELFQ; ICELOR;ICELWD

New	IA_SFA; IA_SFB; IA_SFC; IA_FFA; IA_FFB; IA_FFC; IA_RCN; IA_FCN; IA_SNG; IA_MLT; IA_PLG; IA_HLG; IA_CST; IA_DUG; SYMINS; SMINSR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Area

Definition:	Sea Ice is an area at sea that is covered, in whole or in part, with ice.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

Distinction:	LACICE

Remarks:

Change from Version 4.1:	New attributes added

- Ice Object Class:		Lake Ice

Acronym:	LACICE

Code:	30301

subset ‘Attribute_A’: 	NOBJNM; OBJNAM; ICEACT; ICEAPC; ICELSO; ICEFLZ, ICESPC; ICELVL; ICECST; ICEFTY; ICEDSP; ICEDDR; ICERCN; ICERFQ; ICERMH; ICERXH; ICERDV; ICEKCN, ICEKFQ, ICEKMD, ICEKXD, ICEFCN; ICETCK; ICEMAX; ICEMIN; ICETTY; ICEMLT; ICESCN; ICESCT; ICEDOS; ICELST; ICELFQ; ICELOR; ICELWD

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Area

Definition:	Lake Ice is an area on a lake that is covered, in whole or in part, with ice.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3, 4, 2000, St. John’s, Canada.

“Canadian Ice Service MANICE”, 9th edition, June, 2005

Distinction:	SEAICE

Remarks:

Change from Version 4.1:

- Ice Object Class:		Iceberg Area

Acronym:	BRGARE

Code:	30302

subset  ‘Attribute_A’:	NOBJNM; OBJNAM; ICEBNM

New		ICEBSZ; IA_BCN; IA_BFM; IA_BUH; SYMINS; SMINSR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Area

References:	"Workshop on International Standards for Ice Information in ECDIS,"June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3, 4, 2000, St. John’s, Canada.

“Canadian Ice Service MANICE”, 9th edition, June, 2005

Definition:	An Iceberg Area is an area at sea in which icebergs, bergy bits, or growlers are present.

Distinction:

Remarks:

Change from Version 4.1:	New attributes added

- Ice Object Class:		Fast Ice

Acronym:	I_FAST

Code:	30303

subset ‘Attribute_A’:	ICEAPC; ICESOD; IA_RCN; IA_SNG; IA_MLT; IA_PLG; IA_HLG; IA_AVT; IA_MIN; IA_MAX; SYMINS; SMINSR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Area

Definition:	Fast Ice is stationary ice attached to a shore, ice wall or ice front.  It may have icebergs or other specific ice objects embedded in it.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:	This is a new Object

- Ice Object Class:		Separate Giant Floe

Acronym:			I_FLOE

Code:				30304

subset  ‘Attribute_A’:	ICEAPC; ICESOD; IA_RCN; IA_SNG; IA_MLT; IA_PLG; IA_HLG; IA_AVT; IA_MIN; IA_MAX; SYMINS; SMINSR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:		Area

Definition:	A Separate Giant Floe is a giant floe or an ice island which has a precise contour defined.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:	This is a new Object

- Ice Object Class:		Ice Edge

Acronym:	ICELNE

Code:	30320

subset ‘Attribute_A’:		NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	The demarcation at any given time between the open sea and sea ice of any kind and in any concentration, whether fast or drifting.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“Canadian Ice Service MANICE”, 9th edition, June, 2005

Distinction:	BRGLNE, OPNLNE, LKILNE

Remarks:	Note the distinction from OPNLNE

Change from Version 4.1:

- Ice Object Class:		Iceberg Limit

Acronym:	BRGLNE

Code:	30321

subset ‘Attribute_A’:	NOBJNM; OBJNAM;

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

References:	“Canadian Ice Service MANICE”, 9th edition, June, 2005.

Geometric Primitive:	Line

Distinction:	 ICELNE, OPNLNE, LKILNE

Definition:	Limit of all known Icebergs

Remarks:

Change from Version 4.1:

- Ice Object Class:		Limit of Open Water

Acronym:	OPNLNE

Code:	30322

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP:

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	The demarcation at any given time between sea ice and freely navigable water, in which sea ice is present in concentrations less than 1/10.

References:	“Canadian Ice Service MANICE”, 9th edition, June, 2005.

Distinction:	ICELNE, BRGLNE, LKILNE

Remarks:	Note the distinction from ICELNE

Change from Version 4.1:

Ice Object Class:		Limit of All Known Ice

Acronym:	LKILNE

Code:	30323

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP:

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	The limit of all known ice, including both sea ice of any kind and icebergs.

References:	“Canadian Ice Service MANICE”, 9th edition, June, 2005.

Distinction:	ICELNE, BRGLNE, OPNLNE

Remarks:	This line is a key product of the International Ice Patrol (IIP), to support safe navigation in the Northwest Atlantic under the UN Convention on Safety of Life at Sea (SOLAS).

Change from Version 4.1:

Ice Object Class:		Line of Ice Ridge

Acronym:	I_RIDG

Code:	30324

subset ‘Attribute_A’:	ICERDV; IA_RMH; IA_RXH

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP:

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	Line of Ice Ridge is a line or wall of broken ice forced up by pressure processes.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:	This is a new Object.

Ice Object Class:		Line of Ice Lead

Acronym:	I_LEAD

Code:	30325

subset ‘Attribute_A’:	ICESOD;IA_OBN; IA_DVW; IA_DMW; IA_DXW

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP:

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	Line of Ice Lead identifies any passage-way(s) through ice which is (are) navigable by surface vessels.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:	This is a new Object.

Ice Object Class:		Line of Ice Fracture

Acronym:	I_FRAL

Code:	30326

subset ‘Attribute_A’:	ICESOD;IA_OBN; IA_DVW; IA_DMW; IA_DXW

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP:

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	Any break or rupture through the ice cover, or through the single floe, resulting from deformation processes. Length may vary from a few meters to a few kilometers.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:	This is a new Object.

Ice Object Class:		Line of Ice Crack

Acronym:	I_FRAL

Code:	30327

subset ‘Attribute_A’:	ICESOD;IA_OBN; IA_DVW; IA_DMW; IA_DXW

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP:

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	Line of Ice Crack identifies any ice breakup, but no passage-way(s) for surface vessels.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:	This is a new Object.

- Ice Object Class:		Ice Compacting

Acronym:	ICECOM

Code:	30350

subset “Attribute_A’:	NOBJNM; OBJNAM; ICECST

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Pieces of ice are said to be compacting when they are subjected to a converging motion, which increases ice concentration and/or produces stresses which may result in ice deformation.

References:	“International System of Sea-Ice Symbols, WMO No. 259, TP. 145, Supplement No. 4, 1970.”

Distinction:	ICESHR, ICEDFT, ICEDIV

Remarks:

Change from Version 4.1:

- Ice Object Class:		Ice Lead

Acronym:	ICELEA

Code:	30351

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICELOC; ICELST; ICELWD

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Ice Lead identifies any fracture(s) or passage-way(s) through ice which is (are) navigable by surface vessels.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3, 4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEFRA

Remarks:

Change from Version 4.1:

- Ice Object Class:		Iceberg

Acronym:	ICEBRG

Code:	30352

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEBSZ; ICEDSP; ICEDDR;

New	ICEBNM; IA_BFM; IA_BUH

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS, PICREP

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	An Iceberg is a massive piece of ice, greatly varying in shape and showing more than 5 meters above the sea surface which has broken away from a glacier, and which may be afloat or grounded. This Object Class also includes smaller forms of glacial ice, known as “Bergy Bits” and “Growlers”, which are defined by their size Attribute.

References:	"Workshop on International Standards for Ice Information in ECDIS,"June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	FLOBRG

Remarks:

Change from Version 4.1:	Attributes added

- Ice Object Class:		Floeberg

Acronym:	FLOBRG

Code:	30353

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEDSP; ICEDDR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A Floeberg is a massive piece of sea ice composed of a hummock or a group of hummocks, frozen together and separated from any ice surroundings. They typically protrude up to 5 meters above the sea surface.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEBRG

Remarks:

Change from Version 4.1:

- Ice Object Class:		Ice Thickness

Acronym:	ICETHK

Code:	30354

subset ‘Attribute_A”:	NOBJNM; OBJNAM; ICETCK; ICEMAX; ICEMIN; ICETTY

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Ice Thickness provides a measure or estimate of ice thickness.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3, 4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:

- Ice Object Class:		Ice Shear

Acronym:	ICESHR

Code:	30355

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	An area of drift ice is subject to shear when the ice motion varies significantly in the direction normal to the motion, subjecting the ice to rotational forces

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEDFT, ICEDIV, ICECOM

Remarks:

Change from Version 4.1:

- Ice Object Class:		Ice Divergence

Acronym:	ICEDIV

Code:	30356

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Ice fields or floes in an area are subject to diverging or dispersive motion, thus reducing ice concentration and/or relieving stresses in the ice.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEDFT, ICESHR, ICECOM

Remarks:

Change from Version 4.1:

- Ice Object Class:		Ice Ridge/Hummock

Acronym:	ICERDG

Code:	30357

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICERCN; ICERFQ; ICERMH; ICERXH, ICERDV

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	An Ice Ridge is a line or wall of broken ice forced up by pressure.

A Hummock is a hillock of broken ice which has been forced upward by pressure.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEKEL, ICERFT

Remarks:

Change from Version 4.1:

- Ice Object Class:		Ice Keel/Bummock

Acronym:	ICEKEL

Code:	30358

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEKCN; ICEKFQ; ICEKMD; ICEKXD

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	From a submariner’s point of view, a Keel is a downward projecting ridge on the underside of the ice canopy - the counterpart of a Ridge. A Bummock is the counterpart of a hummock on the underside of the ice canopy.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICERDG, ICERFT

Remarks:

Change from Version 4.1:

- Ice Object Class:		Ice Drift

Acronym:	ICEDFT

Code:	30359

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEDSP; ICEDDR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Motion of an ice field or floe as a result of forces such as wind and currents.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICESHR, ICEDIV, ICECOM

Remarks:

Change from Version 4.1:

Ice Object Class:		Ice Fracture

Acronym:	ICEFRA

Code:	30360

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEFTY, ICELOC;

New	IA_OBN; ICESOD; IA_DVW; IA_DMW; IA_DXW

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Any break or rupture through the ice pack, or a single floe, resulting from deformation processes. Length may vary from a few metres to many kilometres.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICELEA

Remarks:

Change from Version 4.1:	Attributes added

- Ice Object Class:		Ice Rafting

Acronym:	ICERFT

Code:	30361

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEFCN

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Pressure processes whereby one piece of ice overrides another.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICERDG, ICEKEL

Remarks:

Change from Version 4.1:

- Ice Object Class:		Jammed Brash Barrier

Acronym:	JMDBRR

Code:	30362

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A strip or narrow belt of new, young or brash ice (usually 100-500 metres wide) formed at the edge of either drift or fast ice.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:

- Ice Object Class:		Stage of Melt

Acronym:	STGMLT

Code:	30363

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEMLT

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A description of the stage of melt of the ice; i.e. whether it has formed puddles on the surface and whether these have frozen.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:

- Ice Object Class:		Snow Cover

Acronym:	SNWCVR

Code:	30364

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICESCN; ICESCT; ICEDOS

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A description of the amount of snow covering the ice.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.1:

- Ice Object Class:		Strips and Patches

Acronym:	STRPTC

Code:	30365

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICESPC

subset’ Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A strip is a long narrow area of floating ice, about 1 kilometre or less in width, usually composed of small fragments detached from the main mass of ice, and run together under the influence of wind, swell or current. If the area of ice becomes more rounded in shape, it is referred to as a patch.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989”

Distinction:

Remarks:

Change from Version 4.1:

- Ice Object Class:		Grounded Hummock

Acronym:	I_GRHM

Code:	30366

subset ‘Attribute_A’:	IA_BUH

subset’ Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Grounded Hummock identifies a hummock formation which is stranded.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989”

Distinction:

Remarks:

Change from Version 4.1:	This is a new Object

# FEATURE ATTRIBUTES FOR ICE OBJECTS:

The proposed Attributes for Ice Objects are described in accordance with the format specified in:

  - “IHO Transfer Standard for Digital Hydrographic Data”, Special Publication No. 57,  International Hydrographic Organization, Monaco, Edition 3.1 – Appendix A, Chapter 2 - Attributes, November 2000.

All Attributes are intended to be of type “feature”, meaning “carries the description characteristics of a feature”.

## ICE FEATURE ATTRIBUTE SUMMARY:

| Ice Feature Attribute | Acronym | Version |
| --- | --- | --- |
| Ice Attribute Total Concentration | ICEACT | 30 300 |
| Ice Attribute Partial Concentration | ICEAPC | 30 301 |
| Ice Stage of Development | ICESOD | 30 302 |
| Lake Ice Stage of Development | ICELSO | 30 303 |
| Floe Sizes | ICEFLZ | 30 304 |
| Melt Stage | ICEMLT | 30 305 |
| Concentration of Strips and Patches | ICESPC | 30 306 |
| Number of Icebergs in Area | ICEBNM | 30 307 |
| Level Ice | ICELVL | 30 308 |
| Compacting Strength | ICECST | 30 309 |
| Ice Fracture Type | ICEFTY | 30 310 |
| Ice Lead Status | ICELST | 30 311 |
| Frequency of Leads or Fractures | ICELFQ | 30 312 |
| Orientation of Leads or Fractures | ICELOR | 30 313 |
| Ice Lead or Fracture Width | ICELWD | 30 314 |
| Ice Location Information | ICELOC | 30 315 |
| Iceberg Size | ICEBSZ | 30 316 |
| Ice Drift Direction | ICEDDR | 30 317 |
| Ice Drift Speed | ICEDSP | 30 318 |
| Ice Average Thickness | ICETCK | 30 319 |
| Maximum Ice Thickness | ICEMAX | 30 320 |
| Minimum Ice Thickness | ICEMIN | 30 321 |
| Ice Thickness Type | ICETTY | 30 322 |
| Snow Depth | ICESCT | 30 323 |
| Snow Cover Concentration | ICESCN | 30 324 |
| Direction Of Sastrugi | ICEDOS | 30 325 |
| Ice Ridge Concentration | ICERCN | 30 326 |
| Ice Ridge Classification | ICERDV | 30 327 |
| Ice Ridge Mean Height | ICERMH | 30 328 |
| Ice Ridge Frequency | ICERFQ | 30 329 |
| Ice Ridge Maximum Height | ICERXH | 30 330 |
| Ice Keel Concentration | ICEKCN | 30 331 |
| Ice Keel Frequency | ICEKFQ | 30 332 |
| Ice Keel Mean Depth | ICEKMD | 30 333 |
| Ice Keel Maximum Depth | ICEKXD | 30 334 |
| Ice Rafting Concentration | ICEFCN | 30 335 |
| Ice Stage of Development / Ice Form for the 1-st p.c. | IA_SFA | 30 336 |
| Ice Stage of Development / Ice Form for the 2-nd p.c. | IA_SFB | 30 337 |
| Ice Stage of Development / Ice Form for the 3-rd p.c. | IA_SFC | 30 338 |
| Ice Breccia for the 1-st p.c. | IA_FFA | 30 339 |
| Ice Breccia for the 2-nd p.c. | IA_FFB | 30 340 |
| Ice Breccia for the 3-rd p.c. | IA_FFC | 30 341 |
| Hummocks concentration | IA_RCN | 30 342 |
| Rafting | IA_FCN | 30 343 |
| Snow concentration | IA_SNG | 30 344 |
| Stage of melting | IA_MLT | 30 345 |
| Contamination | IA_PLG | 30 346 |
| Hills concentration | IA_HLG | 30 347 |
| Compacting | IA_CST | 30 348 |
| Fractures concentration | IA_DUG | 30 349 |
| Average (prevailing) ice thickness | IA_AVT | 30 350 |
| Max. ice thickness | IA_MAX | 30 351 |
| Min. ice thickness | IA_MIN | 30 352 |
| Icebergs concentration | IA_BCN | 30 353 |
| (Prevailing) iceberg form | IA_BFM | 30 354 |
| Max. height of the above-water part (iceberg / grounded hummock) | IA_BUH | 30 355 |
| Average height of the hummocks | IA_RMH | 30 356 |
| Max. height of the hummocks | IA_RXH | 30 357 |
| Quantity of the ice objects | IA_OBN | 30 358 |
| (Prevailing) width (lead, fracture, crack) | IA_DVW | 30 359 |
| Max. width (lead, fracture, crack) | IA_DXW | 30 360 |
| Min. width (lead, fracture, crack) | IA_DMW | 30 361 |
| The national Russian coloring | SMINSR | 30 362 |
| The international coloring | SYMINS | 192 |

- Ice Attribute:		Total Concentration

Acronym:	ICEACT

Code:	30300

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01	Ice Free

02	Open Water (< 1/10 ice)

03	Bergy Water

10	1/10 ice

12	1/10 to 2/10 ice

13	1/10 to 3/10 ice

20	2/10 ice

23	2/10 to 3/10 ice

24	2/10 to 4/10 ice

30	3/10 ice

34	3/10 to 4/10 ice

35	3/10 to 5/10 ice

40	4/10 ice

45	4/10 to 5/10 ice

46	4/10 to 6/10 ice

50	5/10 ice

56	5/10 to 6/10 ice

57	5/10 to 7/10 ice

60	6/10 ice

67	6/10 to 7/10 ice

68	6/10 to 8/10 ice

70	7/10 ice

78	7/10 to 8/10 ice

79	7/10 to 9/10 ice

80	8/10 ice

81	8/10 to 10/10 ice

89	8/10 to 9/10 ice

90	9/10 ice

91	9/10 to 10/10 or 9+/10 ice

92	10/10 ice

99	Undetermined/Unknown

Definition:	ICEACT specifies the total concentration of ice in an area.  It represents the ratio expressed in tenths describing the total area of the water surface covered by ice as a fraction of the whole area.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989”

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Remarks:	Corresponds to “C” in International System of Sea Ice Symbols.

Corresponds to “CT” in SIGRID-3

Change from Version 4.1:

- Ice Attribute:		Partial Concentration

Acronym:	ICEAPC

Code:	30301

Attribute Type:	List

Expected Input:

ID			Meaning

01	Ice Free

02	Open Water (< 1/10 ice)

03	Bergy Water

10	1/10 ice

12	1/10 to 2/10 ice

13	1/10 to 3/10 ice

20	2/10 ice

23	2/10 to 3/10 ice

24	2/10 to 4/10 ice

30	3/10 ice

34	3/10 to 4/10 ice

35	3/10 to 5/10 ice

40	4/10 ice

45	4/10 to 5/10 ice

46	4/10 to 6/10 ice

50	5/10 ice

56	5/10 to 6/10 ice

57	5/10 to 7/10 ice

60	6/10 ice

67	6/10 to 7/10 ice

68	6/10 to 8/10 ice

70	7/10 ice

78	7/10 to 8/10 ice

79	7/10 to 9/10 ice

80	8/10 ice

81	8/10 to 10/10 ice

89	8/10 to 9/10 ice

90	9/10 ice

91	9/10 to 10/10 or 9+/10 ice

92	10/10 ice

99	Undetermined/Unknown

Definition:	ICEAPC specifies the partial concentrations of ice in an area. (‘Ca, Cb and Cc’).

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Remarks:	Partial concentrations of ice are reported in order of decreasing thickness and are represented as an S-57 List (or repeating) attribute. Values are separated by a comma.

When only one ice type is present the partial concentration shall not be indicated.

Missing values are represented by the absence of any value of the attribute, which in ISO 8211 encoding of S57, would be adjacent commas.

Corresponds to “Ca, Cb, Cc ” in International System of Sea Ice Symbols.

Corresponds to “CA, CB, CC” in SIGRID-3

Change from Version 4.1:

- Ice Attribute:		Ice Stage of Development

Acronym:	ICESOD

Code:	30302

Attribute Type:	List

Expected Input:

ID			Meaning

01	Ice Free

70	Brash Ice

80	No stage of development

81	New Ice  (<10 cm)

82	Nilas Ice Rind  (<10 cm)

83	Young Ice  (10 to <30 cm)

84	Grey Ice (10 to <15 cm)

85	Grey – White Ice  (15 to <30 cm)

86	First Year Ice  (30 to 200 cm)

87	Thin First Year Ice (30 to <70 cm)

88	Thin First Year Ice Stage 1 (30 to <50 cm)

89	Thin First Year Ice Stage 2 (50 to <70 cm)

90	Code not currently assigned

91	Medium First Year Ice  (70 to 120 cm)

92	Code not currently assigned

93	Thick First Year Ice (>120 cm)

94	Residual Ice

95	Old Ice

96	Second Year Ice

97	Multi-Year Ice

98	Glacier Ice (Icebergs)

99	Undetermined/Unknown

Definition:	Ice Stage of Development describe the ages and thicknesses of the ice (‘So,Sa,Sb,Sc and Sd’).

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Distinction:	IA_SFA, IA_SFB, IA_SFC, IA_FFA, IA_FFB, IA_FFC

Remarks:	Ice Stages of Development may be reported as a single enumerated value or as a set of values of thicknesses. The set of values is represented as an S-57 List (or repeating) attribute.

Corresponds to “So, Sa, Sb, Sc, Sd” in International System of Sea Ice Symbols.

Corresponds to “SO, SA, SB, SC, SD” in SIGRID-3

In conformity with the International System of Sea Ice Symbols, Stage of Development is reported in order from the thickest to the thinnest. The following categories are defined:

So – Stage of Development of ice thicker than Sa but having a concentration of less than 1/10.

Sa - Thickest/oldest; Stage of Development of ice concentration Ca.

Sb - Second thickest/oldest; Stage of Development of ice concentration Cb.

Sc - Third thickest/oldest; Stage of Development of ice concentration Cc.

Sd – Stage of Development of any other remaining class.

Missing values are represented by the absence of any value of the attribute, which in ISO 8211 encoding of S57, would be adjacent commas.

Change from Version 4.1:	Code 94 defined

Distinctions added

- Ice Attribute:		Lake Ice Stage of Development

Acronym:	ICELSO

Code:	30303

Attribute Type:	List

Expected Input:

ID			Meaning

01	New Lake Ice  (<5cms)

02	Thin Lake Ice (5-<15cms)

03	Medium Lake Ice (15-<30cms)

04	Thick Lake Ice (30-70cms)

05	Very Thick Lake Ice (>70cms)

70	Brash Ice

99	Undetermined/Unknown

Definition:	Lake Ice Stages of Development describe the ages and thicknesses of lake ice. (‘So,Sa,Sb,Sc and Sd’)

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

“Canadian Ice Service MANICE”, 9th edition, June, 2005

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Distinction:	IA_SFA, IA_SFB, IA_SFC, IA_FFA, IA_FFB, IA_FFC

Remarks:	Lake Ice Stages of Development may be reported as a single enumerated value or as a set of values of thicknesses. The set of values is represented as an S-57 List (or repeating) attribute.

Corresponds to “So, Sa, Sb, Sc, Sd” in International System of Sea Ice Symbols.

Corresponds to “SO, SA, SB, SC, SD” in SIGRID-3

Stage of Development is reported in order from the thickest to the thinnest. The following categories are defined:

So – Stage of Development of ice thicker than Sa but having a concentration of less than 1/10.

Sa - Thickest/oldest; Stage of Development of ice concentration Ca.

Sb - Second thickest/oldest; Stage of Development of ice concentration Cb.

Sc - Third thickest/oldest; Stage of Development of ice concentration Cc.

Sd – Stage of Development of any other remaining class.

Missing values are represented by the absence of any value of the attribute that in ISO 8211 encoding of S57 would be adjacent commas.

Change from Version 4.1:	Distinctions added

- Ice Attribute:		Floe Sizes

Acronym:	ICEFLZ

Code:	30304

Attribute Type:	List

Expected Input:

ID			Meaning

01				Pancake Ice (30 cm to 3m across)

02				Shuga/Small Ice Cake; Brash Ice (<2m across)

03				Ice Cake (<20m across)

04				Small Floe (20 to <100m across)

05				Medium Floe (100 to 500m)

06				Big Floe (500 to <2000m across)

07				Vast Floe (2000 to10000m across)

08				Giant Floe (>10000m across)

09				Fast Ice

10				Growlers, Floebergs or Floebits

11				Icebergs

99				Undetermined/Unknown

Definition: 	Floe Sizes describe the predominate forms of ice floe sizes (Fa, Fb and Fc) corresponding to the ice Stages of Development Sa, Sb and Sc respectively.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.” Ice in ECDIS Workshop,” June 3,4, 2000, St. John’s, Canada.

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

Distinction:	IA_SFA, IA_SFB, IA_SFC, IA_FFA, IA_FFB, IA_FFC

Remarks:	The "Floe Sizes" Attribute indicates the floe size corresponding to the respective stage identified in the Stages of Development Attribute and reported as a single enumerated value or as a set of values represented as an S-57 List (or repeating) attribute.

Corresponds to “Fa, Fb, Fc” in International System of Sea Ice Symbols.

Corresponds to “FA, FB, FC” in SIGRID-3

Missing values are represented by the absence of any value of the attribute that in ISO 8211 encoding of S57 would be adjacent commas.

Change from Version 4.1:	Distinctions added

- Ice Attribute:		Melt Stage

Acronym:	ICEMLT

Attribute Type:	Enumerated

Code:	30305

Expected Input:

ID			Meaning

01	Few Puddles

02	Many Puddles

03	Flooded Ice

04	Few Thaw Holes

05	Many Thaw Holes

06	Dried Ice

07	Rotten Ice

08	Few Frozen Puddles

09	All Puddles Frozen

10	No Melt

99	Undetermined/Unknown

Definitions:	The Stage of Melt describes the stages of the melting ice.

Puddle: 	An accumulation of water on ice, mainly due to the melting of snow, but in some more advanced stages also the melting of ice.

Thaw Hole:	Vertical holes formed in ice when surface puddles melt through to the underlying water.

Dried Ice:	Ice-surface from which water has disappeared after the formation of cracks and thaw holes. During the process of drying the surface whitens.

Rotten Ice:	Ice which has become honey-combed and is in an advanced state of disintegration.

Flooded Ice:	Ice which has been flooded and is heavily loaded by water or water and wet snow.

Frozen Puddle: 	A puddle which has frozen over.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

Remarks:

Change from Version 4.1:

- Ice Attribute:		Concentration of Strips and Patches

Acronym:	ICESPC

Attribute Type:	Enumerated

Code:	30306

Expected Input:

ID			Meaning

11	Strips and Patches (concentrations 1/10)

12	Strips and Patches (concentrations 2/10)

13	Strips and Patches (concentrations 3/10)

14	Strips and Patches (concentrations 4/10)

15	Strips and Patches (concentrations 5/10)

16	Strips and Patches (concentrations 6/10)

17	Strips and Patches (concentrations 7/10)

18	Strips and Patches (concentrations 8/10)

19	Strips and Patches (concentrations 9/10)

20	Strips and Patches (concentrations 10/10)

99	Undetermined/Unknown

Definition:	A strip is a long narrow area of floating ice, about 1 kilometre or less in width, usually composed of small fragments detached from the main mass of ice, and run together under the influence of wind, swell or current. If the area of ice becomes more rounded in shape, it is referred to as a patch. ICESPC indicates the concentration in tenths within the area of Strips and Patches.

References:	“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	The concentration of Strips and Patches are provided exclusive of Floe Size values. When a Strips and Patches value is supplied, Floe Size values are null.

Change from Version 4.1:

- Ice Attribute:		Number of Icebergs in Area

Acronym:	ICEBNM

Attribute Type:	Integer

Code:	30307

Expected Input:	The number of icebergs in an area expressed by a density measurement on the basis of a grid.

Definitions:	 ICEBNM indicates the number of Icebergs within a specified area.

Distinction:	IA_BCN

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Canadian Ice Service MANICE”, 9th edition, June, 2005.

Remarks:	The area is defined by the dimensions of one degree latitude by one degree longitude and the attribute will be limited to 80 degrees North or South due to distortion at the poles.

Although measurements are not taken, areas that contain icebergs also usually contain bergy bits, and growlers.

Change from Version 4.1:	Added distinction

- Ice Attribute:		Level Ice

Acronym:	ICELVL

Code:	30308

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01				Level (undeformed)

02				Deformed

99				Undetermined/Unknown

Definition:	ICELVL is an indication as to whether or not the ice has been affected by deformation

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	Use of this Attribute is a regional practice in the Baltic Sea.

Change from Version 4.1:

- Ice Attribute:		Compacting Strength

Acronym:	ICECST

Attribute Type:	Enumerated

Code:	30309

Expected Input:

| ID | Meaning |
| --- | --- |
| 01 | Little compacting |
| 10 | Slight compacting |
| 12 | Slight to moderate compacting |
| 20 | Moderate compacting |
| 23 | Moderate to strong compacting |
| 30 | Strong compacting |
| 98 | Ice divergence |
| 99 | Undetermined / Unknown |

Definition:	ICECST is an indication of strength of the compacting of the ice.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:	Code table for expected input revised to allow ranges

- Ice Attribute:		Ice Fracture Type

Acronym:	ICEFTY

Attribute Type:	Enumerated

Code:	30310

Expected Input:

ID			Meaning

01			Crack (0 to 1m wide)

02			Very Small Fracture (>1m to 50m wide)

03			Small Fracture (>50m to 200m wide)

04			Medium Fracture (>200m to 500m wide)

05			Large Fracture (>500m wide)

Definition:	ICEFTY indicates the type of fracture, based upon width.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Lead Status

Acronym:	ICELST

Code:	30311

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			Open Lead

02			Frozen Lead

99			Undetermined/Unknown

Definition:	The Ice Lead Status indicates the surface nature of the lead.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Frequency of Leads or Fractures

Acronym:	ICELFQ

Code:	30312

Attribute Type:	Integer

Expected Input:	The number of leads or fractures per nautical mile

Definition:	ICERFQ indicates the frequency of leads or fractures in number per nautical mile.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute: 		Orientation of Leads or Fractures

Acronym:	ICELOR

Code:	30313

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			No Leads or Fractures

02			NE

03			E

04			SE

05			S

06			SW

07			W

08			NW

09			N

10			Variable

99			Undetermined/Unknown

Definition: 	ICELOR indicates the predominant orientation of leads and fractures.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Lead (or Fracture or Crack) Width

Acronym:	ICELWD

Code:	30314

Attribute Type:	Integer

Expected Input: 	Numeric value of the width expressed in metres

Definition:	ICELWD indicates the width of a lead or fracture or crack in metres.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:	Attribute type changed from floating point to integer.

- Ice Attribute:		Ice Location Information

Acronym:	ICELOC

Code:	30315

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			Specific Location

02			Presence in Area

Definitions:	ICELOC indicates whether the break is at a specific location, or whether there is a presence in the area.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Iceberg Size

Acronym:	ICEBSZ

Code:	30316

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			Growler (<1m asl)

02			Bergy Bit (1-<5m asl; 5-<15m length)

03			Small Iceberg (5-15m asl; 15-60m length)

04			Medium Iceberg (16-45m asl; 61-120m length)

05 			Large Iceberg (46-75m asl; 121-200m length)

06			Very Large Iceberg (>75m asl; >200m length)

07			Ice Island Fragment

08	Ice Island (in the Northern Hemisphere) or Very Large Tabular Berg (in the Southern Ocean)

09				Radar Target

99				Undetermined/Unknown

Definitions: 	The "Iceberg Size" categorizes the size of an iceberg.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Drift Direction

Acronym:	ICEDDR

Attribute Type:	Enumerated

Code:	30317

Expected Input:

ID			Meaning

01			No Ice Motion

02			Ice Drift to NE

03			Ice Drift to E

04			Ice Drift to SE

05			Ice Drift to S

06			Ice Drift to SW

07			Ice Drift to W

08			Ice Drift to NW

09			Ice Drift to N

10			Variable

99			Undetermined/Unknown

Definition:	ICEDDR indicates the direction in which an ice mass is drifting.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Drift Speed

Acronym:	ICEDSP

Attribute Type:	Floating

Code:	30318

Expected Input:	A numeric value of the speed of an ice mass expressed in knots.

Definitions: 	ICEDSP describes the speed in knots at which an ice mass is traveling.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Average Thickness

Acronym:	ICETCK

Attribute Type:	Integer

Code:	30319

Expected Input:	A numeric value indicating the average thickness of the ice in centimeters.

Definitions: 	Ice Average Thickness defines the average thickness of the ice.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	The accuracy of this Attribute value is approximately 10% (i.e. within 5 cm for the first 30 cm of thickness and within 10 cm for measurements up to 1.5 m).

Change from Version 4.1:	Attribute type changed from floating point to integer

- Ice Attribute:		Maximum Ice Thickness

Acronym:	ICEMAX

Code:	30320

Attribute Type:	Integer

Expected Input:	A numeric value indicating the maximum thickness of the ice in centimeters.

Definition:	ICEMAX specifies the maximum thickness of the ice.

References:	“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	The accuracy of this Attribute value is approximately 10% (i.e. within 5 cm for the first 30 cm of thickness and within 10 cm for measurements up to 1.5 m).

Change from Version 4.1:	Attribute type changed from floating point to integer

- Ice Attribute:		Minimum Ice Thickness

Acronym:	ICEMIN

Code:	30321

Attribute Type:	Integer

Expected Input:	A numeric value indicating the minimum thickness of the ice in centimeters.

Definition:	ICEMIN specifies the minimum thickness of the ice.

References:	“Ice in ECDIS Workshop,” June 3,4, 2000, St. John’s, Canada

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	The accuracy of this Attribute value is approximately 10% (i.e. within 5 cm for the first 30 cm of thickness and within 10 cm for measurements up to 1.5 m).

Change from Version 4.1:	Attribute type changed from floating point to integer

- Ice Attribute:		Ice Thickness Type

Acronym:	ICETTY

Code:	30322

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01 				Measured

02 				Estimated

99				Undetermined/Unknown

Definition: 	ICETTY indicated whether the thickness of the ice was measured or estimated.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Snow Depth

Acronym:	ICESCT

Code:	30323

Attribute Type:	Floating

Expected Input:	A numeric value indicating the depth of the snow cover in centimeters.

Definitions:	ICESCT specifies the depth of snow cover on the ice.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	A range of values can be used if the Snow Cover varies, or the precise value is uncertain.

Change from Version 4.1:

- Ice Attribute:		Snow Cover Concentration

Acronym:	ICESCN

Code:	30324

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			1/10

02			2/10

03			3/10

04			4/10

05			5/10

06			6/10

07			7/10

08			8/10

09			9/10

10			10/10

11			<1/10

12			No Snow Cover

99			Undetermined/Unknown

Definitions:	ICESCN indicates the concentration (aerial coverage) of snow in tenths.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Direction of Sastrugi

Acronym:	ICEDOS

Code:	30325

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			No Sastrugi

02			NE

03				E

04				SE

05				S

06				SW

07				W

08				NW

09				N

10				Variable

99				Undetermined/Unknown

Definitions: 	ICEDOS indicates the bearing of a sastrugi.

Sastrugi: Sharp, irregular ridges formed on a snow surface by wind erosion and deposition. On mobile floating ice the ridges are parallel to the direction of the wind at the time they were formed.

References:	“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Ridge Concentration

Acronym:	ICERCN

Code:	30326

Attribute Type:	List

Expected Input:

| ID | Meaning |
| --- | --- |
| 01 | 0/10 – 1/10 |
| 10 | 1/10 |
| 12 | 1/10 – 2/10 |
| 20 | 2/10 |
| 23 | 2/10 – 3/10 |
| 30 | 3/10 |
| 34 | 3/10 – 4/10 |
| 40 | 4/10 |
| 45 | 4/10 – 5/10 |
| 50 | 5/10 |
| 56 | 5/10 – 6/10 |
| 60 | 6/10 |
| 67 | 6/10 – 7/10 |
| 70 | 7/10 |
| 78 | 7/10 – 8/10 |
| 80 | 8/10 |
| 89 | 8/10 – 9/10 |
| 90 | 9/10 |
| 91 | 9/10 – 10/10 |
| 92 | 10/10 |
| 98 | No Ridging |
| 99 | Undetermined/Unknown |

Definitions:	ICERCN describes the concentration of hummocked ice in an ice area.  Up to three values may be given in the list to correspond to the partial concentrations in ICEAPC.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	Possible versions of encoding for the 1st, 2nd and 3rd partial concentration:

- XX

- XX , XX

- XX , XX , XX

Change from Version 4.1:	Changed from an enumerated value to a list to allow different ridge concentrations to be described for each partial concentration

Code table for expected input revised to allow ranges

- Ice Attribute:		Ice Ridge Classification

Acronym:	ICERDV

Code:	30327

Attribute Type:	Enumerated

Expected Input:

ID	Meaning

01	New Ridge

02	Weathered Ridge

03	Very Weathered Ridge

04	Aged Ridge

05	Consolidated Ridge

06	Ridge

07	Ridged Ice Zone

08	Ridge Barrier

99		Undetermined/Unknown

Definitions:	ICERDV describes the predominant type of ice ridge(s) present.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:	IDs for additional ridge descriptions added

- Ice Attribute:		Ice Ridge Mean Height

Acronym:	ICERMH

Code:	30328

Attribute Type:	Integer

Expected Input:	A numeric value indicating the mean height of the ridge(s).

Definitions:	ICERMH indicates the mean height of ice ridge(s) in decimetres.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:	Attribute type changed from floating point to integer.

- Ice Attribute:		Ice Ridge Frequency

Acronym:	ICERFQ

Code:	30329

Attribute Type:	Integer

Expected Input:	The number of ice ridges per nautical mile

Definitions:	ICERFQ indicates the frequency of ice ridges in number per nautical mile

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Ridge Maximum Height

Acronym:	ICERXH

Code:	30330

Attribute Type:	Integer

Expected Input:	A numeric value indicating the maximum height of the ice ridge(s).

Definitions:	 ICERMT indicates the maximum height of ice ridge(s) in decimetres.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:	Attribute type changed from floating point to integer.

- Ice Attribute:		Ice Keel Concentration

Acronym:	ICEKCN

Code:	30331

Attribute Type:	Enumerated

Expected Input:

| ID | Meaning |
| --- | --- |
| 01 | 0/10 – 1/10 |
| 10 | 1/10 |
| 12 | 1/10 – 2/10 |
| 20 | 2/10 |
| 23 | 2/10 – 3/10 |
| 30 | 3/10 |
| 34 | 3/10 – 4/10 |
| 40 | 4/10 |
| 45 | 4/10 – 5/10 |
| 50 | 5/10 |
| 56 | 5/10 – 6/10 |
| 60 | 6/10 |
| 67 | 6/10 – 7/10 |
| 70 | 7/10 |
| 78 | 7/10 – 8/10 |
| 80 | 8/10 |
| 89 | 8/10 – 9/10 |
| 90 | 9/10 |
| 91 | 9/10 – 10/10 |
| 92 | 10/10 |
| 98 | No Keels |
| 99 | Undetermined/Unknown |

Definitions:	ICEKCN describes the concentration of ice keels beneath an ice area.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989.

Remarks:

Change from Version 4.1:	Code table for expected input revised to allow ranges

- Ice Attribute:		Ice Keel Frequency

Acronym:	ICEKFQ

Code:	30332

Attribute Type:	Integer

Expected Input:	The number of ice keels per nautical mile

Definitions:	ICERFQ indicates the frequency of ice keels in number per nautical mile

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Keel Mean Depth

Acronym:	ICEKMD

Code:	30333

Attribute Type:	Floating

Expected Input:	A numeric value indicating the mean depth of the ice keels.

Definitions:	ICERMT indicates the mean depth of ice keels in decimetres.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Keel Maximum Depth

Acronym:	ICEKXD

Code:	30334

Attribute Type:	Floating

Expected Input:	A numeric value indicating the maximum depth of the ice keels.

Definitions:	 ICERMT indicates the maximum depth of ice keels in decimetres.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:

- Ice Attribute:		Ice Rafting Concentration

Acronym:	ICEFCN

Code:	30335

Attribute Type:	Enumerated

Expected Input:

| ID | Meaning |
| --- | --- |
| 01 | 0/10 – 1/10 |
| 10 | 1/10 |
| 12 | 1/10 – 2/10 |
| 20 | 2/10 |
| 23 | 2/10 – 3/10 |
| 30 | 3/10 |
| 34 | 3/10 – 4/10 |
| 40 | 4/10 |
| 45 | 4/10 – 5/10 |
| 50 | 5/10 |
| 56 | 5/10 – 6/10 |
| 60 | 6/10 |
| 67 | 6/10 – 7/10 |
| 70 | 7/10 |
| 78 | 7/10 – 8/10 |
| 80 | 8/10 |
| 89 | 8/10 – 9/10 |
| 90 | 9/10 |
| 91 | 9/10 – 10/10 |
| 92 | 10/10 |
| 98 | No Rafting |
| 99 | Undetermined/Unknown |

Definitions:	ICEFCN describes the concentration of ice rafting in an ice area.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.1:	Code table for expected input revised to allow ranges

- Ice Attribute:		Brash Ice

Acronym:	ICEBRS

Code:	30362

Attribute Type:	List

Expected Input:	List of 4 IDs from below representing, in order from first to last, the concentrations of Very Thick Brash, Thick Brash, Medium Brash and Thin Brash Ice.

| ID | Meaning |
| --- | --- |
| 01 | 0/10 – 1/10 |
| 10 | 1/10 |
| 12 | 1/10 – 2/10 |
| 20 | 2/10 |
| 23 | 2/10 – 3/10 |
| 30 | 3/10 |
| 34 | 3/10 – 4/10 |
| 40 | 4/10 |
| 45 | 4/10 – 5/10 |
| 50 | 5/10 |
| 56 | 5/10 – 6/10 |
| 60 | 6/10 |
| 67 | 6/10 – 7/10 |
| 70 | 7/10 |
| 78 | 7/10 – 8/10 |
| 80 | 8/10 |
| 89 | 8/10 – 9/10 |
| 90 | 9/10 |
| 91 | 9/10 – 10/10 |
| 92 | 10/10 |
| 98 | No Brash ice |
| 99 | Undetermined/Unknown |

Definition:	ICEBRS specifies the concentrations of very thick brash (“AV”), thick brash (“AK”), medium brash (“AM”) and thin brash ice (“AT”).

- Very Thick Brash Ice	>4 m

- Thick Brash Ice		>2 - 4 m

- Medium Brash Ice		1 – 2 m

- Thin Brash Ice		<1 m

Example:	ICEBRS: [30, 20, 50, 01]

represents 3/10 of Very Thick Brash, 2/10 of Thick Brash, 4/10 of Medium Brash and no Thin Brash in an area.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Remarks:	Ice Attribute ICEBRS is only used when Code 70 is reported in Ice Attribute ICESOD or ICELSO

Change from Version 4.1:	Numeric code changed to avoid conflict with new attributes

Expected input code table changed for consistency with new attributes

Ice Attribute:		Combination Ice Stage of Development and Floe Size for the 1st partial concentration

Acronym:	IA_SFA

Code:	30336

Attribute Type:	List

Expected Input:

| ID | Meaning |  |
| --- | --- | --- |
| 01 | Pancake Ice (30 cm – 3 m) | Floe Size (FF) |
| 02 | Shuga/Small Ice Cake; Brash Ice (<2 m) |  |
| 03 | Ice Cake (<20 m) |  |
| 04 | Small Floe (20 – 100 m) |  |
| 05 | Medium Floe (100-500 m) |  |
| 06 | Big Floe (500 – 2000 m) |  |
| 07 | Vast Floe (2 – 10 km) |  |
| 08 | Giant Floe (>10 km) |  |
| 80 | No stage of development | Ice Stage of Development (SS) |
| 81 | New Ice (<5 cm) |  |
| 82 | Nilas Ice (<10 cm) |  |
| 83 | Young Ice (10 to <30 cm) |  |
| 84 | Grey Ice (10 to <15 cm) |  |
| 85 | Grey – White Ice (15 to <30 cm) |  |
| 86 | First Year Ice (30 to 200 cm) |  |
| 87 | Thin First Year Ice (30 to <70 cm) |  |
| 88 | Thin First Year Ice Stage 1 (30 to <50 cm) |  |
| 89 | Thin First Year Ice Stage 2 (50 to <70 cm) |  |
| 90 | Code not currently assigned |  |
| 91 | Medium First Year Ice (70 to 120 cm) |  |
| 92 | Code not currently assigned |  |
| 93 | Thick First Year Ice (>120 cm) |  |
| 94 | Residual Ice |  |
| 95 | Old Ice |  |
| 96 | Second Year Ice |  |
| 97 | Multi-Year Ice |  |
| 99 | Undetermined/Unknown |  |

Definitions:	IA_SFA describes combination(s) of Ice Stage of Development and Floe Size for the first partial concentration in an ice area.  Up to three (stage of development / form of ice) combinations are allowed to describe the ice in the first partial concentration group.

References:

Distinction:	ICESOD, ICEFLZ, IA_FFA

Remarks:	IA_SFA, IA_SFB and IA_SFC present an alternative encoding to ICESOD and ICEFLZ under the following rules:

        - Major stages of development (old, first-year, young, new) shall be delineated by different partial concentrations in ICEAPC

        - Stages of development belonging to the same major stage may be encoded inside using both single or different partial concentrations

        - Up to three forms of ice are allowed for each partial concentration group

- SS / FF

- SS / FF , SS / FF

- SS / FF , SS / FF , SS / FF

Example A:	total concentration: 9/10

3/10 of thick first year, 5/10 of grey-white and 1/10 of new ice;

thick first year ice has some vast floes, some big floes and some small floes;

grey-white ice has some medium floes and some small floes;

new ice is all in the form of shuga:

ICEACT: [90]

ICEAPC: [30, 50, 10]

IA_SFA: [93/07, 93/06, 93/04]

IA_SFB: [85/05, 85/04]

IA_SFC: [81/02]

Example B:	total concentration: 10/10

7-8/10 of old ice, 2 of some thick and some medium first-year and 0-1/10 of some grey and some grey-white young ice;

old ice has some medium and big floes;

thick and medium have medium floes;

grey and grey-white have small floes:

ICEACT: [92]

ICEAPC: [78, 20, 02]

IA_SFA: [95/06, 95/05]

IA_SFB: [93/05, 91/05]

IA_SFC: [85/04, 84/04]

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Combination Ice Stage of Development and Floe Size for the 2nd partial concentration

Acronym:	IA_SFB

Code:	30337

Attribute Type:	List

Expected Input:

| ID | Meaning |  |
| --- | --- | --- |
| 01 | Pancake Ice (30 cm – 3 m) | Floe Size (FF) |
| 02 | Shuga/Small Ice Cake; Brash Ice (<2 m) |  |
| 03 | Ice Cake (<20 m) |  |
| 04 | Small Floe (20 – 100 m) |  |
| 05 | Medium Floe (100-500 m) |  |
| 06 | Big Floe (500 – 2000 m) |  |
| 07 | Vast Floe (2 – 10 km) |  |
| 08 | Giant Floe (>10 km) |  |
| 80 | No stage of development | Ice Stage of Development (SS) |
| 81 | New Ice (<5 cm) |  |
| 82 | Nilas Ice (<10 cm) |  |
| 83 | Young Ice (10 to <30 cm) |  |
| 84 | Grey Ice (10 to <15 cm) |  |
| 85 | Grey – White Ice (15 to <30 cm) |  |
| 86 | First Year Ice (30 to 200 cm) |  |
| 87 | Thin First Year Ice (30 to <70 cm) |  |
| 88 | Thin First Year Ice Stage 1 (30 to <50 cm) |  |
| 89 | Thin First Year Ice Stage 2 (50 to <70 cm) |  |
| 90 | Code not currently assigned |  |
| 91 | Medium First Year Ice (70 to 120 cm) |  |
| 92 | Code not currently assigned |  |
| 93 | Thick First Year Ice (>120 cm) |  |
| 94 | Residual Ice |  |
| 95 | Old Ice |  |
| 96 | Second Year Ice |  |
| 97 | Multi-Year Ice |  |
| 99 | Undetermined/Unknown |  |

Definitions:	IA_SFB describes combination(s) of Ice Stage of Development and Floe Size for the second partial concentration in an ice area.  Up to three (stage of development / form of ice) combinations are allowed to describe the ice in the second partial concentration group.

References:

Distinction:	ICESOD, ICEFLZ, IA_FFB

Remarks:	IA_SFA, IA_SFB and IA_SFC present an alternative encoding to ICESOD and ICEFLZ under the following rules:

        - Major stages of development (old, first-year, young, new) shall be delineated by different partial concentrations in ICEAPC

        - Stages of development belonging to the same major stage may be encoded inside using both single or different partial concentrations

        - Up to three stages of development / form of ice combinations are allowed for each partial concentration group

- SS / FF

- SS / FF , SS / FF

- SS / FF , SS / FF , SS / FF

Example A:	total concentration: 9/10

3/10 of thick first year, 5/10 of grey-white and 1/10 of new ice;

thick first year ice has some vast floes, some big floes and some small floes;

grey-white ice has some medium floes and some small floes;

new ice is all in the form of shuga:

ICEACT: [90]

ICEAPC: [30, 50, 10]

IA_SFA: [93/07, 93/06, 93/04]

IA_SFB: [85/05, 85/04]

IA_SFC: [81/02]

Example B:	total concentration: 10/10

7-8/10 of old ice, 2 of some thick and some medium first-year and 0-1/10 of some grey and some grey-white young ice;

old ice has some medium and big floes;

thick and medium have medium floes;

grey and grey-white have small floes:

ICEACT: [92]

ICEAPC: [78, 20, 02]

IA_SFA: [95/06, 95/05]

IA_SFB: [93/05, 91/05]

IA_SFC: [85/04, 84/04]

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Combination Ice Stage of Development and Floe Size for the 3rd partial concentration

Acronym:	IA_SFC

Code:	303368

Attribute Type:	List

Expected Input:

| ID | Meaning |  |
| --- | --- | --- |
| 01 | Pancake Ice (30 cm – 3 m) | Floe Size (FF) |
| 02 | Shuga/Small Ice Cake; Brash Ice (<2 m) |  |
| 03 | Ice Cake (<20 m) |  |
| 04 | Small Floe (20 – 100 m) |  |
| 05 | Medium Floe (100-500 m) |  |
| 06 | Big Floe (500 – 2000 m) |  |
| 07 | Vast Floe (2 – 10 km) |  |
| 08 | Giant Floe (>10 km) |  |
| 80 | No stage of development | Ice Stage of Development (SS) |
| 81 | New Ice (<5 cm) |  |
| 82 | Nilas Ice (<10 cm) |  |
| 83 | Young Ice (10 to <30 cm) |  |
| 84 | Grey Ice (10 to <15 cm) |  |
| 85 | Grey – White Ice (15 to <30 cm) |  |
| 86 | First Year Ice (30 to 200 cm) |  |
| 87 | Thin First Year Ice (30 to <70 cm) |  |
| 88 | Thin First Year Ice Stage 1 (30 to <50 cm) |  |
| 89 | Thin First Year Ice Stage 2 (50 to <70 cm) |  |
| 90 | Code not currently assigned |  |
| 91 | Medium First Year Ice (70 to 120 cm) |  |
| 92 | Code not currently assigned |  |
| 93 | Thick First Year Ice (>120 cm) |  |
| 94 | Residual Ice |  |
| 95 | Old Ice |  |
| 96 | Second Year Ice |  |
| 97 | Multi-Year Ice |  |
| 99 | Undetermined/Unknown |  |

Definitions:	IA_SFC describes combination(s) of Ice Stage of Development and Floe Size for the first partial concentration in an ice area.  Up to three (stage of development / form of ice) combinations are allowed to describe the ice in the third partial concentration group.

References:

Distinction:	ICESOD, ICEFLZ, IA_FFC

Remarks:	IA_SFA, IA_SFB and IA_SFC present an alternative encoding to ICESOD and ICEFLZ under the following rules:

        - Major stages of development (old, first-year, young, new) shall be delineated by different partial concentrations in ICEAPC

        - Stages of development belonging to the same major stage may be encoded inside using both single or different partial concentrations

        - Up to three forms of ice are allowed for each partial concentration group

- SS / FF

- SS / FF , SS / FF

- SS / FF , SS / FF , SS / FF

Example A:	total concentration: 9/10

3/10 of thick first year, 5/10 of grey-white and 1/10 of new ice;

thick first year ice has some vast floes, some big floes and some small floes;

grey-white ice has some medium floes and some small floes;

new ice is all in the form of shuga:

ICEACT: [90]

ICEAPC: [30, 50, 10]

IA_SFA: [93/07, 93/06, 93/04]

IA_SFB: [85/05, 85/04]

IA_SFC: [81/02]

Example B:	total concentration: 10/10

7-8/10 of old ice, 2 of some thick and some medium first-year and 0-1/10 of some grey and some grey-white young ice;

old ice has some medium and big floes;

thick and medium have medium floes;

grey and grey-white have small floes:

ICEACT: [92]

ICEAPC: [78, 20, 02]

IA_SFA: [95/06, 95/05]

IA_SFB: [93/05, 91/05]

IA_SFC: [85/04, 84/04]

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Ice Breccia for the first partial concentration

Acronym:	IA_FFA

Code:	30339

Attribute Type:	List

Expected Input:

| ID | Meaning |  |
| --- | --- | --- |
| 01 | Pancake Ice (30 cm – 3 m) | Floe Size (FF) |
| 02 | Shuga/Small Ice Cake; Brash Ice (<2 m) |  |
| 03 | Ice Cake (<20 m) |  |
| 04 | Small Floe (20 – 100 m) |  |
| 05 | Medium Floe (100-500 m) |  |
| 06 | Big Floe (500 – 2000 m) |  |
| 07 | Vast Floe (2 – 10 km) |  |
| 08 | Giant Floe (>10 km) |  |
| 80 | No stage of development | Ice Stage of Development (SS) |
| 81 | New Ice (<5 cm) |  |
| 82 | Nilas Ice (<10 cm) |  |
| 83 | Young Ice (10 to <30 cm) |  |
| 84 | Grey Ice (10 to <15 cm) |  |
| 85 | Grey – White Ice (15 to <30 cm) |  |
| 86 | First Year Ice (30 to 200 cm) |  |
| 87 | Thin First Year Ice (30 to <70 cm) |  |
| 88 | Thin First Year Ice Stage 1 (30 to <50 cm) |  |
| 89 | Thin First Year Ice Stage 2 (50 to <70 cm) |  |
| 90 | Code not currently assigned |  |
| 91 | Medium First Year Ice (70 to 120 cm) |  |
| 92 | Code not currently assigned |  |
| 93 | Thick First Year Ice (>120 cm) |  |
| 94 | Residual Ice |  |
| 95 | Old Ice |  |
| 96 | Second Year Ice |  |
| 97 | Multi-Year Ice |  |
| 99 | Undetermined/Unknown |  |

Definitions:	Ice breccia is pieces of ice of different ages frozen together.  IA _FFA describes the combination(s) of Ice Stage of Development and Floe Size for the first partial concentration in an ice area.  Up to three (stage of development / form of ice) combinations plus the age of the younger ice consolidating the main pieces are allowed to describe the ice in the first partial concentration group.

References:

Distinction:	ICESOD, ICEFLZ, IA_SFA

Remarks:	IA_FFA, IA_FFB and IA_FFC present an alternative encoding to ICESOD and ICEFLZ under the following rules:

        - Major stages of development (old, first-year, young, new) shall be delineated by different partial concentrations in ICEAPC

        - Stages of development belonging to the same major stage may be encoded inside using both single or different partial concentrations

        - Up to three forms of ice are allowed for each partial concentration group

- SS / FF, SS1

- SS / FF , SS / FF, SS1

- SS / FF , SS / FF , SS / FF, SS1

  - Where SS1 is the stage of development of the younger ice consolidating the main pieces

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Ice Breccia for the second partial concentration

Acronym:	IA_FFB

Code:	30340

Attribute Type:	List

Expected Input:

| ID | Meaning |  |
| --- | --- | --- |
| 01 | Pancake Ice (30 cm – 3 m) | Floe Size (FF) |
| 02 | Shuga/Small Ice Cake; Brash Ice (<2 m) |  |
| 03 | Ice Cake (<20 m) |  |
| 04 | Small Floe (20 – 100 m) |  |
| 05 | Medium Floe (100-500 m) |  |
| 06 | Big Floe (500 – 2000 m) |  |
| 07 | Vast Floe (2 – 10 km) |  |
| 08 | Giant Floe (>10 km) |  |
| 80 | No stage of development | Ice Stage of Development (SS) |
| 81 | New Ice (<5 cm) |  |
| 82 | Nilas Ice (<10 cm) |  |
| 83 | Young Ice (10 to <30 cm) |  |
| 84 | Grey Ice (10 to <15 cm) |  |
| 85 | Grey – White Ice (15 to <30 cm) |  |
| 86 | First Year Ice (30 to 200 cm) |  |
| 87 | Thin First Year Ice (30 to <70 cm) |  |
| 88 | Thin First Year Ice Stage 1 (30 to <50 cm) |  |
| 89 | Thin First Year Ice Stage 2 (50 to <70 cm) |  |
| 90 | Code not currently assigned |  |
| 91 | Medium First Year Ice (70 to 120 cm) |  |
| 92 | Code not currently assigned |  |
| 93 | Thick First Year Ice (>120 cm) |  |
| 94 | Residual Ice |  |
| 95 | Old Ice |  |
| 96 | Second Year Ice |  |
| 97 | Multi-Year Ice |  |
| 99 | Undetermined/Unknown |  |

Definitions:	Ice breccia is pieces of ice of different ages frozen together.  IA _FFB describes the combination(s) of Ice Stage of Development and Floe Size for the second partial concentration in an ice area.  Up to three (stage of development / form of ice) combinations plus the age of the younger ice consolidating the main pieces are allowed to describe the ice in the second partial concentration group.

References:

Distinction:	ICESOD, ICEFLZ, IA_SFB

Remarks:	IA_FFA, IA_FFB and IA_FFC present an alternative encoding to ICESOD and ICEFLZ under the following rules:

        - Major stages of development (old, first-year, young, new) shall be delineated by different partial concentrations in ICEAPC

        - Stages of development belonging to the same major stage may be encoded inside using both single or different partial concentrations

        - Up to three forms of ice are allowed for each partial concentration group

- SS / FF, SS1

- SS / FF , SS / FF, SS1

- SS / FF , SS / FF , SS / FF, SS1

  - Where SS1 is the stage of development of the younger ice consolidating the main pieces

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Ice Breccia for the third partial concentration

Acronym:	IA_FFC

Code:	30341

Attribute Type:	List

Expected Input:

| ID | Meaning |  |
| --- | --- | --- |
| 01 | Pancake Ice (30 cm – 3 m) | Floe Size (FF) |
| 02 | Shuga/Small Ice Cake; Brash Ice (<2 m) |  |
| 03 | Ice Cake (<20 m) |  |
| 04 | Small Floe (20 – 100 m) |  |
| 05 | Medium Floe (100-500 m) |  |
| 06 | Big Floe (500 – 2000 m) |  |
| 07 | Vast Floe (2 – 10 km) |  |
| 08 | Giant Floe (>10 km) |  |
| 80 | No stage of development | Ice Stage of Development (SS) |
| 81 | New Ice (<5 cm) |  |
| 82 | Nilas Ice (<10 cm) |  |
| 83 | Young Ice (10 to <30 cm) |  |
| 84 | Grey Ice (10 to <15 cm) |  |
| 85 | Grey – White Ice (15 to <30 cm) |  |
| 86 | First Year Ice (30 to 200 cm) |  |
| 87 | Thin First Year Ice (30 to <70 cm) |  |
| 88 | Thin First Year Ice Stage 1 (30 to <50 cm) |  |
| 89 | Thin First Year Ice Stage 2 (50 to <70 cm) |  |
| 90 | Code not currently assigned |  |
| 91 | Medium First Year Ice (70 to 120 cm) |  |
| 92 | Code not currently assigned |  |
| 93 | Thick First Year Ice (>120 cm) |  |
| 94 | Residual Ice |  |
| 95 | Old Ice |  |
| 96 | Second Year Ice |  |
| 97 | Multi-Year Ice |  |
| 99 | Undetermined/Unknown |  |

Definitions:	Ice breccia is pieces of ice of different ages frozen together.  IA _FFA describes the combination(s) of Ice Stage of Development and Floe Size for the third partial concentration in an ice area.  Up to three (stage of development / form of ice) combinations plus the age of the younger ice consolidating the main pieces are allowed to describe the ice in the third partial concentration group.

References:

Distinction:	ICESOD, ICEFLZ, IA_SFC

Remarks:	IA_FFA, IA_FFB and IA_FFC present an alternative encoding to ICESOD and ICEFLZ under the following rules:

        - Major stages of development (old, first-year, young, new) shall be delineated by different partial concentrations in ICEAPC

        - Stages of development belonging to the same major stage may be encoded inside using both single or different partial concentrations

        - Up to three forms of ice are allowed for each partial concentration group

- SS / FF, SS1

- SS / FF , SS / FF, SS1

- SS / FF , SS / FF , SS / FF, SS1

  - Where SS1 is the stage of development of the younger ice consolidating the main pieces

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Snow cover

Acronym:			IA_SNG

Code:				30 344

Attribute Type:		Enumerated

Expected Input:

| ID | Meaning |
| --- | --- |
| 01 | Little snow cover |
| 10 | Thin snow cover (<5 cm) |
| 12 | <20 cm |
| 20 | Medium snow cover (5-20 cm) |
| 23 | >5 cm |
| 30 | Thick snow cover (>20 cm) |
| 98 | No snow cover |
| 99 | Undetermined/Unknown |

Definitions:	IA_SNG describes the degree of snow cover in an ice area.

References:	“Manual on conducting ice air reconnaissance”

Remarks:

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Stage of Melting

Acronym:			IA_MLT

Code:				30 345

Attribute Type:		Enumerated

Expected Input:

| ID | Meaning |  |
| --- | --- | --- |
| 01 | 0/5 – 1/5 | or 0/3 – 1/3 |
| 10 | 1/5 | or 1/3 |
| 12 | 1/5 – 2/5 | or 1/3 – 2/3 |
| 20 | 2/5 | or 2/3 |
| 23 | 2/5 – 3/5 | or 2/3 – 3/3 |
| 30 | 3/5 | or 3/3 |
| 34 | 3/5 – 4/5 |  |
| 40 | 4/5 |  |
| 45 | 4/5 – 5/5 |  |
| 50 | 5/5 |  |
| 98 | No Melting |  |
| 99 | Undetermined/Unknown |  |

Definitions:	IA_MLT describes the stage of melting according to the 5-point Russian national scale

References:	“Manual on conducting ice air reconnaissance”

Remarks:	Need to include the Russian national scale

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Contamination

Acronym:			IA_PLG

Code:				30 3456

Attribute Type:		Enumerated

Expected Input:

| ID | Meaning |
| --- | --- |
| 01 | No or insignificant Contamination |
| 10 | <1/3 of area |
| 12 | <2/3 of area |
| 20 | 1/3 – 2/3 of area |
| 23 | >1/3 area |
| 30 | >2/3 area |
| 98 | No Contamination |
| 99 | Undetermined / Unknown |

Definitions:	IA_PLG describes the degree of contamination (aerial coverage) in thirds based on the Russian national 3-point scale.

References:		“Manual on conducting ice air reconnaissance”

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Concentration of Hills

Acronym:			IA_HLG

Code:				30 347

Attribute Type:		Enumerated

Expected Input:

| ID | Meaning |
| --- | --- |
| 01 | Few hills |
| 10 | Slight concentration of hills |
| 12 | Slight to moderate concentration of hills |
| 20 | Moderate concentration of hills |
| 23 | Moderate to heavy concentration of hills |
| 30 | Heavy concentration of hills |
| 98 | No hills |
| 99 | Undetermined / Unknown |

Definitions:	IA_HLG describes the degree of hilling in an ice area (usually multi-year ice).

References:		“Manual on conducting ice air reconnaissance”

Remarks:

Change from Version 4.1:	This is a new attribute.

Ice Attribute:		Fractures Concentration

Acronym:			IA_DUG

Code:				30 349

Attribute Type:		Enumerated

Expected Input:

| ID | Meaning |
| --- | --- |
| 10 | Frequency of Cracks and Leads seldom then in 10 km on Route |
| 20 | Frequency of Cracks and Leads in 5 – 10 km on Route |
| 30 | Frequency of Cracks and Leads in 3 – 5 km on Route |
| 40 | Frequency of Cracks and Leads in 2 – 3 km on Route |
| 50 | Frequency of Cracks and Leads often then in 2 km on Route |
| 60 | Small and Medium Floes – 1/10-3/10; Big Floes – 7/10-10/10 Conc. |
| 70 | Small and Medium Floes – 4/10-6/10; Big Floes – 4/10-6/10 Conc. |
| 80 | Small and Medium Floes – 7/10-10/10; Big Floes – 1/10-3/10 Conc. |
| 90 | Small and Medium Floes Only |
| 92 | Small Floes Only |
| 98 | No Fractures |
| 99 | Undetermined / Unknown |

Definitions:	IA_DUG describes the degree of disunity in an ice area.

References:		“Manual on conducting ice air reconnaissance”

Remarks:

Change from Version 4.1:	This is a new attribute.

- Ice Attribute:		Iceberg Concentration

Acronym:	IA_BCN

Code:	30353

Attribute Type:	Enumerated

Expected Input:

| ID | Meaning |
| --- | --- |
| 10 | >45 nm between bergs |
| 12 | >15 nm between bergs |
| 20 | 15 - 44 nm between bergs |
| 23 | 10 - 44 nm between bergs |
| 30 | 10 - 14 nm between bergs |
| 34 | 7 - 14 nm between bergs |
| 40 | 7 - 10 nm between bergs |
| 45 | 5 - 10 nm between bergs |
| 50 | 5 - 6 nm between bergs |
| 56 | 3 - 6 nm between bergs |
| 60 | 3 - 4 nm between bergs |
| 67 | 1 - 4 nm between bergs |
| 70 | 1 - 2 nm between bergs |
| 78 | 0.5 - 2.0 nm between bergs |
| 80 | 0.5 - 1.0 nm between bergs |
| 89 | <1.0 nm between bergs |
| 90 | <0.5 nm between bergs |
| 98 | No Icebergs |
| 99 | Undetermined/Unknown |

Definitions:	IA_BCN specifies the total concentration of icebergs in an area.

References:	“Manual on conducting ice air reconnaissance”

Distinction:	ICEBNM

Remarks:	An alternative to ICEBNM

Change from Version 4.1:	This is a new attribute.

- Ice Attribute:		Iceberg Form

Acronym:	IA_BFM

Code:	30354

Attribute Type:	Enumerated

Expected Input:

| ID | Meaning |
| --- | --- |
| 01 | Domed |
| 02 | Tabular |
| 03 | Sloping |
| 04 | Pinnacled |
| 05 | Dry-dock |
| 06 | Glacier (irregular) |
| 07 | Blocky |
| 08 | Weathered |
| 99 | Undetermined/Unknown |

Definitions:	IA_BFM specifies the prevailing form of icebergs in an area.

References:

Remarks:

Change from Version 4.1:	This is a new attribute.

- Ice Attribute:		Maximum Height of Iceberg Topside

Acronym:	IA_BUH

Code:	30355

Attribute Type:	Integer

Expected Input:	A numeric value indicating the maximum height of the iceberg in meters.

Definitions:	IA_BUF specifies the maximum height of an iceberg above the waterline in meters.

References:

Remarks:

Change from Version 4.1:	This is a new attribute.

- Ice Attribute:		Number of Ice Objects

Acronym:	IA_OBN

Code:	30358

Attribute Type:	Integer

Expected Input:	A numeric value indicating the number of ice objects.

Definitions:	IA_OBN defines the number of ice objects (cracks, leads, fractures, icebergs).

References:

Remarks:

Change from Version 4.1:	This is a new attribute.

- Ice Attribute:		Maximum Width of Ice Lead (or Fracture or Crack)

Acronym:	IA_DXW

Code:	30360

Attribute Type:	Integer

Expected Input:	A numeric value indicating the maximum width of an ice lead or fracture or crack in meters.

Definitions:	IA_DXW defines the maximum width of the lead or fracture or crack.

References:

Remarks:

Change from Version 4.1:	This is a new attribute.

- Ice Attribute:		Minimum Width of Ice Lead (or Fracture or Crack)

Acronym:	IA_DMW

Code:	30361

Attribute Type:	Integer

Expected Input:	A numeric value indicating the minimum width of an ice lead or fracture or crack in meters.

Definitions:	IA_DMW defines the minimum width of the lead or fracture or crack.

References:

Remarks:

Change from Version 4.1:	This is a new attribute.

REFERENCES

- Report: "Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Ottawa, Canada.

- Report: “Workshop on the Use of ECDIS in Ice Navigation,” May 1996, Hamburg, Germany.

- “Proposal for Ice Objects for use in S57”, Canadian Hydrographic Service, September 1996.

- Report: “Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

- “MANICE – Manual of Standards Procedures for Observing and Reporting Ice Conditions”, 8th Edition, 1984, Canadian Ice Centre, Ottawa, Canada.

- “Manual on conducting ice air reconnaissance”, 1981, Gidrometeoizdat, Leningrad, USSR.

- “IHO Transfer Standard for Digital Hydrographic Data”, Special Publication No. 57,  International Hydrographic Organization, Monaco, Edition 3.1 – November 2000.

- WMO/OMM/BMO – No. 259. TP. 145; ”Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

- “SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

- “ECDIS Ice Objects”, Version 3.0, Canadian Ice Service, March, 2001

- “Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

- Appendix A

- Attribute Types

For more detailed information see S57 IHO Transfer Standard for Hydrographic Data, Appendix A, Chapter 2,– Introduction: Section 2.1,  International Hydrographic Organization, Monaco, 18 November 1996.

Each Attribute is assigned to one of six types:

- Enumerated (‘E’): The expected input is a number selected from a list of pre-defined attribute values. Exactly one value must be chosen.

- List (‘L’):	The expected input is a number selected from a list of pre-defined attribute values. Where more than one value is used they must normally be separated but in special cases slashes (‘/’) may be used.

- Floating (‘F’):	The expected input is a floating-point numeric value with defined range, resolution, units, and format.

- Integer (‘I’):	The expected input is an integer numeric value with defined range, units, and format.

- Coded string (‘A’):	The expected input is a string of ASCII characters in a predefined format. (refer to S57 Appendix A, Annex A).

- Free Text (‘S’):	The expected input is a free-format alphanumeric string. It may be a file name that points to a text or graphic file.

Depending on the attribute type, the expected input is defined in the following ways:

For ‘E’ and “L’ type attributes a list of ID-numbers with associated, defined meanings is given.

For ‘A’, ‘F’, ‘I’ and ‘S’ type attributes the expected input is indicated in accordance with the type (see above).

In certain circumstances, it may be necessary to indicate to the recipient of a data set that the value of a certain attribute for an instance of an object class is not included. A zero length attribute value sub-field encodes this fact.

- Appendix B

- List of Non-ice S57 Attributes Referred to by Ice Objects

For more detailed information see S57 IHO Transfer Standard for Hydrographic Data, Appendix A, Chapter 2,– Attributes:  Section 2.2,  International Hydrographic Organization, Monaco, 18 November 1996.

Attribute Set A:

- NOBJNM: The individual name of an object in the national language

- OBJNAM: The individual name of an object in English

Attribute Set B:

- INFORM: Information – Textual information about an object

- NINFORM: Information – Textual information about an object in the national language

- SCAMIN: Scale Minimum - The minimum scale at which the object may be used; e.g. for ECDIS presentation.

- SCAMAX: Scale Maximum - The Maximum scale at which the object may be used; e.g. for ECDIS presentation.

- TXTDSC: Textual Description - A string encoding the file name of an external text file that contains the text in English.

- NTXTDS: Textual Description - A string encoding the file name of an external text file that contains the text in the national language.

- PICREP: Pictorial Representation - A string encoding the file name of an external graphic file (pixel or vector).

Attribute Set C:

- RECDAT: Recording Date - The date when the object was captured, edited or deleted.

- RECIND: Recording Indication – The procedure used for the encoding and entering of data.

- SORDAT: Source Date – The production data of the source, i.e. the date of measurement.

- SORIND: Source Indication – Information about the source of the object.