JCOMM

Expert Team on Sea Ice

Electronic Chart Systems

Ice Objects Catalogue

Version 4.1

(DRAFT)

February 12, 2010

Not Yet Approved

Canadian Ice Service

Environment Canada

373 Sussex Drive, Block E-3

Ottawa, K1A 0H3

Canada

Table of Contents

Record of Changes	iv

1.	Introduction	1

2.	APPROACH TO HARMONIZATION	3

3.	ICE OBJECT CLASSES	5

ICE OBJECT SUMMARY:	5

Ice Object Class:	 Sea Ice	6

Ice Object Class:	 Lake Ice	7

Ice Object Class:	 Iceberg Area	8

Ice Object Class:	 Ice Edge	9

Ice Object Class:	 Iceberg Limit	10

Ice Object Class:	 Limit of Open Water	11

Ice Object Class:	 Limit of All Known Ice	12

Ice Object Class:	 Ice Compacting	13

Ice Object Class:	 Ice Lead	14

Ice Object Class:	 Iceberg	15

Ice Object Class:	 Floeberg	16

Ice Object Class:	 Ice Thickness	17

Ice Object Class:	 Ice Shear	18

Ice Object Class:	 Ice Divergence	19

Ice Object Class:	 Ice Ridge/Hummock	20

Ice Object Class:	 Ice Keel/Bummock	21

Ice Object Class:	 Ice Drift	22

Ice Object Class:	 Ice Fracture	23

Ice Object Class:	 Ice Rafting	24

Ice Object Class:	 Jammed Brash Barrier	25

Ice Object Class:	 Stage of Melt	26

Ice Object Class:	 Snow Cover	27

Ice Object Class:	 Strips and Patches	28

4.	FEATURE ATTRIBUTES FOR ICE OBJECTS:	29

ICE FEATURE ATTRIBUTE SUMMARY:	29

Ice Attribute:	 Total Concentration	31

Ice Attribute:	 Partial Concentration	33

Ice Attribute:	 Ice Stage of Development	35

Ice Attribute:	 Lake Ice Stage of Development	37

Ice Attribute:	 Floe Sizes	39

Ice Attribute:	 Melt Stage	41

Ice Attribute:	 Concentration of Strips and Patches	43

Ice Attribute:	 Number of Icebergs in Area	44

Ice Attribute:	 Level Ice	45

Ice Attribute:	 Compacting Strength	46

Ice Attribute:	 Ice Fracture Type	47

Ice Attribute:	 Ice Lead Status	48

Ice Attribute:	 Frequency of Leads or Fractures	49

Ice Attribute: 	 Orientation of Leads or Fractures	50

Ice Attribute:	 Ice Lead or Fracture Width	51

Ice Attribute:	 Ice Location Information	52

Ice Attribute:	 Iceberg Size	53

Ice Attribute:	 Ice Drift Direction	54

Ice Attribute:	 Ice Drift Speed	55

Ice Attribute:	 Ice Average Thickness	56

Ice Attribute:	 Maximum Ice Thickness	57

Ice Attribute:	 Minimum Ice Thickness	58

Ice Attribute:	 Ice Thickness Type	59

Ice Attribute:	 Snow Depth	60

Ice Attribute:	 Snow Cover Concentration	61

Ice Attribute:	 Direction of Sastrugi	62

Ice Attribute:	 Ice Ridge Concentration	63

Ice Attribute:	 Ice Ridge Classification	64

Ice Attribute:	 Ice Ridge Mean Height	65

Ice Attribute:	 Ice Ridge Frequency	66

Ice Attribute:	 Ice Ridge Maximum Height	67

Ice Attribute:	 Ice Keel Concentration	68

Ice Attribute:	 Ice Keel Frequency	69

Ice Attribute:	 Ice Keel Mean Depth	70

Ice Attribute:	 Ice Keel Maximum Depth	71

Ice Attribute:	 Ice Rafting Concentration	72

Ice Attribute:	 Brash Ice	73

5.	REFERENCES	75

Appendix A	76

Attribute Types	76

Appendix B	77

List of Non-ice S57 Attributes Referred to by Ice Objects	77

# Record of Changes

Version 4.0	Initial version approved 30 March 2007

Version 4.1	Draft

    - S-57 Marine Ice Object (MOI) numeric codes have been assigned to all sea ice objects and attributes

    - Amended definition of Code 91 for ICEACT and ICEAPC to include “9+/10” ice concentration

    - Added Brash Ice Code 70 to Ice Objects ICESOD and ICESLO

    - Added Ice Attribute ICEBRS

    - Minor typographical and formatting errors fixed

    - Amended references to more accurate citations

    - “Remarks” updated for most ice features to indicate specific changes from Version 4.0

ECDIS Ice Objects

# Introduction

A number of northern nations, particularly, Canada, the United States, Germany and those bordering on the Baltic Sea, maintain Ice Services, and issue ice charts on a regular basis during winter months. These ice charts are used on ships as an aid to navigation in ice infested waters, and as ECDIS becomes more widely available on ships navigating these northern waters, it will be important to provide ice data in a form that can be used in those systems.

Several workshops have been held in order to investigate how ice objects should be handled for ECDIS, An initial meeting held in Ottawa on June 26-29, 1995, addressed many of the potential issues. For that first meeting, a report was prepared by SevenC’s, a German ECDIS company, which proposed a potential set of new S-57 compliant ice objects. A second report was subsequently prepared by IDON Corporation, which included a review of the level of ice object support in the existing S-57 version 2, and the DIGEST Standard Feature and Attribute Catalogue (FACC), Version 1.2. This report was reviewed by the ice experts from those nations present at the initial meeting (Germany, the United States and Canada), following which IDON Corporation formatted the ice objects into the form required for S-57 and produced Version 1.0 of the Ice Object Catalogue.

A second workshop held in Hamburg, May 21-23, 1996 included participation of ice experts from most of the Baltic nations as well as the US, Canada, and Argentina. This group reviewed and approved the Ice Object Catalogue with minor changes that were incorporated into Version 2.0 of the specifications.

SevenC’s proposed further modifications to the ice objects at a third workshop held in St. John’s, Newfoundland, June 5, 6th, 2000. Terraqueous Technologies and IDON Technologies integrated these recommendations into Version 3.0.

Little work has been done on the Ice Objects Catalogue itself since the release of Version 3.0 in June, 2000. However, Dr. Paul Birkel, of the MITRE Corporation in the U.S., performed an analysis of the catalogue and identified specific issues and internal consistencies that should be addressed. Additionally, within the International Hydrographic Organization (IHO), the Digital Geospatial Information Working Group (DGIWG) has set up an ice register within the Feature and Attributes (FAD) Registry. The Expert Team on Sea Ice (ETSI) of JCOMM/WMO has been named “register owner”.

The Ice Register was presented to the 2005 meeting in Ottawa of the International Ice Charting Working Group (IICWG). At that meeting, it was recommended that a review by ice experts was needed to ensure harmonization with present ice community standards, including WMO Nomenclature and Symbology, and SIGRID-3 data transfer format. The U.S. National Ice Center took that action. The NIC presented the results of that review to the 2006 meeting of the IICWG in Helsinki, Finland. At the Helsinki meeting, an action was taken to prepare a new draft of the ECDIS Ice Objects for submission to the WMO and the IICWG. The Canadian Ice Service (CIS) took on this review, which has resulted in this updated Ice Object Catalogue – Version 4. Version 4 will be presented to the next meeting of ETSI in March, 2007.

Subsequent to adoption at the meeting of the Expert Team on Sea Ice (ETSI) in March, 2007 and final approval by the International Hydrographic Organization (IHO), Version 4.0 was officially released. Since that time, S-57 Marine Ice Object (MOI) numeric codes have been assigned to all sea ice objects and attributes, using the numeric range approved at the 4th meeting of the IHO-IEC Harmonizing Working Group on Marine Information Overlays (HGMIO4) in May, 2007.

ECDIS/ENC BACKGROUND

An Electronic Navigational Chart (ENC) provides the data component of an Electronic Chart Display Information System (ECDIS), which is under review in many countries as an approved aid to navigation, and as the legal equivalent of a paper chart. However, a paper chart is not the only aid to navigation and in ice infested waters, Ice Charts are also used. It is therefore a natural extension to the ECDIS to incorporate ice data.

The presentation of ice data on an ECDIS is not necessarily a simple matter. One major difficulty is the potential clutter that the addition of ice data could cause on an ECDIS display. It may be that a chart display system can only operate as a true ECDIS without the ice information, and that the inclusion of an additional thematic group (layer) of ice information would downgrade an ECDIS to the lower status of an Electronic Chart System (ECS). However, it is still important for ice information to be fully compatible with an ECDIS system, because in certain waters it is vital for safety to be able to see the ice conditions integrated with the navigational chart data. This is an issue that requires further discussion by the ECDIS Colours and Symbols Working Group.

The second issue is the management of the frequent updates required for ice data. However, in order to begin to understand the scope of the problem, it is necessary to identify what are the potential ice objects and how they would be expressed in the S-57 standard.

The International Hydrographic Organization (IHO) has approved version 3.0 of the S-57 exchange standard for hydrographic information, with a minor upgrade to version 3.1 in November, 2000. This exchange standard consists of three primary parts. The first is the IHO S-57 Catalogue of Objects and Attributes. The second component is the specification of the S57 Exchange Format, and the third component is the Product Specification for Electronic Navigational Charts (ENCs). These three components, together with a mechanism for dynamic update, define an Electronic Navigational Chart Product. By designing the ice objects in a manner compatible with S-57, it should be possible to make use of the update mechanism designed for ECDIS for ice information.

The International Hydrographic Organization (IHO) and the Digital Geographic Information Working Group DGIWG, have worked on the harmonization of the DIGEST standard and the S-57 standard, and although the specification of ice objects is also of interest to the DGIWG the DIGEST Feature and Attribute Catalogue Committee (FACC) felt that the subject of ice objects was outside of their scope of interest.

The International Organization for Standardization (ISO) Technical Committee on Geographic Information and Geomatics is also examining object/feature coding. This standard (ISO 19110) is now at the draft international standard (DIS) stage. It establishes the method of describing an object catalogue such as the S-57 Object Catalogue in a universal manner. The IHO Transfer Standard Maintenance and Application Development Working Group (TSMADWG) is considering following this standard in future revisions of the IHO S-57 standard.

Finally, TSMADWG is presently developing the next IHO digital data transfer standard, to be called Special Publication S-100. Since this new standard is not yet finalized, the Features and Attributes described in this document are based upon S-57 Version 3.1.

# APPROACH TO HARMONIZATION

The Features and Attributes described in this document have, to the extent possible, been harmonized with the following ice community standards:

- "WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989 which includes:

  - Sea-Ice Nomenclature, Suppl. No. 5; and,

  - International System of Sea Ice Symbols, Suppl. No. 4

- “SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

WMO standards are quite outdated, and are internally inconsistent. The Nomenclature provides descriptive definitions designed to aid in visual observation of ice information. The System of Sea Ice Symbols was designed to standardize symbology that appears on ice charts which, at the time of publication, were primary produced in hardcopy. The main element of the WMO symbolization for ice information is the "Egg Code". Attributes are described in the form of an egg-shaped (oval) symbol that contains four horizontal rows to express up to 12 numerical values for different ice parameters. Other symbols are also included, to represent ice parameters at a particular location, either precisely or in the near vicinity.

SIGRID-3 is the vector data exchange format for ice chart information among ice services and with the Global Digital Sea Ice Data Banks for archival information. Polygons and associated attribute codes are supported, but line and point information is not. Thus, SIGRID-3 equates well to S-57 “area” Feature Objects, but not to “line” and “point” Feature Objects. However, since SIGRID-3 is a more recently developed standard than WMO, its attribute codes better represent the ice information (e.g.: ranges of concentrations, strips and patches) within the “Egg Code” used on International ice charts.

Thus, the harmonization of the Ice Objects Catalogue with WMO and SIGRID-3 standards, which resulted in this document, used the following approach:

- For the most part, WMO Symbology was used as a basis to develop S-57 Ice Feature Objects. This will enable Internationally-accepted symbology to be used for any ECDIS/ENC-displayed ice products.

- Because of internal inconsistencies within WMO ice symbol tables, and in order to support “Strips and Patches”, SIGRID-3 attribute codes are used as a basis for those Attributes related to the standard “Egg Code”.

- Since SIGRID-3 attribute codes do not support them, this version of the Ice Objects Catalogue does not support the “Double-Egg” sub-attributes which would be required with the Attribute “Strips and Patches” within the SEAICE and LACICE “area” Objects.

- For simplicity, Ice Feature Objects are defined as “area”, “point” or “line” only. For those point symbols in WMO Symbology that relate to either a “Specific Location”, or a “Presence in the Area”, a new “Ice Location” Attribute is defined.

- Neither WMO nor SIGRID-3 support line-type Objects. Additionally, there is no support for Iceberg information products produced by IIP or CIS, or for “Stage of Development” Attributes for Lake Ice (LACICE). These particular Ice Feature Objects have thus been harmonized with “MANICE – Manual of Standards Procedures for Observing and Reporting Ice Conditions”, 8th Edition, 1984, Canadian Ice Centre, Ottawa, Canada.

- During this harmonization process, some of the Attributes have been moved up to become Ice Feature Objects, so that the associated WMO attributes can be better accommodated without having to define “sub-attributes”. These include: Ice Fracture, Ice Compacting, Snow Cover, Stage of Melt, etc.

- Many of the Attributes associated with “point” Ice Feature Objects have also been added as Attributes to the SEAICE and LACICE “area” Feature Objects. This is done to provide S-57 support to future map-type/polygon-based ice information products, such as an ice lead product, ice drift products, etc.

- The following "regional" item has been included: "ICELVL (Level ice)" as an attribute to the SEAICE and LACICE objects. This is supported by WMO Symbology for use in the Baltic, for hatching or colouring of ice charts.

- Through discussion with Dr. Paul Birkel of The MITRE Corporation, and with Paul Seymour of NIC, Dr. Birkel’s comments from his analysis of October, 2003, have been included into this version of the Ice Objects Catalogue.

# ICE OBJECT CLASSES

The proposed Ice Feature Object Classes are described in accordance with the format specified in:

  - “IHO Transfer Standard for Digital Hydrographic Data”, Special Publication No. 57,  International Hydrographic Organization, Monaco, Edition 3.1 – Appendix A, IHO Object Catalogue, November 2000.

All Objects are of type “geo”, meaning “carries the descriptive characteristics of a real world entity.

## ICE OBJECT SUMMARY:

| Ice Object Class | Acronym | Code |
| --- | --- | --- |
| Polygon |  |  |
| Sea Ice | SEAICE | 30 300 |
| Lake Ice | LACICE | 30 301 |
| Iceberg Area | BRGARE | 30 302 |
| Polyline |  |  |
| Ice Edge | ICELNE | 30 320 |
| Iceberg Limit | BRGLNE | 30 321 |
| Limit of Open Water | OPNLNE | 30 322 |
| Limit of All Known Ice | LKILNE | 30 323 |
| Point |  |  |
| Ice Compacting | ICECOM | 30 350 |
| Ice Lead | ICELEA | 30 351 |
| Iceberg | ICEBRG | 30 352 |
| Floeberg | FLOBRG | 30 353 |
| Ice Thickness | ICETHK | 30 354 |
| Ice Shear | ICESHR | 30 355 |
| Ice Divergence | ICEDIV | 30 356 |
| Ice Ridge/Hummock | ICERDG | 30 357 |
| Ice Keel/Bummock | ICEKEL | 30 358 |
| Ice Drift | ICEDFT | 30 359 |
| Ice Fracture | ICEFRA | 30 360 |
| Ice Rafting | ICERFT | 30 361 |
| Jammed Brash Barrier | JMDBRR | 30 362 |
| Stage of Melt | STGMLT | 30 363 |
| Snow Cover | SNWCVR | 30 364 |
| Strips and Patches | STRPTC | 30 365 |

- Ice Object Class:		Sea Ice

Acronym:	SEAICE

Code:	30300

subset ‘Attribute_A’: 	NOBJNM; OBJNAM; ICEACT; ICEAPC; ICESOD; ICEFLZ; ICESPC; ICELVL; ICECST; ICEFTY; ICEDSP; ICEDDR; ICERCN; ICERFQ; ICERMH; ICERXH; ICERDV; ICEKCN, ICEKFQ, ICEKMD, ICEKXD, ICEFCN; ICETCK; ICEMAX; ICEMIN; ICETTY; ICEMLT; ICESCN; ICESCT; ICEDOS; ICELST; ICELFQ; ICELOR;ICELWD

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Area

Definition:	Sea Ice is an area at sea that is covered, in whole or in part, with ice.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

Distinction:	LACICE

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Lake Ice

Acronym:	LACICE

Code:	30301

subset ‘Attribute_A’: 	NOBJNM; OBJNAM; ICEACT; ICEAPC; ICELSO; ICEFLZ, ICESPC; ICELVL; ICECST; ICEFTY; ICEDSP; ICEDDR; ICERCN; ICERFQ; ICERMH; ICERXH; ICERDV; ICEKCN, ICEKFQ, ICEKMD, ICEKXD, ICEFCN; ICETCK; ICEMAX; ICEMIN; ICETTY; ICEMLT; ICESCN; ICESCT; ICEDOS; ICELST; ICELFQ; ICELOR; ICELWD

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Area

Definition:	Lake Ice is an area on a lake that is covered, in whole or in part, with ice.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3, 4, 2000, St. John’s, Canada.

“Canadian Ice Service MANICE”, 9th edition, June, 2005

Distinction:	SEAICE

Remarks:

Change from Version 4.0:	Numeric code added

- Ice Object Class:		Iceberg Area

Acronym:	BRGARE

Code:	30302

subset  ‘Attribute_A’:	NOBJNM; OBJNAM; ICEBNM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Area

References:	"Workshop on International Standards for Ice Information in ECDIS,"June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3, 4, 2000, St. John’s, Canada.

“Canadian Ice Service MANICE”, 9th edition, June, 2005

Definition:	An Iceberg Area is an area at sea in which icebergs, bergy bits, or growlers are present.

Distinction:

Remarks:

Change from Version 4.0:	Numeric code added

- Ice Object Class:		Ice Edge

Acronym:	ICELNE

Code:	30320

subset ‘Attribute_A’:		NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	The demarcation at any given time between the open sea and sea ice of any kind and in any concentration, whether fast or drifting.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“Canadian Ice Service MANICE”, 9th edition, June, 2005

Distinction:	BRGLNE, OPNLNE, LKILNE

Remarks:	Note the distinction from OPNLNE

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Iceberg Limit

Acronym:	BRGLNE

Code:	30321

subset ‘Attribute_A’:	NOBJNM; OBJNAM;

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

References:	“Canadian Ice Service MANICE”, 9th edition, June, 2005.

Geometric Primitive:	Line

Distinction:	 ICELNE, OPNLNE, LKILNE

Definition:	Limit of all known Icebergs

Remarks:

Change from Version 4.0:	Numeric code added

- Ice Object Class:		Limit of Open Water

Acronym:	OPNLNE

Code:	30322

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP:

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	The demarcation at any given time between sea ice and freely navigable water, in which sea ice is present in concentrations less than 1/10.

References:	“Canadian Ice Service MANICE”, 9th edition, June, 2005.

Distinction:	ICELNE, BRGLNE, LKILNE

Remarks:	Note the distinction from ICELNE

Change from Version 4.0:	Numeric code added

- Ice Object Class:		Limit of All Known Ice

Acronym:	LKILNE

Code:	30323

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP:

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Line

Definition:	The limit of all known ice, including both sea ice of any kind and icebergs.

References:	“Canadian Ice Service MANICE”, 9th edition, June, 2005.

Distinction:	ICELNE, BRGLNE, OPNLNE

Remarks:

This line is a key product of the International Ice Patrol (IIP), to support safe navigation in the Northwest Atlantic under the UN Convention on Safety of Life at Sea (SOLAS).

Change from Version 4.0:	Numeric code added

Remarks amended

- Ice Object Class:		Ice Compacting

Acronym:	ICECOM

Code:	30350

subset “Attribute_A’:	NOBJNM; OBJNAM; ICECST

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Pieces of ice are said to be compacting when they are subjected to a converging motion, which increases ice concentration and/or produces stresses which may result in ice deformation.

References:	“International System of Sea-Ice Symbols, WMO No. 259, TP. 145, Supplement No. 4, 1970.”

Distinction:	ICESHR, ICEDFT, ICEDIV

Remarks:

Change from Version 4.0:	Numeric code added

- Ice Object Class:		Ice Lead

Acronym:	ICELEA

Code:	30351

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICELOC; ICELST; ICELWD

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Ice Lead identifies any fracture(s) or passage-way(s) through ice which is (are) navigable by surface vessels.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3, 4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEFRA

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Iceberg

Acronym:	ICEBRG

Code:	30352

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEBSZ; ICEDSP; ICEDDR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS, PICREP

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	An Iceberg is a massive piece of ice, greatly varying in shape and showing more than 5 meters above the sea surface which has broken away from a glacier, and which may be afloat or grounded. This Object Class also includes smaller forms of glacial ice, known as “Bergy Bits” and “Growlers”, which are defined by their size Attribute.

References:	"Workshop on International Standards for Ice Information in ECDIS,"June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	FLOBRG

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Floeberg

Acronym:	FLOBRG

Code:	30353

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEDSP; ICEDDR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A Floeberg is a massive piece of sea ice composed of a hummock or a group of hummocks, frozen together and separated from any ice surroundings. They typically protrude up to 5 meters above the sea surface.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEBRG

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Ice Thickness

Acronym:	ICETHK

Code:	30354

subset ‘Attribute_A”:	NOBJNM; OBJNAM; ICETCK; ICEMAX; ICEMIN; ICETTY

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Ice Thickness provides a measure or estimate of ice thickness.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3, 4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Ice Shear

Acronym:	ICESHR

Code:	30355

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	An area of drift ice is subject to shear when the ice motion varies significantly in the direction normal to the motion, subjecting the ice to rotational forces

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEDFT, ICEDIV, ICECOM

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Ice Divergence

Acronym:	ICEDIV

Code:	30356

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Ice fields or floes in an area are subject to diverging or dispersive motion, thus reducing ice concentration and/or relieving stresses in the ice.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEDFT, ICESHR, ICECOM

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Ice Ridge/Hummock

Acronym:	ICERDG

Code:	30357

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICERCN; ICERFQ; ICERMH; ICERXH, ICERDV

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	An Ice Ridge is a line or wall of broken ice forced up by pressure.

A Hummock is a hillock of broken ice which has been forced upward by pressure.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States. “Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICEKEL, ICERFT

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Ice Keel/Bummock

Acronym:	ICEKEL

Code:	30358

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEKCN; ICEKFQ; ICEKMD; ICEKXD

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	From a submariner’s point of view, a Keel is a downward projecting ridge on the underside of the ice canopy - the counterpart of a Ridge. A Bummock is the counterpart of a hummock on the underside of the ice canopy.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICERDG, ICERFT

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Ice Drift

Acronym:	ICEDFT

Code:	30359

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEDSP; ICEDDR

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Motion of an ice field or floe as a result of forces such as wind and currents.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICESHR, ICEDIV, ICECOM

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Ice Fracture

Acronym:	ICEFRA

Code:	30360

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEFTY, ICELOC

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Any break or rupture through the ice pack, or a single floe, resulting from deformation processes. Length may vary from a few metres to many kilometres.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICELEA

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Ice Rafting

Acronym:	ICERFT

Code:	30361

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEFCN

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	Pressure processes whereby one piece of ice overrides another.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:	ICERDG, ICEKEL

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Jammed Brash Barrier

Acronym:	JMDBRR

Code:	30362

subset ‘Attribute_A’:	NOBJNM; OBJNAM

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A strip or narrow belt of new, young or brash ice (usually 100-500 metres wide) formed at the edge of either drift or fast ice.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Stage of Melt

Acronym:	STGMLT

Code:	30363

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICEMLT

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A description of the stage of melt of the ice; i.e. whether it has formed puddles on the surface and whether these have frozen.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Snow Cover

Acronym:	SNWCVR

Code:	30364

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICESCN; ICESCT; ICEDOS

subset ‘Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A description of the amount of snow covering the ice.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Distinction:

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Object Class:		Strips and Patches

Acronym:	STRPTC

Code:	30365

subset ‘Attribute_A’:	NOBJNM; OBJNAM; ICESPC

subset’ Attribute_B’:	INFORM; NINFOM; SCAMIN; SCAMAX; TXTDSC; NTXTDS; PICREP;

subset ‘Attribute_C’:	RECDAT; RECIND; SORDAT; SORIND;

Geometric Primitive:	Point

Definition:	A strip is a long narrow area of floating ice, about 1 kilometre or less in width, usually composed of small fragments detached from the main mass of ice, and run together under the influence of wind, swell or current. If the area of ice becomes more rounded in shape, it is referred to as a patch.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989”

Distinction:

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- FEATURE ATTRIBUTES FOR ICE OBJECTS:

The proposed Attributes for Ice Objects are described in accordance with the format specified in:

  - “IHO Transfer Standard for Digital Hydrographic Data”, Special Publication No. 57,  International Hydrographic Organization, Monaco, Edition 3.1 – Appendix A, Chapter 2 - Attributes, November 2000.

All Attributes are intended to be of type “feature”, meaning “carries the description characteristics of a feature”.

## ICE FEATURE ATTRIBUTE SUMMARY:

| Ice Feature Attribute | Acronym | Version |
| --- | --- | --- |
| Ice Attribute Total Concentration | ICEACT | 30 300 |
| Ice Attribute Partial Concentration | ICEAPC | 30 301 |
| Ice Stage of Development | ICESOD | 30 302 |
| Lake Ice Stage of Development | ICELSO | 30 303 |
| Floe Sizes | ICEFLZ | 30 304 |
| Melt Stage | ICEMLT | 30 305 |
| Concentration of Strips and Patches | ICESPC | 30 306 |
| Number of Icebergs in Area | ICEBNM | 30 307 |
| Level Ice | ICELVL | 30 308 |
| Compacting Strength | ICECST | 30 309 |
| Ice Fracture Type | ICEFTY | 30 310 |
| Ice Lead Status | ICELST | 30 311 |
| Frequency of Leads or Fractures | ICELFQ | 30 312 |
| Orientation of Leads or Fractures | ICELOR | 30 313 |
| Ice Lead or Fracture Width | ICELWD | 30 314 |
| Ice Location Information | ICELOC | 30 315 |
| Iceberg Size | ICEBSZ | 30 316 |
| Ice Drift Direction | ICEDDR | 30 317 |
| Ice Drift Speed | ICEDSP | 30 318 |
| Ice Average Thickness | ICETCK | 30 319 |
| Maximum Ice Thickness | ICEMAX | 30 320 |
| Minimum Ice Thickness | ICEMIN | 30 321 |
| Ice Thickness Type | ICETTY | 30 322 |
| Snow Depth | ICESCT | 30 323 |
| Snow Cover Concentration | ICESCN | 30 324 |
| Direction Of Sastrugi | ICEDOS | 30 325 |
| Ice Ridge Concentration | ICERCN | 30 326 |
| Ice Ridge Classification | ICERDV | 30 327 |
| Ice Ridge Mean Height | ICERMH | 30 328 |
| Ice Ridge Frequency | ICERFQ | 30 329 |
| Ice Ridge Maximum Height | ICERXH | 30 330 |
| Ice Keel Concentration | ICEKCN | 30 331 |
| Ice Keel Frequency | ICEKFQ | 30 332 |
| Ice Keel Mean Depth | ICEKMD | 30 333 |
| Ice Keel Maximum Depth | ICEKXD | 30 334 |
| Ice Rafting Concentration | ICEFCN | 30 335 |

- Ice Attribute:		Total Concentration

Acronym:	ICEACT

Code:	30300

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01	Ice Free

02	Open Water (< 1/10 ice)

03	Bergy Water

10	1/10 ice

12	1/10 to 2/10 ice

13	1/10 to 3/10 ice

20	2/10 ice

23	2/10 to 3/10 ice

24	2/10 to 4/10 ice

30	3/10 ice

34	3/10 to 4/10 ice

35	3/10 to 5/10 ice

40	4/10 ice

45	4/10 to 5/10 ice

46	4/10 to 6/10 ice

50	5/10 ice

56	5/10 to 6/10 ice

57	5/10 to 7/10 ice

60	6/10 ice

67	6/10 to 7/10 ice

68	6/10 to 8/10 ice

70	7/10 ice

78	7/10 to 8/10 ice

79	7/10 to 9/10 ice

80	8/10 ice

81	8/10 to 10/10 ice

89	8/10 to 9/10 ice

90	9/10 ice

91	9/10 to 10/10 or 9+/10 ice

92	10/10 ice

99	Undetermined/Unknown

Definition:	ICEACT specifies the total concentration of ice in an area.  It represents the ratio expressed in tenths describing the total area of the water surface covered by ice as a fraction of the whole area.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989”

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Remarks:	Corresponds to “C” in International System of Sea Ice Symbols.

Corresponds to “CT” in SIGRID-3

Change from Version 4.0:	Numeric code added

Code 91 amended

References added

Remarks amended

- Ice Attribute:		Partial Concentration

Acronym:	ICEAPC

Code:	30301

Attribute Type:	List

Expected Input:

ID			Meaning

01	Ice Free

02	Open Water (< 1/10 ice)

03	Bergy Water

10	1/10 ice

12	1/10 to 2/10 ice

13	1/10 to 3/10 ice

20	2/10 ice

23	2/10 to 3/10 ice

24	2/10 to 4/10 ice

30	3/10 ice

34	3/10 to 4/10 ice

35	3/10 to 5/10 ice

40	4/10 ice

45	4/10 to 5/10 ice

46	4/10 to 6/10 ice

50	5/10 ice

56	5/10 to 6/10 ice

57	5/10 to 7/10 ice

60	6/10 ice

67	6/10 to 7/10 ice

68	6/10 to 8/10 ice

70	7/10 ice

78	7/10 to 8/10 ice

79	7/10 to 9/10 ice

80	8/10 ice

81	8/10 to 10/10 ice

89	8/10 to 9/10 ice

90	9/10 ice

91	9/10 to 10/10 or 9+/10 ice

92	10/10 ice

99	Undetermined/Unknown

Definition:	ICEAPC specifies the partial concentrations of ice in an area. (‘Ca, Cb and Cc’).

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Remarks:

Partial concentrations of ice are reported in order of decreasing thickness and are represented as an S-57 List (or repeating) attribute. Values are separated by a comma.

When only one ice type is present the partial concentration shall not be indicated.

Missing values are represented by the absence of any value of the attribute, which in ISO 8211 encoding of S57, would be adjacent commas.

Corresponds to “Ca, Cb, Cc ” in International System of Sea Ice Symbols.

Corresponds to “CA, CB, CC” in SIGRID-3

Change from Version 4.0:	Numeric code added

Code 91 amended

References added

Remarks amended

- Ice Attribute:		Ice Stage of Development

Acronym:	ICESOD

Code:	30302

Attribute Type:	List

Expected Input:

ID			Meaning

01	Ice Free

70	Brash Ice

80	No stage of development

81	New Ice  (<10 cm)

82	Nilas Ice Rind  (<10 cm)

83	Young Ice  (10 to <30 cm)

84	Grey Ice (10 to <15 cm)

85	Grey – White Ice  (15 to <30 cm)

86	First Year Ice  (30 to 200 cm)

87	Thin First Year Ice (30 to <70 cm)

88	Thin First Year Ice Stage 1 (30 to <50 cm)

89	Thin First Year Ice Stage 2 (50 to <70 cm)

90	Code not currently assigned

91	Medium First Year Ice  (70 to 120 cm)

92	Code not currently assigned

93	Thick First Year Ice (>120 cm)

94	Code not currently assigned

95	Old Ice

96	Second Year Ice

97	Multi-Year Ice

98	Glacier Ice (Icebergs)

99	Undetermined/Unknown

Definition:	Ice Stage of Development describe the ages and thicknesses of the ice (‘So,Sa,Sb,Sc and Sd’).

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Remarks:

Ice Stages of Development may be reported as a single enumerated value or as a set of values of thicknesses. The set of values is represented as an S-57 List (or repeating) attribute.

Corresponds to “So, Sa, Sb, Sc, Sd” in International System of Sea Ice Symbols.

Corresponds to “SO, SA, SB, SC, SD” in SIGRID-3

In conformity with the International System of Sea Ice Symbols, Stage of Development is reported in order from the thickest to the thinnest. The following categories are defined:

So – Stage of Development of ice thicker than Sa but having a concentration of less than 1/10.

Sa - Thickest/oldest; Stage of Development of ice concentration Ca.

Sb - Second thickest/oldest; Stage of Development of ice concentration Cb.

Sc - Third thickest/oldest; Stage of Development of ice concentration Cc.

Sd – Stage of Development of any other remaining class.

Missing values are represented by the absence of any value of the attribute, which in ISO 8211 encoding of S57, would be adjacent commas.

Change from Version 4.0:	Numeric code added

Code 70 added

References added

Remarks amended

- Ice Attribute:		Lake Ice Stage of Development

Acronym:	ICELSO

Code:	30303

Attribute Type:	List

Expected Input:

ID			Meaning

01	New Lake Ice  (<5cms)

02	Thin Lake Ice (5-<15cms)

03	Medium Lake Ice (15-<30cms)

04	Thick Lake Ice (30-70cms)

05	Very Thick Lake Ice (>70cms)

70	Brash Ice

99	Undetermined/Unknown

Definition:	Lake Ice Stages of Development describe the ages and thicknesses of lake ice. (‘So,Sa,Sb,Sc and Sd’)

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

“Canadian Ice Service MANICE”, 9th edition, June, 2005

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Remarks:	Lake Ice Stages of Development may be reported as a single enumerated value or as a set of values of thicknesses. The set of values is represented as an S-57 List (or repeating) attribute.

Corresponds to “So, Sa, Sb, Sc, Sd” in International System of Sea Ice Symbols.

Corresponds to “SO, SA, SB, SC, SD” in SIGRID-3

Stage of Development is reported in order from the thickest to the thinnest. The following categories are defined:

So – Stage of Development of ice thicker than Sa but having a concentration of less than 1/10.

Sa - Thickest/oldest; Stage of Development of ice concentration Ca.

Sb - Second thickest/oldest; Stage of Development of ice concentration Cb.

Sc - Third thickest/oldest; Stage of Development of ice concentration Cc.

Sd – Stage of Development of any other remaining class.

Missing values are represented by the absence of any value of the attribute that in ISO 8211 encoding of S57 would be adjacent commas.

Change from Version 4.0:	Numeric code added

Code 70 added

References added

Remarks amended

- Ice Attribute:		Floe Sizes

Acronym:	ICEFLZ

Code:	30304

Attribute Type:	List

Expected Input:

ID			Meaning

01				Pancake Ice (30 cm to 3m across)

02				Shuga/Small Ice Cake; Brash Ice (<2m across)

03				Ice Cake (<20m across)

04				Small Floe (20 to <100m across)

05				Medium Floe (100 to 500m)

06				Big Floe (500 to <2000m across)

07				Vast Floe (2000 to10000m across)

08				Giant Floe (>10000m across)

09				Fast Ice

10				Growlers, Floebergs or Floebits

11				Icebergs

99				Undetermined/Unknown

Definition: 	Floe Sizes describe the predominate forms of ice floe sizes (Fa, Fb and Fc) corresponding to the ice Stages of Development Sa, Sb and Sc respectively.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.” Ice in ECDIS Workshop,” June 3,4, 2000, St. John’s, Canada.

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

Remarks:

The "Floe Sizes" Attribute indicates the floe size corresponding to the respective stage identified in the Stages of Development Attribute and reported as a single enumerated value or as a set of values represented as an S-57 List (or repeating) attribute.

Corresponds to “Fa, Fb, Fc” in International System of Sea Ice Symbols.

Corresponds to “FA, FB, FC” in SIGRID-3

Missing values are represented by the absence of any value of the attribute that in ISO 8211 encoding of S57 would be adjacent commas.

Change from Version 4.0:	Numeric code added

Optional use of Fp and Fs removed

References added

Remarks amended

- Ice Attribute:		Melt Stage

Acronym:	ICEMLT

Attribute Type:	Enumerated

Code:	30305

Expected Input:

ID			Meaning

01	Few Puddles

02	Many Puddles

03	Flooded Ice

04	Few Thaw Holes

05	Many Thaw Holes

06	Dried Ice

07	Rotten Ice

08	Few Frozen Puddles

09	All Puddles Frozen

10	No Melt

99	Undetermined/Unknown

Definitions:	The Stage of Melt describes the stages of the melting ice.

Puddle: 	An accumulation of water on ice, mainly due to the melting of snow, but in some more advanced stages also the melting of ice.

Thaw Hole:	Vertical holes formed in ice when surface puddles melt through to the underlying water.

Dried Ice:	Ice-surface from which water has disappeared after the formation of cracks and thaw holes. During the process of drying the surface whitens.

Rotten Ice:	Ice which has become honey-combed and is in an advanced state of disintegration.

Flooded Ice:	Ice which has been flooded and is heavily loaded by water or water and wet snow.

Frozen Puddle: 	A puddle which has frozen over.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Concentration of Strips and Patches

Acronym:	ICESPC

Attribute Type:	Enumerated

Code:	30306

Expected Input:

ID			Meaning

11	Strips and Patches (concentrations 1/10)

12	Strips and Patches (concentrations 2/10)

13	Strips and Patches (concentrations 3/10)

14	Strips and Patches (concentrations 4/10)

15	Strips and Patches (concentrations 5/10)

16	Strips and Patches (concentrations 6/10)

17	Strips and Patches (concentrations 7/10)

18	Strips and Patches (concentrations 8/10)

19	Strips and Patches (concentrations 9/10)

20	Strips and Patches (concentrations 10/10)

99	Undetermined/Unknown

Definition:	A strip is a long narrow area of floating ice, about 1 kilometre or less in width, usually composed of small fragments detached from the main mass of ice, and run together under the influence of wind, swell or current. If the area of ice becomes more rounded in shape, it is referred to as a patch. ICESPC indicates the concentration in tenths within the area of Strips and Patches.

References:	“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

The concentration of Strips and Patches are provided exclusive of Floe Size values. When a Strips and Patches value is supplied, Floe Size values are null.

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Number of Icebergs in Area

Acronym:	ICEBNM

Attribute Type:	Integer

Code:	30307

Expected Input:	The number of icebergs in an area expressed by a density measurement on the basis of a grid.

Definitions:	 ICEBNM indicates the number of Icebergs within a specified area.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Canadian Ice Service MANICE”, 9th edition, June, 2005.

Remarks:	The area is defined by the dimensions of one degree latitude by one degree longitude and the attribute will be limited to 80 degrees North or South due to distortion at the poles.

Although measurements are not taken, areas that contain icebergs also usually contain bergy bits, and growlers.

Change from Version 4.0:	Numeric code added

- Ice Attribute:		Level Ice

Acronym:	ICELVL

Code:	30308

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01				Level (undeformed)

02				Deformed

99				Undetermined/Unknown

Definition:	ICELVL is an indication as to whether or not the ice has been affected by deformation

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	Use of this Attribute is a regional practice in the Baltic Sea.

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Compacting Strength

Acronym:	ICECST

Attribute Type:	Enumerated

Code:	30309

Expected Input:

ID			Meaning

01			Slight Compacting

02			Considerable Compacting

03			Strong Compacting

99			Undetermined/Unknown

Definition:	ICECST is an indication of strength of the compacting of the ice.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Ice Fracture Type

Acronym:	ICEFTY

Attribute Type:	Enumerated

Code:	30310

Expected Input:

ID			Meaning

01			Crack (0 to 1m wide)

02			Very Small Fracture (>1m to 50m wide)

03			Small Fracture (>50m to 200m wide)

04			Medium Fracture (>200m to 500m wide)

05			Large Fracture (>500m wide)

Definition:	ICEFTY indicates the type of fracture, based upon width.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Ice Lead Status

Acronym:	ICELST

Code:	30311

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			Open Lead

02			Frozen Lead

99			Undetermined/Unknown

Definition:	The Ice Lead Status indicates the surface nature of the lead.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Attribute:		Frequency of Leads or Fractures

Acronym:	ICELFQ

Code:	30312

Attribute Type:	Integer

Expected Input:	The number of leads or fractures per nautical mile

Definition:	ICERFQ indicates the frequency of leads or fractures in number per nautical mile.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute: 		Orientation of Leads or Fractures

Acronym:	ICELOR

Code:	30313

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			No Leads or Fractures

02			NE

03			E

04			SE

05			S

06			SW

07			W

08			NW

09			N

10			Variable

99			Undetermined/Unknown

Definition: 	ICELOR indicates the predominant orientation of leads and fractures.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Ice Lead or Fracture Width

Acronym:	ICELWD

Code:	30314

Attribute Type:	Floating

Expected Input: 	Numeric value of the width expressed in metres

Definition:	ICELWD indicates the width of a lead or fracture in metres.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Ice Location Information

Acronym:	ICELOC

Code:	30315

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			Specific Location

02			Presence In Area

Definitions:	ICELOC indicates whether the break is at a specific location, or whether there is a presence in the area.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Iceberg Size

Acronym:	ICEBSZ

Code:	30316

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			Growler (<1m asl)

02			Bergy Bit (1-<5m asl; 5-<15m length)

03			Small Iceberg (5-15m asl; 15-60m length)

04			Medium Iceberg (16-45m asl; 61-120m length)

05 			Large Iceberg (46-75m asl; 121-200m length)

06			Very Large Iceberg (>75m asl; >200m length)

07			Ice Island Fragment

08	Ice Island (in the Northern Hemisphere) or Very Large Tabular Berg (in the Southern Ocean)

09				Radar Target

99				Undetermined/Unknown

Definitions: 	The "Iceberg Size" categorizes the size of an iceberg.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Ice Drift Direction

Acronym:	ICEDDR

Attribute Type:	Enumerated

Code:	30317

Expected Input:

ID			Meaning

01			No Ice Motion

02			Ice Drift to NE

03			Ice Drift to E

04			Ice Drift to SE

05			Ice Drift to S

06			Ice Drift to SW

07			Ice Drift to W

08			Ice Drift to NW

09			Ice Drift to N

10			Variable

99			Undetermined/Unknown

Definition:	ICEDDR indicates the direction in which an ice mass is drifting.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Ice Drift Speed

Acronym:	ICEDSP

Attribute Type:	Floating

Code:	30318

Expected Input:	A numeric value of the speed of an ice mass expressed in knots.

Definitions: 	ICEDSP describes the speed in knots at which an ice mass is traveling.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

Units of speed added to definition

References amended

Remarks amended

- Ice Attribute:		Ice Average Thickness

Acronym:	ICETCK

Attribute Type:	Floating

Code:	30319

Expected Input:	A numeric value indicating the average thickness of the ice in centimeters.

Definitions: 	Ice Average Thickness defines the average thickness of the ice.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

The accuracy of this Attribute value is approximately 10% (i.e. within 5 cm for the first 30 cm of thickness and within 10 cm for measurements up to 1.5 m).

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Maximum Ice Thickness

Acronym:	ICEMAX

Code:	30320

Attribute Type:	Floating

Expected Input:	A numeric value indicating the maximum thickness of the ice in centimeters.

Definition:	ICEMAX specifies the maximum thickness of the ice.

References:	“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	The accuracy of this Attribute value is approximately 10% (i.e. within 5 cm for the first 30 cm of thickness and within 10 cm for measurements up to 1.5 m).

Change from Version 4.0:	Numeric code added

References amended

- Ice Attribute:		Minimum Ice Thickness

Acronym:	ICEMIN

Code:	30321

Attribute Type:	Floating

Expected Input:	A numeric value indicating the minimum thickness of the ice in centimeters.

Definition:	ICEMIN specifies the minimum thickness of the ice.

References:	“Ice in ECDIS Workshop,” June 3,4, 2000, St. John’s, Canada

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:	The accuracy of this Attribute value is approximately 10% (i.e. within 5 cm for the first 30 cm of thickness and within 10 cm for measurements up to 1.5 m).

Change from Version 4.0:	Numeric code added

References amended

- Ice Attribute:		Ice Thickness Type

Acronym:	ICETTY

Code:	30322

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01 				Measured

02 				Estimated

99				Undetermined/Unknown

Definition: 	ICETTY indicated whether the thickness of the ice was measured or estimated.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Snow Depth

Acronym:	ICESCT

Code:	30323

Attribute Type:	Floating

Expected Input:	A numeric value indicating the depth of the snow cover in centimeters.

Definitions:	ICESCT specifies the depth of snow cover on the ice.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

A range of values can be used if the Snow Cover varies, or the precise value is uncertain.

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Snow Cover Concentration

Acronym:	ICESCN

Code:	30324

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			1/10

02			2/10

03			3/10

04			4/10

05			5/10

06			6/10

07			7/10

08			8/10

09			9/10

10			10/10

11			<1/10

12			No Snow Cover

99			Undetermined/Unknown

Definitions:	ICESCN indicates the concentration (aerial coverage) of snow in tenths.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Direction of Sastrugi

Acronym:	ICEDOS

Code:	30325

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			No Sastrugi

02			NE

03				E

04				SE

05				S

06				SW

07				W

08				NW

09				N

10				Variable

99				Undetermined/Unknown

Definitions: 	ICEDOS indicates the bearing of a sastrugi.

Sastrugi: Sharp, irregular ridges formed on a snow surface by wind erosion and deposition. On mobile floating ice the ridges are parallel to the direction of the wind at the time they were formed.

References:	“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada

"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Attribute:		Ice Ridge Concentration

Acronym:	ICERCN

Code:	30326

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			1/10

02			2/10

03			3/10

04			4/10

05			5/10

06			6/10

07			7/10

08			8/10

09			9/10

10			10/10

11			<1/10

12			No Ridging

99			Undetermined/Unknown

Definitions:	ICERCN describes the concentration of ice ridges in an ice area.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

- Ice Attribute:		Ice Ridge Classification

Acronym:	ICERDV

Code:	30327

Attribute Type:	Enumerated

Expected Input:

ID	Meaning

01	New Ridge

02	Weathered Ridge

03	Very Weathered Ridge

04	Aged Ridge

05		Consolidated Ridge

99		Undetermined/Unknown

Definitions:	ICERDV describes the predominant type of ice ridge(s) present.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Ice Ridge Mean Height

Acronym:	ICERMH

Code:	30328

Attribute Type:	Floating

Expected Input:	A numeric value indicating the mean height of the ridge(s).

Definitions:	ICERMH indicates the mean height of ice ridge(s) in decimetres.

References:	"Workshop on International Standards for Ice Information in ECDIS,” June 27-29, 1995, Canada/Germany/United States.

“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

Units of measure added to definition

References amended

Remarks amended

- Ice Attribute:		Ice Ridge Frequency

Acronym:	ICERFQ

Code:	30329

Attribute Type:	Integer

Expected Input:	The number of ice ridges per nautical mile

Definitions:	ICERFQ indicates the frequency of ice ridges in number per nautical mile

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Ice Ridge Maximum Height

Acronym:	ICERXH

Code:	30330

Attribute Type:	Floating

Expected Input:	A numeric value indicating the maximum height of the ice ridge(s).

Definitions:	 ICERMT indicates the maximum height of ice ridge(s) in decimetres.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

Units of measure added to definition

References amended

Remarks amended

- Ice Attribute:		Ice Keel Concentration

Acronym:	ICEKCN

Code:	30331

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			1/10

02			2/10

03			3/10

04			4/10

05			5/10

06			6/10

07			7/10

08			8/10

09			9/10

10			10/10

11			<1/10

12			No Keels

99			Undetermined/Unknown

Definitions:	ICEKCN describes the concentration of ice keels beneath an ice area.

References:	”WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989.

Remarks:

Change from Version 4.0:	Numeric code added

References added

Remarks amended

- Ice Attribute:		Ice Keel Frequency

Acronym:	ICEKFQ

Code:	30332

Attribute Type:	Integer

Expected Input:	The number of ice keels per nautical mile

Definitions:	ICERFQ indicates the frequency of ice keels in number per nautical mile

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References added

Remarks amended

- Ice Attribute:		Ice Keel Mean Depth

Acronym:	ICEKMD

Code:	30333

Attribute Type:	Floating

Expected Input:	A numeric value indicating the mean depth of the ice keels.

Definitions:	ICERMT indicates the mean depth of ice keels in decimetres.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

Units of measure added to definition

References added

Remarks amended

- Ice Attribute:		Ice Keel Maximum Depth

Acronym:	ICEKXD

Code:	30334

Attribute Type:	Floating

Expected Input:	A numeric value indicating the maximum depth of the ice keels.

Definitions:	 ICERMT indicates the maximum depth of ice keels in decimetres.

References:	"WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols", WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

Units of measure added to definition

References added

Remarks amended

- Ice Attribute:		Ice Rafting Concentration

Acronym:	ICEFCN

Code:	30335

Attribute Type:	Enumerated

Expected Input:

ID			Meaning

01			1/10

02			2/10

03			3/10

04			4/10

05			5/10

06			6/10

07			7/10

08			8/10

09			9/10

10			10/10

11			<1/10

12			No Rafting

99			Undetermined/Unknown

Definitions:	ICEFCN describes the concentration of ice rafting in an ice area.

References:	“WMO Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

Remarks:

Change from Version 4.0:	Numeric code added

References amended

Remarks amended

- Ice Attribute:		Brash Ice

Acronym:	ICEBRS

Code:	30336

Attribute Type:	List

Expected Input:	List of 4 IDs from below representing, in order from first to last, the concentrations of Very Thick Brash, Thick Brash, Medium Brash and Thin Brash Ice.

ID			Meaning

01	0/10 ice

02	< 1/10 ice

10	1/10 ice

12	1/10 to 2/10 ice

13	1/10 to 3/10 ice

20	2/10 ice

23	2/10 to 3/10 ice

24	2/10 to 4/10 ice

30	3/10 ice

34	3/10 to 4/10 ice

35	3/10 to 5/10 ice

40	4/10 ice

45	4/10 to 5/10 ice

46	4/10 to 6/10 ice

50	5/10 ice

56	5/10 to 6/10 ice

57	5/10 to 7/10 ice

60	6/10 ice

67	6/10 to 7/10 ice

68	6/10 to 8/10 ice

70	7/10 ice

78	7/10 to 8/10 ice

79	7/10 to 9/10 ice

80	8/10 ice

81	8/10 to 10/10 ice

89	8/10 to 9/10 ice

90	9/10 ice

91	9/10 to 10/10 or 9+/10 ice

92	10/10 ice

99	Undetermined/Unknown

Definition:	ICEBRS specifies the concentrations of very thick brash (“AV”), thick brash (“AK”), medium brash (“AM”) and thin brash ice (“AT”).

Very Thick Brash Ice	>4 m

Thick Brash Ice	>2 - 4 m

Medium Brash Ice	1 – 2 m

Thin Brash Ice		<1 m

Example:	ICEBRS: [30, 20, 50, 01]

represents 3/10 of Very Thick Brash, 2/10 of Thick Brash, 4/10 of Medium Brash and no Thin Brash in an area.

References:	"Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Canada/Germany/United States.

“SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

“Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

Remarks:	Ice Attribute ICEBRS is only used when Code 70 is reported in Ice Attribute ICESOD or ICELSO

Change from Version 4.0:	This is a new attribute.

- REFERENCES

- Report: "Workshop on International Standards for Ice Information in ECDIS," June 27-29, 1995, Ottawa, Canada.

- Report: “Workshop on the Use of ECDIS in Ice Navigation,” May 1996, Hamburg, Germany.

- “Proposal for Ice Objects for use in S57”, Canadian Hydrographic Service, September 1996.

- Report: “Ice in ECDIS Workshop,” June 3-4, 2000, St. John’s, Canada.

- “MANICE – Manual of Standards Procedures for Observing and Reporting Ice Conditions”, 8th Edition, 1984, Canadian Ice Centre, Ottawa, Canada.

- “IHO Transfer Standard for Digital Hydrographic Data”, Special Publication No. 57,  International Hydrographic Organization, Monaco, Edition 3.1 – November 2000.

- WMO/OMM/BMO – No. 259. TP. 145; ”Sea-Ice Nomenclature and International System of Sea-Ice Symbols”, WMO Publication No. 259, Suppl. No. 5, 1989

- “SIGRID-3: A Vector Archive Format for Sea Ice Charts”, JCOMM Technical Report No. 23, 2004

- “ECDIS Ice Objects”, Version 3.0, Canadian Ice Service, March, 2001

- “Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue”; JCOMM Expert Team on Sea Ice Meeting IV, Document 2.6.1; March 2010.

- Appendix A

- Attribute Types

For more detailed information see S57 IHO Transfer Standard for Hydrographic Data, Appendix A, Chapter 2,– Introduction: Section 2.1,  International Hydrographic Organization, Monaco, 18 November 1996.

Each Attribute is assigned to one of six types:

- Enumerated (‘E’): The expected input is a number selected from a list of pre-defined attribute values. Exactly one value must be chosen.

- List (‘L’):	The expected input is a number selected from a list of pre-defined attribute values. Where more than one value is used they must normally be separated but in special cases slashes (‘/’) may be used.

- Floating (‘F’):	The expected input is a floating-point numeric value with defined range, resolution, units, and format.

- Integer (‘I’):	The expected input is an integer numeric value with defined range, units, and format.

- Coded string (‘A’):	The expected input is a string of ASCII characters in a predefined format. (refer to S57 Appendix A, Annex A).

- Free Text (‘S’):	The expected input is a free-format alphanumeric string. It may be a file name that points to a text or graphic file.

Depending on the attribute type, the expected input is defined in the following ways:

For ‘E’ and “L’ type attributes a list of ID-numbers with associated, defined meanings is given.

For ‘A’, ‘F’, ‘I’ and ‘S’ type attributes the expected input is indicated in accordance with the type (see above).

In certain circumstances, it may be necessary to indicate to the recipient of a data set that the value of a certain attribute for an instance of an object class is not included. A zero length attribute value sub-field encodes this fact.

- Appendix B

- List of Non-ice S57 Attributes Referred to by Ice Objects

For more detailed information see S57 IHO Transfer Standard for Hydrographic Data, Appendix A, Chapter 2,– Attributes:  Section 2.2,  International Hydrographic Organization, Monaco, 18 November 1996.

Attribute Set A:

- NOBJNM: The individual name of an object in the national language

- OBJNAM: The individual name of an object in English

Attribute Set B:

- INFORM: Information – Textual information about an object

- NINFORM: Information – Textual information about an object in the national language

- SCAMIN: Scale Minimum - The minimum scale at which the object may be used; e.g. for ECDIS presentation.

- SCAMAX: Scale Maximum - The Maximum scale at which the object may be used; e.g. for ECDIS presentation.

- TXTDSC: Textual Description - A string encoding the file name of an external text file that contains the text in English.

- NTXTDS: Textual Description - A string encoding the file name of an external text file that contains the text in the national language.

- PICREP: Pictorial Representation - A string encoding the file name of an external graphic file (pixel or vector).

Attribute Set C:

- RECDAT: Recording Date - The date when the object was captured, edited or deleted.

- RECIND: Recording Indication – The procedure used for the encoding and entering of data.

- SORDAT: Source Date – The production data of the source, i.e. the date of measurement.

- SORIND: Source Indication – Information about the source of the object.