Appendix I

- Proposed Changes to Harmonize the WMO Sea Ice Nomenclature and Symbology, the SIGRID-3 Coding Standard and the ENC Ice Objects Catalogue

Summary of Proposed Changes

| Reference | Change to WMO 259 | Change to SIGRID | Change to Ice Objects |
| --- | --- | --- | --- |
| LACICE | Add Lake Ice terms |  |  |
| BRGARE |  | Remove Floeberg from Table 4-13 |  |
| ICEBRG | Add iceberg types and renumber articles 4.7-4.12 |  |  |
| ICEKEL | Add symbol for ice keel |  |  |
| ICEFRA | Amend definition of symbols for “cracks” |  |  |
| ICEACT |  | Amend Table 4.1 to remove code “00” | Change definition of Code 91 in ICEACT to include 9+/10 |
| ICEAPC |  |  | Change definition of Code 91 in ICEAPC to include 9+/10 |
| ICESOD | Add 4.4.8.1.2 Windrow | Amend Table 4.2 to remove code “00”, clarify ambiguities in thickness ranges. | Add “Brash Ice” with Code 70 to ICESOD. Add new Ice Attribute ICEBRS |
| ICELSO |  |  | Add “Brash Ice” with Code 70 to ICELSO. |
| ICEFLZ | Discontinue use of the second variant to describe the forms of ice – remove Fp and Fs from Sea Ice Symbols |  |  |
| ICEFLZ |  | Amend Table 4.3 to remove code “00” and clarify ambiguities in size ranges. |  |
| ICEMLT |  | Amend Table 4.11 to remove code “00” and add term for “many puddles”. |  |

# Ice Object LACICE

## Observation

Lake Ice Stages of Development are not supported by WMO Symbology. In producing the Ice Objects Catalogue Version 4.0, the values for this attribute were taken from “MANICE – Manual of Standards Procedures for Observing and Reporting Ice Conditions”, 8th Edition, 1984, Canadian Ice Centre, Ottawa, Canada and have been in use for many years on Great Lakes ice charts produced by the Canadian Ice Service and the National Ice Center.

## Recommendation

It is recommended that the Sea Ice Nomenclature and Symbology be expanded to include terminology for lake ice and be re-titled accordingly.  The definitions of the terms should be extracted from the Canadian Ice Service MANICE, most recent edition.

Specifically:

Re-title the document and its tables to:  “Floating Ice Nomenclature and Symbology”.

Add the following new terms to the Nomenclature:

| Item No. | English | French | Russian | Spanish |
| --- | --- | --- | --- | --- |
| 2.7 | Development of Lake Ice: Because of the absence of salt in the water, the freezing and growth processes of lake ice are considerably different from those of sea ice. Generally, lake ice forms and is destroyed more quickly than sea ice, is more brittle and harder than sea ice. |  |  |  |
| 2.7.1 | New Lake Ice: Recently formed lake ice less than 5 cm thick. |  |  |  |
| 2.7.2 | Thin Lake Ice: Lake ice that is 5-15 cm in thickness. |  |  |  |
| 2.7.3 | Medium Lake Ice: Lake ice that is 15-30 cm in thickness. |  |  |  |
| 2.7.4 | Thick Lake Ice: Lake ice that is 30-70 cm in thickness. |  |  |  |
| 2.7.5 | Very Thick Lake Ice: Lake ice that is greater than 70 cm in thickness. |  |  |  |

## Rationale

There is continuous winter navigation on the Great Lakes and the North American Ice Service produces regular ice charts and bulletins in the same manner and with similar symbology as for the ice-encumbered oceans.  However, there is no existing international terminology and symbology for lake ice.  Since many terms and symbols for lake ice are similar to those for sea ice and since the information providers and users are largely the same, it is proposed that the best way to rectify this deficiency is to expand the Sea Ice Nomenclature to include lake ice.

# Ice Object BRGARE

## Observation

In the SIGRID-3 standard (SIGRID-3: A VECTOR ARCHIVE FORMAT FOR SEA ICE CHARTS WMO/TD-No. 1214; 2004; JCOMM Technical Report No. 23), Table 4.13 - Ice of land origin (type of iceberg) includes Floeberg with code value 8.  Since a “Floeberg” is, by WMO definition, composed of sea ice and not glacial ice, floebergs have been excluded from the expected inputs of “BRGARE”.  This is consistent with both Canadian Ice Service and International Ice Patrol practice.

## Recommendation

Remove “Floeberg” from Table 4.13 – Ice of land origin (type of ice iceberg) in the SIGRID-3 standard.  The revised table would be:

Table 4.13 Ice of land origin (type of iceberg)

| Form |  | Size |  |
| --- | --- | --- | --- |
| Growler and or bergy bit | 1 | unspecified | 0 |
| Iceberg, unspecified | 2 | small | 1 |
| Iceberg, glacier berg | 3 | medium | 2 |
| Iceberg, dome | 4 | large | 3 |
| Iceberg, pinnacled | 5 | very large | 4 |
| Iceberg, tabular | 6 |  |  |
| Ice island | 7 |  |  |
| Radar target | 9 |  |  |

# Ice Object ICEBRG

## Observation

The WMO terminology does not include all of the iceberg descriptions in use by the International Ice Patrol and the Canadian Ice Service.

## Recommendation

It is recommended that definitions for “dry-docked” and “blocky” icebergs be added, items 4.3.8 (Glacier berg), 4.3.9 (Tabular berg), 4.3.10 (Ice island), 4.3.11(Bergy bit) and 4.3.12 (Growler) be renumbered as below for consistency and mention of Antarctic large tabular bergs be made.

Specifically:

- amend Item 4.3.7 to included additional terms

- delete the items above and replace with the following:

| Item No. | English | French | Russian | Spanish |
| --- | --- | --- | --- | --- |
| 4.3.7 | Iceberg: Cf. 10.4.2 – A massive piece of ice of greatly varying shape, protruding more than 5 m above sea level, which has broken away from a glacier, and which may be afloat or aground. Icebergs may be described as tabular, dome-shaped, sloping, pinnacled, dry-docked, blocky, weathered or glacier bergs in addition to having a size qualifier. |  |  |  |
| 4.3.7.1 | Glacier berg: Cf. 10.4.2.1 – An irregularly shaped iceberg |  |  |  |
| 4.3.7.2 | Tabular berg: Cf. 10.4.2.2 – A flat-topped iceberg. Most tabular bergs form by calving from an ice shelf and show horizontal banding (cf. Ice island). |  |  |  |
| 4.3.7.3 | Domed Iceberg: An iceberg which is smooth and rounded on top. |  |  |  |
| 4.3.7.4 | Sloping Iceberg: An iceberg which is rather flat on top and with steep vertical sides on one end, sloping to lesser sides on the other end. |  |  |  |
| 4.3.7.5 | Pinnacled Iceberg: An iceberg with a central spire or pyramid, with one or more spires. |  |  |  |
| 4.3.7.6 | Dry-docked Iceberg: An iceberg which is eroded such that a U-shaped slot is formed near or at water level, with twin columns or pinnacles. |  |  |  |
| 4.3.7.7 | Blocky Iceberg: A flat-topped iceberg with steep vertical sides. |  |  |  |
| 4.3.7.8 | Weathered Iceberg: An iceberg that shows marked signs of deterioration from the effects of atmosphere and ocean. |  |  |  |
| 4.3.7.9 | Ice island: Cf. 10.4.3 - A large piece of floating ice protruding about 5 m above sea-level, which has broken away from an Arctic ice shelf, having a thickness of 30-50 m and an area of from a few thousand m² to 500 km² or more, and usually characterized by a regularly undulating surface which gives it a ribbed appearance from the air. This term is not commonly used in the Antarctic, where the term very large tabular iceberg describes the same phenomenon. |  |  |  |
| 4.3.7.10 | Ice Island Fragment: Piece of an ice island that has broken away from the main mass. |  |  |  |
| 4.3.7.11 | Very Large Iceberg: A piece of glacier ice extending more than 75 m above sea level and with a length of more than 200 m. |  |  |  |
| 4.3.7.12 | Large Iceberg: A piece of glacier ice extending 46 to 75 m above sea level and with a length of 121 to 200 m. |  |  |  |
| 4.3.7.13 | Medium Iceberg: A piece of glacier ice extending 16 to 45 m above sea level and with a length of 61 to 120 m. |  |  |  |
| 4.3.7.14 | Small Iceberg: A piece of glacier ice extending 5 to 15 m above sea level and with a length of 15 to 60 m. |  |  |  |
| 4.3.7.14 | Bergy bit: Cf. 10.4.4 - A large piece of floating glacier ice, generally showing less than 5 m above sea-level but more than 1 m and normally about 100-300 m² in area. |  |  |  |
| 4.3.7.15 | Growler: Cf. 10.4.5 - Amended by ETSI-I (2001) to read: Piece of ice smaller than a bergy bit and floating less than 1 m above the sea surface, a growler generally appears white but sometimes transparent or blue-green in colour. Extending less than 1 m above the sea surface and normally occupying an area of about 20 m², growlers are difficult to distinguish when surrounded by sea ice or in high sea state. |  |  |  |

## Rationale

The International Ice Patrol is the most official source of iceberg information in the North Atlantic and has the largest base of clients for iceberg information.  The International Sea (Floating) Ice Nomenclature should reflect the established practice of the IIP by including all of the terminology in use.

Additionally, the current numbering of articles 4.3.7 – 4.3.12 is not consistent with other sections of the Nomenclature.  Article 4.3.7 is the general definition of “iceberg” and all of the specific types of icebergs should be sub-sections of 4.3.7.

# Ice Object ICEKEL

## Observation

There is no established WMO symbol to depict “Keel/Bummock”.

## Recommendation

It is recommended that the WMO symbol for Ridges/Hummocks be inverted, so the straight line representing the water surface is at the top - and used for Keels/Bummocks.

- C  - Concentration (areal coverage) in tenths

- f   - frequency in number per nautical mile (f is an alternative for C)

- h  - mean depth expressed in decimetres

- hx - maximum depth expressed in decimetres

Note that the parameter “a” is omitted because there is no meaningful keel “classification” equivalent to ridge classification.  Keels “weather” and consolidate too quickly.

## Rationale

Although not in wide public use, this symbol should be include for completeness and application in the submarine community.

# Ice Object ICEFRA

## Observation

There is no established WMO symbol to depict “Fracture”.

## Recommendation

It is recommended that the existing symbols for “crack” be used to depict any “fracture”.

Fracture (symbol for a fracture at a specific location)

Fracture (symbol indicating presence of fractures in the area)

## Rationale

The Nomenclature uses the term “fracture” (Ref: 7.1) for any generic break or rupture in very close pack ice, compact pack ice consolidated pack ice, fast ice or a single floe.  The term “crack” (Ref: 7.1.1) is specific type of fracture of width less than 1 metre.  For navigation purposes, there seems to be little rationale for distinguishing cracks as distinct from other fractures.

There is already a separate symbol for “lead” (Ref: 7.3) which is a navigationally important type of fracture.

Two classes of symbols, one for generic fractures and one for leads, would seem to be sufficient for portraying openings in the ice for navigational purposes.

# Ice Attribute ICEACT

## Observation

The codes for ICEACT are not fully in alignment with SIGRID-3 codes. Neither the code set for ICEACT nor the SIGRID-3 code set supports the North American use of “9+ tenths” concentration.

## Recommendation

It is recommended that the definition of code 91 in ICEACT be changed to “9/10-10/10 or “9+/10”.

It is also recommended that the SIGRID-3 Code Table 4.1 be amended as follows:

- Change the code for “Ice Free” from “00” to “98” (The purpose for this change is that the ECDIS standard discourages the use of “00” since it is reserved for special situations.)

- Change the definition of code value “99” from “unknown” to “undetermined /unknown” for consistency both internally in SIGRID-3and with ICEACT

## Rationale

“9+ tenths” concentration is defined and supported by the WMO Sea Ice Nomenclature and has been in use on ice charts for a long time.  It represents a navigationally important category of ice concentration – almost completely ice covered but not consolidated.  ETSI-III approved an amendment to the SIGRID-3 Table 4.1 to support its use and it should be continued for ENC ice information.

The code “00” is generally discouraged in S-57 because it is reserved for special situations that are not applicable to ice charts.  The proposed change to an unused code number has the advantage of being backwards compatible with existing SIGRID-3 files.  Software to read SIGRID-3 can treat “00” and the new code as both meaning the same thing.  Software to produce SIGRID-3 should be changed to use only the new code.

# Ice Attribute ICEAPC

## Observation

The codes for ICEAPC do not support the North American use of “9+ tenths” concentration.

## Recommendation

It is recommended that, for consistency, with SIGRID-3 and the above recommended change to ICEACT, the definition of code 91 in ICEAPC be changed to “9/10-10/10 or “9+/10”.

## Rationale

This change is necessary for compatibility if the change to ICEACT is accepted.

# Ice Attribute ICESOD

## Observation

The codes for ICESOD are not fully in alignment with JCOMM SIGRID-3 codes and do not reflect the changes to SIGRID-3 adopted at ETSI-III regarding “brash ice”.

## Recommendation

It is recommended to add an ID of 70 to the expected input for ICESOD with a meaning of “Brash Ice”.

It is recommended to add a new Ice Attribute, ICEBRS, to describe the concentrations of Very Thick Brash Ice (>4 m), Thick Brash Ice (>2 - 4 m), Medium Brash Ice (1 – 2 m) and Thin Brash Ice (<1 m) respectively.  The expected input for these four new attributes will be similar to ice attribute ICEACT.

It is recommended that the term “windrow” be added to the Sea Ice Nomenclature as article 4.4.8.1.2 with a meaning the same as 4.4.8.1.1 Jammed brash barrier and a note that this term is used in the Baltic Sea.  The symbol for “jammed brash barrier” be associated with “windrow” as well.

It is recommended that the SIGRID-3 Code Table 4.2 be amended as follows:

- Change the code for “ice free” to “01” (the ECDIS standard discourages the use of “00”).

- In order to remove ambiguities in range values, add “<” and “>” symbols as appropriate to conform to the definitions for ICESOD.

- Add a thickness range for “New Ice”.

- Remove the upper thickness limit of 200 cm for First Year Ice (Code 86).  Although First Year Ice usually never exceeds 200cm, there is no physical reason that it could not.  Removing the upper limit is consistent with the existing definition of Thick First Year Ice (Code 93).

The revised Table 4.2 would look like:

Table 4.2 Thickness of ice or stage of development codes for variable identifiers SA, SB, SC, CN, and CD.

| Stage of Development | Thickness | Code Figure |
| --- | --- | --- |
| Ice Free |  | 01 |
| Brash Ice | Given by AV, AK, AM, AT in Table 3.3 | 70 |
| No Stage of Development |  | 80 |
| New Ice | < 10 cm | 81 |
| Nilas, Ice Rind | < 10 cm | 82 |
| Young Ice | 10 - <30 cm | 83 |
| Grey Ice | 10 - <15 cm | 84 |
| Grey - White Ice | 15 - <30 cm | 85 |
| First Year Ice |  30 cm | 86 |
| Thin First Year Ice | 30 - <70 cm | 87 |
| Thin First Year Stage 1 | 30 - <50 cm | 88 |
| Thin First Year Stage 2 | 50 - <70 cm | 89 |
| For Later Use |  | 90 |
| Medium First Year Ice | 70 - <120 cm | 91 |
| For Later Use |  | 92 |
| Thick First Year Ice |  120 cm | 93 |
| For Later Use |  | 94 |
| Old Ice |  | 95 |
| Second Year Ice |  | 96 |
| Multi-Year Ice |  | 97 |
| Glacier Ice |  | 98 |
| Undetermined/Unknown |  | 99 |

## Rationale

Brash Ice is a common occurrence in Canadian waters and is regularly reported on ice observation charts.  ETSI-III approved the inclusion of Brash Ice in SIGRID-3 along with an encoding for its use.  The Ice Object Catalogue should be amended to include Brash Ice for compatibility so that this phenomenon can be displayed on ENCs.

During previous work on standards, it was noted that the “jammed brash barrier” symbol was used on some Baltic Sea ice charts but was termed “windrow”.

Changes to the SIGRID-3 code table are primarily to overcome ambiguities in the definitions and to remove use of the code “00” for reasons described elsewhere.

# Ice Attribute ICELSO

## Observation

The codes for ICELSO do not support “brash ice”.

## Recommendation

It is recommended to add an ID of 70 to the expected input for ICELSO with a meaning of “Brash Ice”.

The proposed new Ice Attributes, ICEBAV, ICEBAK, ICEBAM and ICEBAT (See ICESOD) would also apply to ICELSO.

## Rationale

For consistency with the practice adopted for brash sea ice, brash lake ice is also commonly observed.

# Ice Attribute ICEFLZ

## Observation

The International System of Sea Ice Symbols (Ref: WMO Sea Ice Nomenclature No. 259 Supplement No. 4) provides two variants to describe the forms of ice.  The first variant uses Fa, Fb and Fc corresponding to the three stages of development Sa, Sb, and Sc. The second variant uses the parameters Fp and Fs to describe the predominant and secondary floe sizes, independent from the stages of development.  This practice was originally carried into the SIGRID-3 encoding through the use of FA, FB and FC or, alternatively, the parameter CF.  ETSI-III recognized the possibility of this confusion and approved the removal of the parameter CF from the SIGRID-3 code.  However, the Sea Ice Nomenclature was not amended at the same time.

## Recommendation

It is proposed to discontinue use of the second variant to describe the forms of ice.

The parameters Fp and Fs would be removed from the System of Sea Ice Symbols.

## Rationale

The dual, alternative uses to describe the forms of ice can, and have, led to considerable confusion.  As a result, ETSI-III agreed to remove it from the SIGRID-3 code.  It is believed that the second variant, Fp and Fs, is not in widespread use.  It is therefore proposed to eliminate this permitted used.

# Ice Attribute ICEFLZ

## Observation

The codes for ICEFLZ are not fully in alignment with SIGRID-3 codes.

## Recommendation

It is recommended that the SIGRID-3 Code Table 4.3 be amended as follows:

- Change the code for “Pancake ice” to “22” (the ECDIS standard discourages the use of “00”).

- In order to remove ambiguities in range values, add “<” and “>” symbols as appropriate to conform to the definitions above.

- Remove the reference to CF from the heading of Table 4.3 (this is a housekeeping item since the parameter CF was removed from the code at ETSI-III)

The revised Table 4.3 would look like:

Table 4.3 Form of ice codes for variable identifiers FA, FB, FC(, and CF).

| Form | Size/Concentration | Code Figure |
| --- | --- | --- |
| Pancake Ice | 30 cm - 3 m | 22 |
| Shuga/Small Ice Cake, Brash Ice | < 2 m across | 01 |
| Ice Cake | < 20 m across | 02 |
| Small Floe | 20 m - <100 m across | 03 |
| Medium Floe | 100 m - <500 m across | 04 |
| Big Floe | 500 m - <2 km across | 05 |
| Vast Floe | 2 km - <10 km across | 06 |
| Giant Floe |  10 km across | 07 |
| Fast Ice |  | 08 |
| Growlers, Floebergs or Floebiits |  | 09 |
| Icebergs |  | 10 |
| Strips and Patches | concentrations 1/10 | 11 |
| Strips and Patches | concentrations 2/10 | 12 |
| Strips and Patches | concentrations 3/10 | 13 |
| Strips and Patches | concentrations 4/10 | 14 |
| Strips and Patches | concentrations 5/10 | 15 |
| Strips and Patches | concentrations 6/10 | 16 |
| Strips and Patches | concentrations 7/10 | 17 |
| Strips and Patches | concentrations 8/10 | 18 |
| Strips and Patches | concentrations 9/10 | 19 |
| Strips and Patches | concentrations 9+/10 | 91 |
| Strips and Patches | concentrations 10/10 | 20 |
| Level Ice |  | 21 |
| Undetermined/Unknown |  | 99 |

## Rationale

Changes to the SIGRID-3 code table are primarily to overcome ambiguities in the definitions and to remove use of the code “00” for reasons described elsewhere.

# Ice Attribute ICEMLT

## Observation

The codes for ICEMLT are not fully in alignment with JCOMM SIGRID-3 codes.

## Recommendation

It is recommended that the SIGRID-3 Code Table 4.11 be amended as follows:

- Change the code of “No melt” to “01” (the ECDIS standard discourages the use of “00”).

- An additional term for “many puddles” be added with Code “2”.  (this omission was likely inadvertent in the initial version of SIGRID-3).

## Rationale

These changes are to eliminate the use of “00” and to correct an assumed oversight in the original definition.