- World Meteorological Organization

Weather Overlay Product Specification

- December 2013

- Published by the

- World Meteorological Organization

- Geneva

| Version Number | Date | Author | Purpose |
| --- | --- | --- | --- |
| 0.0.1 | August 13, 2013 | J.Powell | Initial Draft |
| 0.0.2 | December 2, 2013 | C. Schultz | Updated draft for review by ETMSS |

Contents	Page

1	Overview	5

1.1	Introduction	5

1.2	References	5

1.3	Terms, definitions and abbreviations	5

1.3.1	Use of Language	5

1.3.2	Terms and Definitions	5

1.3.3	Abbreviations	5

1.4	General Data Product Description	5

1.5	Data product specification metadata	6

1.5.1	IHO Product Specification Maintenance	6

2	Specification Scopes	7

3	Dataset Identification	8

4	Data Content and structure	8

4.1	Introduction	8

4.2	Application Schema	8

4.3	Feature Catalogue	9

4.3.1	Introduction	9

4.3.2	Feature Types	9

4.3.3	Feature Relationship	9

4.3.4	Information Types	9

4.3.5	Attributes	9

4.4	Dataset Types	10

4.4.1	Introduction	10

4.5	Dataset Loading and Unloading	10

4.6	Geometry	10

5	Coordinate Reference Systems (CRS)	11

5.1	Introduction	11

6	Data Quality	11

7	Data Capture and Classification	11

8	Maintenance	11

9	Portrayal	11

10	Data Product format (encoding)	12

10.1	Introduction	12

11	Data Product Delivery	12

11.1	Introduction	12

11.2	Dataset	12

11.2.1	Datasets	12

11.2.2	Dataset file naming	12

11.3	Support Files	12

11.3.1	Support File Naming	12

11.4	Exchange Catalogue	12

12	Metadata	14

12.1	Introduction	14

12.2	Language	14

Annex A - Data Classification and Encoding Guide	15

Annex C – Normative Implementation Guidance	15

Annex D – Feature Catalogue	15

# Overview

This document has been produced by the Joint WMO-IOC Technical Commission for Oceanography and Marine Meteorology (JCOMM) Expert Team on Maritime Safety Services (ETMSS) to produce a data product that can be used as a Nautical Information Publication Overlay (NPIO) within an Electronic Chart Display and Information System (ECDIS). It is based on the IHO S-100 framework specification and the ISO 19100 series of standards. It is a vector product specification that is primarily intended for encoding maritime weather and sea condition analyses and forecasts for situational awareness and planning purposes.

As part of the International Maritime Organization’s (IMO) Global Maritime Distress and Safety System (GMDSS) and International Convention for the Safety of Life at Sea (SOLAS), the WMO Marine Programme broadcasts meteorological information and warnings to mariners.  The world’s oceans are divided into sixteen metareas, or areas of responsibility of dissemination for meteorological information.  Each metarea has one or more countries that are responsible for issuing and disseminating weather bulletins and warnings within the defined metarea location.

This action is intended to define an internationally accepted feature catalogue that models real-world weather and oceanographic phenomena and can be used to create ECDIS-compatible maritime weather charts.  These weather elements will be used by mariners for route-planning and hazard mitigation.

## Scope

This document describes an S-100 compliant product specification for Weather Overlay datasets, which will primarily act as an overlay for S-101 Electronic Navigational Charts for an S-100 based ECDIS, in order to provide the maritime community with greater situational awareness. It specifies the content, structure, and metadata needed for creating a fully compliant Weather Overlay dataset and for its portrayal within an S-100 ECDIS.  This product specification includes the content model, the encoding, the feature catalogue, portrayal catalogue and metadata.

## References

S-100		IHO Universal Hydrographic Data Model

## Terms, definitions and abbreviations

### Use of Language

Within this document:

- “Must” indicates a mandatory requirement.

- “Should” indicates an optional requirement, that is the recommended process to be followed, but is not mandatory.

- “May” means “allowed to” or “could possibly”, and is not mandatory.

### Terms and Definitions

Dataset

Define here not in bold on a new line.

Metarea

### Abbreviations

CRS		Coordinate Reference System

ECDIS		Electronic Chart Display Information System

EPSG		European Petroleum Survey Group

ENC		Electronic Navigational Chart

ETMSS	Expert Team on Maritime Safety Services

GMDSS	Global Maritime Distress and Safety System

IHO		International Hydrographic Organization

IMO		International Maritime Organization

IOC		Intergovernmental Oceanographic Commission

ISO		International Organization for Standardization

JCOMM 	Joint WMO/IOC Commission for Oceanography and Marine Meteorology

SOLAS		International Convention for Safety of Life at Sea

WMO 		World Meteorological Organization

WXO		Weather Overlay

## Weather Overlay Data Product Description

Title:	Weather Overlay

Abstract: 		This data product describes real-world weather and oceanographic phenomena and can be used to create ECDIS-compatible maritime weather analysis and forecast charts.  These weather elements will be used by mariners for route-planning and hazard mitigation.

Content: 	The Product Specification defines all requirements to which Weather Overlay data products must conform.  Specifically it defines the data product content in terms of features and attributes within the feature catalogue. The display of features is defined by the symbols and rule sets contained in the portrayal catalogue. The Data Classification and Encoding Guide (DCEG) provides guidance on how data product content must be captured. (Annex A)

Spatial Extent:

Description: Areas specific to marine navigation

East Bounding Longitude: 180°

West Bounding Longitude: -180°

North Bounding Latitude: 90°

South Bounding Latitude: -90°

Purpose: 	The purpose of a Weather Overlay dataset is to provide official ocean forecast data to act as an overlay to Electronic Navigational Charts to assist in situational awareness and decision-making for the mariner.

## Data product specification metadata

Title:

S-100 Version:	1.0.0

S-10n Version: 	0.0.1

Date:				September 2013

Language: 		English

Classification: 	Unclassified

Contact: 			World Meteorological Organization

7bis, avenue de la Paix

Case postale 2300

CH -1211 Geneva 2

Switzerland

URL: 				www.wmo.int

Identifier: 		S-10n

Maintenance:		Changes to this product specification are coordinated by the WMO Joint WMO/IOC Commission for Oceanography and Marine Meteorology (JCOMM) Expert Team on Maritime Safety Services (ETMSS).

### Product Specification Maintenance

#### Introduction

Changes to the Weather Overlay will be released by the WMO as a new edition, revision, or clarification.

#### New Edition

New Editions of the Weather Overlay introduce significant changes. New Editions enable new concepts, such as the ability to support new functions or applications, or the introduction of new constructs or data types. New Editions are likely to have a significant impact on either existing users or future users of S-10n.

#### Revisions

Revisions are defined as substantive semantic changes. Typically, revisions will correct factual errors; introduce necessary changes that have become evident as a result of practical experience or changing circumstances. A revision must not be classified as a clarification. Revisions could have an impact on either existing users or future users of this specification. All cumulative clarifications must be included with the release of approved corrections revisions.

Changes in a revision are minor and ensure backward compatibility with the previous versions within the same Edition. Newer revisions, for example, introduce new features and attributes. Within the same Edition, a dataset of one version could always be processed with a later version of the feature and portrayal catalogues.

In most cases a new feature or portrayal catalogue will result in a revision of this product specification.

#### Clarification

Clarifications are non-substantive changes. Typically, clarifications: remove ambiguity; correct grammatical and spelling errors; amend or update cross references; insert improved graphics in spelling, punctuation and grammar. A clarification must not cause any substantive semantic change to the product specification.

Changes in a clarification are minor and ensure backward compatibility with the previous versions within the same Edition.  Within the same Edition, a dataset of one clarification version could always be processed with a later version of the feature and portrayal catalogues, and a portrayal catalogue can always rely on earlier versions of the feature catalogues.

Changes in a clarification are minor and ensure backward compatibility with the previous versions

#### Version Numbers

The associated version control numbering to identify changes (n) to the product specification must be as follows:

New Editions denoted as n.0.0

Revisions denoted as n.n.0

Clarifications denoted as n.n.n

# Specification Scopes

Scope ID: 		Root Scope

Level:			Dataset

Level name:	Weather Overlay Dataset

# Dataset Identification

Title: 									Weather Overlay

Alternate Title:						WXO

Abstract: 	Weather Overlay datasets must be produced in accordance with the rules defined in this product specification.  This product specification contains all the information necessary to enable responsible meteorological organizations to produce a consistent Weather Overlay, and manufactures to use that data efficiently in an ECDIS.

Topic Category:	climatologyMeteorologyAtmosphere

(ISO 19115 Domain Code 004)

oceans (ISO 19115 Domain Code 014)

Geographic Description:	Areas specific to ocean forecasts

Spatial Resolution:	Weather Overlay datasets must carry a display scale that is defined by this specification:

Display scale:	1:700,000

Purpose:	The Weather Overlay dataset is for primary use in Electronic Chart Display and Information Systems (ECDIS)

Language: 							English (Mandatory), other (Optional)

Classification: 						Data can be classified as one of the following:

Unclassified

Restricted

Confidential

Secret

Top Secret

Spatial Representation Type: 	Vector

Point of Contact: 					 Producing Agency

Use Limitation: 	This dataset is not to be used for land-based or aviation weather forecast purposes.

# Data Content and structure

## Introduction

The Weather Overlay is a feature-based product.  The content information is described in terms of a general feature model and a feature catalogue.

## Application Schema

<Normally, the full application schema is described in this section.  It can be described using UML, however, for specifications that have large application schemas (such as S-101) it can also be realised in the feature catalogue and the product specification can contain specific examples.>

The Weather Overlay conforms to the General Feature Model (GFM) from S-100 Part 3.  The GFM is the conceptual model and the implementation is defined in the Feature Catalogue.  The Weather Overlay Application Schema is realised in the feature catalogue and the product specification only contains specific examples.

## Feature Catalogue

### Introduction

The Weather Overlay Feature Catalogue describes the feature types, information types, attributes, and attribute values which may be used in a WXO dataset.

The Weather Overlay Feature Catalogue is available in an XML document which conforms to the S-100 XML Feature Catalogue Schema and can be downloaded from the JCOMM. It is also be available in a human readable version.

### Feature Types

#### Geographic

Geographic (geo) feature types form the principle content of the dataset and are fully defined by their associated attributes and information types.

#### Meta

Meta features contain information about other features within a data set. Information defined by meta features override the default metadata values defined by the data set descriptive records.

Meta features must be used to their maximum extent to reduce meta attribution on individual features.

### Attributes

The following clauses specify the different types of attributes that may be used in a product specification.  They may be either simple or complex.

#### Simple Attributes

The following table lists the types of attributes that are used in the WXO

| Type | Definition |
| --- | --- |
| Enumeration | A fixed list of valid identifiers of named literal values |
| Real | A signed Real (floating point) number consisting of a mantissa and an exponent |
| Integer | A signed integer number. The representation of an integer is encapsulation and usage dependent. |
| CharacterString | An arbitrary-length sequence of characters including accents and special characters from a repertoire of one of the adopted character sets |
| Date and Time | A DateTime is a combination of a date and a time type. Character encoding of a DateTime shall follow ISO 8601:1988 EXAMPLE 19850412T101530 |

#### Complex Attributes

Complex attributes are aggregations of other attributes that are either simple or complex. The aggregation is defined by means of attribute bindings.

Figure 1 	In this example Direction has two sub attributes. The Surface Wind Direction Feature may optionally include one instance of the complex attribute Direction.

## Dataset Types

### Introduction

A Data Set is a grouping of features, attributes, geometry and metadata which comprises a specific coverage. A data set can contain only one DataCoverage. The data boundary is defined by the extent of the DataCoverage features and must be contained within the boundingBox.  Data sets and DataCoverage features within these data sets may overlap.

In order to facilitate the efficient processing of Weather Overlay data the geographic coverage of a given maximum display scale must be split into data sets. Each data set must be contained in a physically separate, uniquely identified file on the transfer medium.

Datasets may cross the 180° meridian; this includes both the DataCoverage features and the boundingBox.

## Dataset Loading and Unloading

< TSMAD to provide some guidance on how non ENC data should load and unload in relation to ENC data.>

## Geometry

### S-100 Level 3a Geometry

The underlying geometry of a Weather Overlay is constrained to level 3a which supports 0, 1 and 2 dimensional objects (points, curves and surfaces) as defined by S-100 Part 7 – Spatial Schema.

Level 3a is described by the following constraints:

- Each curve must reference a start and end point (they may be the same).

- Curves must not self-intersect. See Figure 2.

- Areas are represented by a closed loop of curves beginning and ending at a common point.

- In the case of areas with holes, all internal boundaries must be completely contained within the external boundary and the internal boundaries must not intersect each other or the external boundary. Internal boundaries may touch tangentially (i.e. at one point).  See Figure 3.

- The outer boundary of a surface must be in a clockwise direction (surface to the right of the curve) and the curve orientation positive. The inner boundary of a surface must be in a counter-clockwise direction (surface to the right of the curve) and the curve orientation negative. See Figure 4.

Weather Overlay further constrains Level 3a with the following:

- Coincident linear geometry must be avoided when there is a dependency between features.

- The interpolation of GM_CurveSegment must be loxodromic.

- Linear features must not be encoded with a distance between two consecutive vertices which is smaller than 0.3mm at maximum display scale.

Figure 2 - Self Intersect Example

Figure 3 - Area Holes

Figure 4 - Boundary Direction

### Masking

In certain circumstances, the symbolisation of an edge may need to be suppressed. This is done using the Masked Spatial Type [MASK] field of the Feature Type record. The Mask Update Instruction [MUIN] must be set to {1} and Referenced Record name [RRNM] and Referenced Record identifier [RRID] fields must be populated with the values of the referenced spatial record. The Mask Indicator [MIND] must be set to either {1} or {2}  (see Annex B – clause B1.5.13)

DepthArea

Rivers

LandArea

LandArea

Figure 5 - Example without Masking

Rivers

DepthArea

LandArea

LandArea

Masked Curve

Figure 6 - Example of masked edge between Rivers and Depth Area features, where the River should be masked.  In this example MIND is set to {2} – supress portrayal

# Coordinate Reference Systems (CRS)

## Introduction

The coordinate reference system used for this product specification is World Geodetic System 1984 (WGS 84) which is defined by the European Petroleum Survey Group (EPSG) code 4326, (or similar - North American Datum 1983 / Canadian Spatial Reference System).

Spatial data are expressed as latitude (φ) and longitude (λ) geographic coordinates. Latitude values are stored as a negative number to represent a position south of the Equator. Longitude values are stored as a negative number to represent a position west of the Prime Meridian. Coordinates are expressed as real value, degree / degree decimal format. Datasets conforming to this product specification are not projected.

Horizontal coordinate reference system: 	EPSG:4326 (WGS84)

Vertical coordinate reference system:

Pressure

Projection: 								None

Temporal reference system: 			Gregorian calendar

Coordinate reference system registry: 	EPSG Geodetic Parameter Registry

Date type (according to ISO 19115):  	002- publication

Responsible party:  						International Organisation of Oil and Gas Producers (OGP)

URL: 										http://www.ogp.org.uk/

# Data Quality

< The data quality overview element should include at least the intended purpose and statement of quality or lineage. Other data quality elements cover: completeness, logical consistency, positional accuracy, temporal accuracy, thematic accuracy, and anything specifically required for the product being specified.>

# Data Capture and Classification

The Weather Overlay Data Classification and Encoding Guide (DCEG) describes how data describing the real world should be captured using the types defined in the Weather Overlay Feature Catalogue.  This Guide is located in Annex A.

# Maintenance

This clause describes the maintenance process for datasets, source, production process and how feature and portrayal catalogues are to be managed within an S-100 ECDIS.

## Maintenance and Update Frequency

Datasets are maintained as needed and must include mechanisms for updating the Weather Overlay. Datasets are updated temporally according to the internal policies and practices of issuing offices. Updates will be made by new editions.  Amendments and corrections will be produced as necessary.

## Data Source

Data Producers must use applicable sources to maintain and update data and provide a brief description of the sources that were used to produce the dataset.

## Feature and Portrayal Catalogue Management

For each new version of the Weather Overlay Product Specification a cumulative feature and portrayal catalogue will be released.  This will allow the ECDIS to only have to manage a single feature and portrayal catalogue that are compatible with different datasets that compliant to different versions of the product specification.

# Portrayal

| Item Name | Description | M/O | Card | type |
| --- | --- | --- | --- | --- |
| portrayalLibraryCitation | Bibliographic reference to the portrayal library | O | 0..1 | CI_Citation (ISO 19115) |

# Data Product format (encoding)

## Introduction

This clause specifies the encoding for Weather Overlay datasets.  See Annex B for a complete description of the data records, fields and subfields defined in the encoding.

Format Name:		ISO/IEC 8211

Character Set:	ISO 10646 Base Multilingual Plane

Specification:		S-100 profile of ISO/IEC 8211 (part 10A)

## Encoding of Latitude and Longitude

Coordinates are stored as integers.  Latitude and longitude are converted to integers using a multiplication factor held in the Data Set Structure Information field under [CMFX] and [CMFY] (see Annex B – clause B1.6.3).

These coordinate multiplication factors must be set to {10000000} (107) for all datasets.

EXAMPLE	A longitude = 42.0000 is converted into X = longitude * CMFX = 42.0000 * 10000000 = 	420000000.

## Numeric Attribute Encoding

Floating point or integer attribute values must not be padded by non-significant zeroes.

## Text Attribute Values

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8). A BOM (byte order mark) must not be used

## Mandatory Attribute Values

There are four reasons why attribute values may be considered mandatory:

- They determine whether a feature is in the display base,

- Certain features make no logical sense without specific attributes,

- Some attributes are necessary to determine which symbol is to be displayed,

All mandatory attributes are identified in the Feature Catalogue and summarised in Annex A – Data Classification and Encoding Guide.

## Missing Attribute Values

In a base data set, when an attribute code is present but the attribute value is missing, it means that the producer wishes to indicate that this attribute value is unknown.

# Data Product Delivery

## Introduction

Weather Overlay datasets are grouped into exchange sets. Each exchange set consists of one or more Weather Overlay datasets with an associated XML metadata file and a single Exchange Catalogue XML file containing metadata. It may also include one or more support files.

Units of Delivery:					Exchange Set

Transfer Size:						Unlimited

Medium Name:					Digital data delivery

Other Delivery Information:

Each exchange set has a single exchange catalogue which contains the discovery metadata for each dataset and references to any support files.

Support files are supplementary information which are linked to the features by the following fields within the dataset.

- textualDescription

An exchange set is encapsulated into a form suitable for transmission by a mapping called an encoding. An encoding translates each of the elements of the exchange set into a logical form suitable for writing to media and for transmission online. An encoding may also define other elements in addition to the exchange set contents (i.e media identification, data extents etc…) and also may define commercial constructs such as encryption and compression methods.

If the data are transformed, it must not be changed.

This product specification defines the encoding which must be used as a default for transmission of data between parties.

The encoding encapsulates exchange set elements as follows:

Mandatory Elements

- Weather Overlay datasets – ISO 8211 encoding of features/attributes and their associated geometry and metadata.

- Exchange Catalogue – the XML encoded representation of exchange set catalogue features [discovery metadata]. It also includes an additional file level CRC check per dataset.

Optional Elements

- Supplementary files – These are contained within the exchange set as files and the map from the name included within the dataset and the physical location on the media is defined within the Exchange Catalogue.

- Weather Overlay Feature Catalogue – If it is necessary to deliver the latest feature catalogue to the end user it may be done using the Weather Overlay exchange set mechanism for datasets

- Weather Overlay Portrayal Catalogue - If it is necessary to deliver the latest portrayal catalogue to the end user it may be done using the Weather Overlay exchange set mechanism for datasets.

## Dataset

### Datasets

One type of dataset files may be produced and contained within an exchange set:

- New dataset and new edition of a dataset: Including new information which has not been previously distributed by updates.  Each new edition of a data set must have the same name as the data set that it replaces.  A new edition can also be ENC data that has previously been produced for this area and at the same maximum display scale. The encoding structure is located in Annex B1.5

#### Dataset size

Datasets must not exceed 2MB

### Dataset file naming

New datasets will be named according to the following convention:

WXO_XXXX_CC_MM_YYYY.000

Where WXO indicates the dataset, XXXX is the metarea, CC is 2-letter country code, MM indicates the month with two digits, and YYYY is the year.

Each dataset will be issued through new editions for a period of one month.  At the beginning of the next calendar month, a cancellation will be issued and the new dataset issued.

### New Editions and Cancellations

This section defines the sequencing of Weather Overlay datasets for New Editions. In order to ensure that feature type updates are incorporated into an ECDIS in the correct sequence without any omission, a number of parameters encoded in the data are used in the following way:

edition number 	when a data set is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition.

issue date	date up to which the data producer has incorporated all applicable changes. The issue date must be greater than the previous issue date of the dataset.

issue time	time of day in UTC up to which the data producer has incorporated all applicable changes.

In order to cancel a data set, an update dataset file is created for which the edition number must be set to 0. This message is only used to cancel a base dataset file. Where a dataset is cancelled and its name is reused at a later date, the issue date must be greater than the issue date of the cancelled dataset.  When the dataset is cancelled it must be removed from the system.

An exchange set may contain base dataset files and update dataset files for the same datasets. Under these circumstances the update dataset files must follow on in the correct sequential order from the last update applied to the base dataset file.

The Weather Overlay datasets will be issued according to the following schedule:

| Month | Day | Editions Issued | File Name |
| --- | --- | --- | --- |
| 1 | 1 | 1, 2 | WXO_XII_2013_US_2013_01.000 |
| 1 | 2 | 3, 4 | WXO_XII_2013_US_2013_01.000 |
| 1 | 3 | 5, 6, 7 | WXO_XII_2013_US_2013_01.000 |
| 1 | … | … | WXO_XII_2013_US_2013_01.000 |
| 1 | 31 | 66, 67 | WXO_XII_2013_US_2013_01.000 |
| 2 | 1 | Cancellation | WXO_XII_2013_US_2013_01.000 |
| 2 | 1 | 1, 2 | WXO_XII_2013_US_2013_02.000 |
| 2 | 2 | 3, 4 | WXO_XII_2013_US_2013_02.000 |
| 2 | 3 | 5, 6 | WXO_XII_2013_US_2013_02.000 |
| 2 | … | … | WXO_XII_2013_US_2013_02.000 |
| 2 | 28 | 70, 71 | WXO_XII_2013_US_2013_02.000 |
| 2 | 1 | Cancellation | WXO_XII_2013_US_2013_02.000 |
| 3 | 1 | 1, 2 | WXO_XII_2013_US_2013_03.000 |
| 3 | … | … | WXO_XII_2013_US_2013_03.000 |

A new dataset will be issued on the first day of every month.  The editions will start at number 1 and

Increase sequentially with every dataset issuance.  Before a new Weather Overlay dataset is issued in the next month, a cancellation will be issued, cancelling the previous month’s dataset.

## Support Files

Data set support files offer supplementary information that can be included in an ENC exchange set.

- Text files must contain only general text as defined by this standard. (Extensible mark-up language (XML) supports UTF-8 character encoding). (TXT), (XML), (HTM)

| File Types | Extensions | Comment |
| --- | --- | --- |
| Text | TXT |  |
|  | HTM | HTML files must only include inline or embedded Cascading Style Sheet (CSS) information and must not embed Javascript or other dynamic content e.g. DHTML, Flash etc. |
|  | XML | XML documents must only be included in accordance with guidance provided within the Data Classification and Encoding Guide. This may include a schema for the validation of XML documents. |

## 11.3.1 Support File Naming

Supporting files will be named according to the following convention:

WXO_XXXX_CC_MM_YYYY_TTT

Where WXO indicates the dataset, XXXX is the metarea, CC is 2-letter country code, MM indicates the month with two digits, YYYY is the year, and TTT is the type of file (ie: TXT, HTM, XML).

### Support File Management

When a support file is created or a subsequent version is issued it must carry an issue date and a CRC value calculated on the content.  These values are contained in the Support File Metadata as defined in clause 12.1.2 and must not change while the file is still current.

The type of support file is indicated in the “purpose” field of the discovery metadata.  Support files carrying the “deletion” flag may be removed from the ECDIS.  When a feature pointing to a text, picture or application file is deleted or updated so that it no longer references the file, the ECDIS software must check to see whether any other feature referenced the same file, before that file is deleted.

Support files should be stored in a separate folder within the exchange set.

## Exchange Catalogue

<Specify if the datasets will be part of an exchange catalogue>

The exchange catalogue acts as the table of contents for the exchange set. The catalogue file of the exchange set must be named CATALOG.WO. No other file in the exchange set may be named CATALOG.WO.  The contents of the exchange catalogue are described in Clause 12.

## Data integrity

### Weather Overlay data integrity measures

Where there is a high impact on the integrity of data as a result of data corruption, such as to Weather Overlay data, there is a need for a mechanism within the Weather Overlay data itself to ensure it has not changed during transmission/delivery. The mechanism chosen for this assurance is a Cyclic Redundancy Check (CRC). File integrity checks are based on the CRC-32 algorithm (a 32 bit Cyclic Redundancy Check algorithm) as defined in ANSI/IEEE Standard 802.3, the reference for which is given in clause 1.2.

### Processing

Encoding is defined by the following generating polynomial:

G(x) = x 32 + x 26 + x 23 + x 22 + x 16 + x 12 + x 11 + x 10 + x 8 + x 7 + x 5 + x 4 + x 2 + x + 1

Processing is applied to relevant files as they appear in the exchange set.

The CRC value of the file is defined by the following process:

1.	The first 32 bits of the data are complemented.

2.	The n bits of the data are then considered to be the coefficients of a polynomial M(x) of degree n-1.

3.	M(x) is multiplied by x32 and divided by G(x), producing a remainder R(x) of degree <31.

4.	The coefficients of R(x) are considered to be a 32-bit sequence.

5.	The bit sequence is complemented and the result is the CRC.

The hexadecimal format of CRCs are converted to ASCII characters and stored in the “Catalogue Directory” [CATD] field.

# Metadata

## Introduction

For information exchange, there are several categories of metadata required: metadata about the overall exchange catalogue, metadata about each of the datasets contained in the catalogue, and metadata about the support files that make up the package.

This clause defines the mandatory and optional metadata needed for Weather Overlay datasets and exchange sets.  In some cases the metadata may be repeated in a national language.  If this is the case it is noted in the Remarks column.

Figures 14 to 16 outline the overall concept of an S-101 exchange set for the interchange of geospatial data and its relevant metadata. Figure 1 depicts the realization of the ISO 19139 classes which form the foundation of the exchange set. The overall structure of S-101 metadata for exchange sets is modelled in Figure 15. More detailed information about the various classes is shown in Figure 16 and a textual description in the tables at clause 12.3.

The discovery metadata classes have numerous attributes which enable important information about the datasets and accompanying support files to be examined without the need to process the data, e.g. decrypt, decompress, load etc.  Other catalogues can be included in the exchange set in support of the datasets such as feature, portrayal, coordinate reference systems, code lists etc. The attribute “purpose” of the support file metadata provides a mechanism to update support files more easily.

Figure 1 Realization of the Exchange Set Classes

Figure 2 – S-101 ExchangeSet

Figure 3 Weather Overlay Exchange Set

Figure 17 Weather Overlay Exchange Set - Class Details

### Dataset Metadata

| Name | Multiplity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| WXO_DataSetDiscoveryMetadata | - |  |  |  |
| metadataFileIdentifier | 1 |  | CharacterString | The file name must be unique. (name of the metadata file) |
| metadataPointOfContact | 1 |  | CI_ResponsibleParty |  |
| metadataDateStamp | 1 |  | Date |  |
| metadataLanguage | 1 | English | CharacterString | All data sets conforming to this Product Specification must use the English language |
| fileName | 1 |  | CharacterString | Dataset file name (name of the actual file…) |
| filePath | 1 |  | CharacterString | Path to the dataset file, relative to the root directory of the exchange set. The location of the dataset file after the exchange set is unpacked into directory <EXCH_ROOT> will be: <EXCH_ROOT>/<filePath>/<fileName> |
| description | 1 |  | CharacterString | Short description of the area covered by dataset harbour or port name, between two named locations etc. NATIONAL LANGUAGE enabled |
| dataProtection | 1 |  | Boolean | Encrypted or Unencrypted |
| protectionScheme | 0..1 |  | CharacterString | e.g. S-63 |
| copyright | 0..* |  | MD_LegalConstraints ->MD_RestrictionCode <copyright> (ISO 19115) |  |
| classification | 1 | {1} to {5} | Class MD_SecurityConstraints>MD_ClassificationCode (codelist) | 1. unclassified 2. restricted 3. confidential 4. secret 5. top secret |
| purpose | 1 | {1} to {3} | CharacterString MD_Identification>purpose (character string) | 1. New Dataset 2. New Edition 3.Cancellation |
| specificUsage | 0..1 |  | CharacterString MD_USAGE>specificUsage (character string) MD_USAGE>userContactInfo (CI_ResponsibleParty) | Brief description of the resource and/or resource series usage |
| editionNumber | 1 |  | Integer | When a data set is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. Edition number remains the same for re-issue. |
| issueDate | 1 |  | Date | Date on which the data was made available by the data producer. |
| issueTime | 1 |  | Time | Time in UTC at which the data was made available by the data producer. |
| productSpecification | 1 | WXO Version X.X.X | Character String | This must be encoded as WXO X.X.X |
| producingAgency | 1 |  | CI_ResponsibleParty | Agency responsible for producing the data. |
| horizontalDatumReference | 1 | EPSG | CharacterString |  |
| horizontalDatumValue | 1 | 4326 | Integer | WGS84 |
| dataType | 1 | ISO 8211 BINARY | S100_DataFormat |  |
| otherDataTypeDescription | 0..1 |  | CharacterString |  |
| dataCoverage | 1 |  | WXO_DataCoverage | Provides information about data coverage within the dataset |
| checkSum | 1 |  | CharacterString NonNegativeInteger | Expressed in hex notation |

#### DataCoverage

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| WXO_DataCoverage | - | - | - | - |
| ID | 1 |  | Integer | Uniquely identifies the coverage |
| boundingPolygon | 1 |  | EX_BoundingPolygon |  |
| DisplayScale | 1 | 700,000 | Integer |  |

### Support File Metadata

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| WXO_SupportFileDiscoveryMetadata | - |  | - | - |
| fileName | 1 |  | CharacterString |  |
| filePath | 1 |  | CharacterString | Full location from the exchange set root directory |
| purpose | 1 | {1} to {3} | class S100_SupportFilePurpose | - New – A file which is new - Replacement – A file which replaces an existing file - Deletion – deletes an existing file |
| editionNumber | 1 |  | CharacterString | When a data set is initially created, the edition number 1 is assigned to it. The edition number is increased by 1 at each new edition. Edition number remains the same for a re-issue. |
| issueDate | 1 |  | Date | Date on which the data was made available by the data producer. |
| productSpecification | 1 |  | WXO_ProductSpecification | Version of WXO |
| dataType | 1 | {1} to {4} | class S100_SupportFileFormat | - TXT =Text files - XML = Text files |
| dataTypeVersion | 1 |  | CharacterString | The version number of the dataType |
| Comment | 0..1 |  | CharacterString | Any additional Information NATIONAL LANGUAGE enabled |
| checkSum | 1 |  | CharacterString |  |
| digitalSignatureReference | 0..1 |  | CharacterString | Reference to the appropriate digital signature algorithm |
| digitalSignatureValue | 0..1 |  | CharacterString |  |

### Exchange Catalogue File Metadata

The catalogue file is defined in XML schema language.  The Exchange catalogue inherits the dataset discovery metadata and support file discovery metadata.

| Name | Multiplicity | Value | Type | Remarks |
| --- | --- | --- | --- | --- |
| WXO_ExchangeCatalogue | - |  |  | An exchange catalogue contains the discovery metadata about the exchange datasets and support files |
| identifier | 1 |  | CharacterString S100_CatalogueIdentifier | Uniquely identifies this exchange catalogue |
| editionNumber | 1 |  | CharacterString | The edition number of this exchange catalogue |
| contact | 1 |  | S100_CataloguePointofContact CI_ResponsibleParty |  |
| date | 1 |  | Date | Creation date of the exchange catalogue |
| time | 1 |  | Time | Creation time in UTC of the exchange catalogue |
| MetadataLanguage | 1 | English | CharacterString | All data sets conforming to S-101 PS must use English language |
| exchangeCatalogueName | 1 | WEATHERCAT | CharacterString | Catalogue filename |
| exchangeCatalogueDescription | 1 |  | CharacterString | Description of what the exchange catalogue contains NATIONAL LANGUAGE enabled |
| productSpecification | 1 |  |  | WXO Version Number |
| exchangeCatalogueComment | 0..1 |  | CharacterString | Any additional Information NATIONAL LANGUAGE enabled |
| compressionFlag | 1 | {1} to {2} | CharacterString | - Yes - No |
| algorithmMethod | 0..1 | {1} to {2} | CharacterString | Conditional on if compresstionFlag is set to {1} - ZIP - RAR |
| replacedData | 1 |  |  | If a data file is cancelled is it replaced by another data file |
| dataReplacement | 0..1 |  |  | Dataset name |

## Language (S-57 PS 3.11)

The exchange language must be English. Other languages may be used as a supplementary option. National geographic names can be left in their original national language in the international attributes, or transliterated or transcribed and used in the international attributes.

Character strings must be encoded using the character set defined in ISO 10646-1, in Unicode Transformation Format-8 (UTF-8). A BOM (byte order mark) must not be used.

# Annex A - Data Classification and Encoding Guide

| IHO Definition: FEATURE: Definition. (Source of definition). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| S-10n Geo Feature: |  |  |  |  |  |
| Primitives: Point, Curve, Surface Allowable geometric primitive(s) |  |  |  |  |  |
| Real World Example if real world instance(s) of the Feature. | Paper Chart Symbol Example(s) of paper chart equivalent symbology for the Feature. |  | ECDIS Symbol Example(s) of ECDIS symbology for the Feature. |  |  |
| S-10n Attribute |  | Allowable Encoding Value * |  | Type | Multiplicity |
| Category of beer |  | 1 : ale 2 : lager 3 : porter 4 : stout 5 : pilsener |  | EN | 1,1 |
| This section lists the full list of allowable attributes for the S-10n feature. Attributes are listed in alphabetical order. Sub-attributes (Type prefix (S)) of complex (Type C) attributes are listed in alphabetical order and indented directly under the entry for the complex attribute (see below for example). |  | This section lists the allowable encoding values for S-10n (for enumerate (E) Type attributes only). Further information about the attribute is available in Section XX. |  | Attribute type (see clause X.X). | Multiplicity describes the “cardinality” of the attribute in regard to the feature. See clause X.X. |
| Fixed date range |  |  |  | C | 0,1 |
| Date end |  |  |  | (S) DA | 0,1 |
| Date start |  |  |  | (S) DA | 0,1 |
| INT 1 Reference: The INT 1 location(s) of the Feature – by INT1 Section and Section Number. X.X.X Sub-clause heading(s) (see S-4 – B-YYY.Y) Introductory remarks. Includes information regarding the real world entity/situation requiring the encoding of the Feature in the ENC, and where required nautical cartographic principles relevant to the Feature to aid the compiler in determining encoding requirements. Specific instructions to encode the feature. Remarks: - Additional encoding guidance relevant to the feature. X.X.X.X Sub-sub-clause heading(s) (see S-4 – B-CCC.C) Clauses related to specific encoding scenarios for the Feature. (Not required for all Features). Remarks: - Additional encoding guidance relevant to the scenario (only if required). Distinction: List of features in the Product Specification distinct from the Feature. |  |  |  |  |  |

ANNEX B - NORMATIVE

# Data Product format (encoding)

- Introduction

This Product Specification uses the S-100 8211 to encapsulate data.  This annex specifies the interchange format to facilitate the moving of files containing data records between computer systems.  It defines a specific structure which can be used to transmit files containing data type and data structures specific to S-101.

## Data set files

The order of data in each base or update dataset file is described below:

Data set file

Data set general information record

Data set structure information field structure

Data set Coordinate Reference System record structure

Information records

Information

Vector records

Point

Multi point

Curve

Composite Curve

Surface

Feature records

Meta features

Geo features

Aggregated features

Theme features

This order of records will enable the import software to check that the child record exists each time the parent record references it (i.e. it will already have read the child record so it will know if it exists or not).

## Records

Records and fields that do not appear in the following tree structure diagrams are prohibited. The order of records in the files must be the same as that described in these tree structure diagrams.

The combination of the file name and the “Name” of the record must provide a unique world-wide identifier of the record.

## Fields

For base dataset files, some fields may be repeated (indicated by <0..*> or <1..*>) and all of their content may be repeated (indicated by *). In order to reduce the volume of data, the encoder should repeat the sequence of subfields, in preference to creating several fields.

## Subfields

Mandatory subfields must be filled by a non-null value.

Prohibited subfields must be encoded as missing subfields values. The exact meaning of missing attribute values is defined in Annex A.

In the tables following the tree structure diagrams, prescribed values are indicated in the “values” column. The “comment” column contains general comments and an indication of whether the subfield is ASCII or binary coded.

When encoding new base data sets the record update instruction (RUIN) is always set to insert. When encoding updates it can be set to insert, modify or delete.

## Base dataset structure

NOTE:  The number contained in parenthesis () is the number of subfields that are contained in the field.

Base dataset file

|

|--<1>- Data Set General Information record

|   |

|   |--<1>-DSID (13\\*1): Data Set Identification field

|   |

|   |--<1>-DSSI (13): Data Set Structure Information field

|   |

|   |--<0..*>-ATTR (*5): Attribute field (Metadata)

|

|

|--<1>--Data Set Coordinate Reference System record

|   |

|   |--<1>-CSID (3): Coordinate Reference System Record Identifier field

|   |

|   |--<1..*>-CRSH (7): Coordinate Reference System Header field

|      |

|      |--<0..1>-CSAX (*2): Coordinate System Axes field

|      |

|      |--<0..1>-VDAT (4): Vertical Datum field

|

|

|--<0..*>--Information record

|   |

|   |--<1>-IRID (5): Information Type Record Identifier field

|      |

|      |--<0..*>- ATTR (*5): Attribute field

|      |

|      |--<0..*>- INAS (5\\*5): Information Association field

|

|

|--<0..*>-- Point record

|   |

|   |--<1>-PRID (4): Point Record Identifier field

|      |

|      |-<0..*>-INAS (5\\*5): Information Association field

|      |

|      | alternate coordinate representations

|      |

|      *-<1>-C2IT (2): 2-D Integer Coordinate Tuple field

|      |

|      *-<1>-C3IT (4): 3-D Integer Coordinate Tuple field

|

|

|--<0..*>-- Multi Point record

|   |

|   |--<1>-MRID (4): Multi Point Record Identifier field

|      |

|      |-<0..*>-INAS (5\\*5): Information Association field

|      |

|      | alternate coordinate representations

|      |

|      *-<0..*>-C2IL (*2): 2-D Integer Coordinate List field

|      |

|      *-<0..*>-C3IL (1\\*3): 3-D Integer Coordinate List field

|

|

|--<0..*>-- Curve record

|   |

|   |--<1>-CRID (4): Curve Record Identifier field

|      |

|      |-<0..*>-INAS (5\\*5): Information Association field

|      |

|      |-<1>-PTAS (*3): Point Association field

|      |

|      |-<1>-SEGH (1): Segment Header field

|          |

|          |-<1..*>-C2IL (*2): 2-D Integer Coordinate List field

|

|

|--<0..*>-- Composite Curve record

|   |

|   |--<1>-CCID (4): Composite Curve Record Identifier field

|      |

|      |-<0..*>-INAS (5\\*5): Information Association field

|      |

|      |-<0..*>-CUCO (*3): Curve Component field

|

|

|--<0..*>-- Surface record

|   |

|   |--<1>-SRID (4): Surface Record Identifier field

|      |

|      |-<0..*>-INAS (5\\*5): Information Association field

|      |

|      |-<1..*>-RIAS (*5): Ring Association Field

|

|

|--<0..*>-- Feature Type record

|

|--<1>-FRID (5): Feature Type Record Identifier field

|

|-<0..*>-ATTR (*5): Attribute field

|

|-<0..*>-INAS (5\\*5): Information Association field

|

|-<0..*>-SPAS (*6): Spatial Association field

|

|-<0..*>-FASC (5\\*5): Feature Association field

|

|-<0..*>-THAS (*3): Theme Association field

|

|-<0..*>-MASK (*4): Masked Spatial Type field

    - Field Content

### Data Set Identification field - DSID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {10} | b11 | {10} - Data Set Identification |
| Record identification number | RCID | {1} | b14 | Only one record |
| Encoding specification | ENSP | ‘S-100 Part 10a’ | A() | Encoding specification that defines the encoding |
| Encoding specification edition | ENED | “1.1” | A() | Edition of the encoding specification |
| Product identifier | PRSP | “INT.IHO.S-101.1.0” | A() | Unique identifier for the data product as specified in the product specification |
| Product edition | PRED | “1.0” | A() | Edition of the product specification |
| Application profile | PROF | “1” | A() | “1” – EN Profile |
| Dataset file identifier | DSNM |  | A() | The file name including the extension but excluding any path information |
| Dataset title | DSTL |  | A() | The title of the dataset |
| Dataset reference date | DSRD |  | A(8) | The reference date of the dataset Format: YYYYMMDD according to ISO 8601 |
| Dataset language | DSLG | “EN” | A() | The (primary) language used in this dataset |
| Dataset abstract | DSAB | omitted | A() | The abstract of the dataset |
| Dataset edition | DSED |  | A() | See clause ?? |
| Dataset topic category | *DSTC | {04}{14} | b11 | A set of topic categories |

### Data Set Structure Information field - DSSI

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Dataset Coordinate Origin X | DCOX | {0.0} | b48 | Shift used to adjust x-coordinate before encoding |
| Dataset Coordinate Origin Y | DCOY | {0.0} | b48 | Shift used to adjust y-coordinate before encoding |
| Dataset Coordinate Origin Z | DCOZ | {0.0} | b48 | Shift used to adjust z-coordinate before encoding |
| Coordinate multiplication factor for x-coordinate | CMFX | {107} | b14 | Floating point to integer multiplication factor for the x-coordinate or longitude |
| Coordinate multiplication factor for y-coordinate | CMFY | {107} | b14 | Floating point to integer multiplication factor for the y-coordinate or latitude |
| Coordinate multiplication factor for z-coordinate | CMFZ | {100} | b14 | Floating point to integer multiplication factor for the z-coordinate or depths or height |
| Number of Information Type records | NOIR |  | b14 | Number of information records in the data set |
| Number of Point records | NOPN |  | b14 | Number of point records in the data set |
| Number of Multi Point records | NOMN |  | b14 | Number of multi point records in the data set |
| Number of Curve records | NOCN |  | b14 | Number of curve records in the data set |
| Number of Composite Curve records | NOXN |  | b14 | Number of composite curve records in the data set |
| Number of Surface records | NOSN |  | b14 | Number of surface records in the data set |
| Number of Feature Type records | NOFR |  | b14 | Number of feature records in the data set |

### Attribute field - ATTR

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Attribute label/code | *ATLB |  | b12 | A valid attribute code |
| Attribute index | ATIX |  | b12 | Index (position) of the attribute in the sequence of attributes with the same code and the same parent (starting with 1). |
| Parent index | PAIX |  | b12 | Index (position) of the parent complex attribute within this ATTR field (starting with 1). If the attribute has no parent (top level attribute) the value is 0. |
| Attribute Instruction | ATIN | {1} | b11 | {1} - Insert |
| Attribute value | ATVL |  | A() | A string containing a valid value for the domain of the attribute specified by the subfields above. |

    - Information Association field - INAS

| Subfield name | Label | Value | Format | Subfield content and specification |
| --- | --- | --- | --- | --- |
| Referenced Record name | *RRNM | 150 | b11 | Record name of the referenced record |
| Referenced Record identifier | RRID |  | b14 | Record identifier of the referenced record |
| Information Association code | IASS |  | b12 | A valid code for the information association |
| Role code | ROLE |  | b12 | A valid code for the role |
| Information Association Update Instruction | IUIN |  | b11 | {1} - Insert {2} – Delete {3} - Modify |
| Attribute label/code | *ATLB |  | b12 | A valid attribute code |
| Attribute index | ATIX |  | b12 | Index (position) of the attribute in the sequence of attributes with the same code and the same parent (starting with 1). |
| Parent index | PAIX |  | b12 | Index (position) of the parent complex attribute within this INAS field (starting with 1). If the attribute has no parent (top level attribute) the value is 0. |
| Attribute Instruction | ATIN |  | b11 | {1} - Insert {2} - Delete {3} - Modify |
| Attribute value | ATVL |  | A() | A string containing a valid value for the domain of the attribute specified by the subfields above. |

    - Coordinate Reference System Record Identifier field - CSID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {15} | b11 | {15} - Coordinate Reference System Identifier |
| Record identification number | RCID | {1} | b14 | Only one record |
| Number of CRS Components | NCRC |  | b11 | {1} - Single CRS >{1} - Compound CRS |

    - Coordinate Reference System Header field - CRSH

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| CRS index | CRIX |  | b11 | 1 – for the horizontal CRS >1 – for the vertical CRS’s |
| CRS Type | CRST | {1} or {5} | b11 | {1} – 2D Geographic {5} - Vertical |
| Coordinate System Type | CSTY | {1} or {3} | b11 | {1} - Ellipsoidal CS {3} - Vertical CS |
| CRS Name | CRNM | “WGS84” for horizontal CRS “Depth - *” for vertical CRS where * is the name of the vertical datum | A() |  |
| CRS Identifier | CRSI | “4326” – for horizontal CRS “omitted for vertical CRS | A() |  |
| CRS Source | CRSS | {3} for horizontal CRS {255} for vertical CRS | b11 | {3} - EPSG {255} - Not Applicable |
| CRS Source Information | SCRI | omitted | A() |  |

    - Coordinate System Axes field - CSAX

This field is only used for vertical CRS.

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Axis Type | *AXTY | {12} | b11 | {12} – Gravity related depth (orientation down) |
| Axis Unit of Measure | AXUM | {4} | b11 | {4} - Metre |

    - Vertical Datum field – VDAT

- This field is only used for vertical CRS.

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Datum Name | DTNM |  | A() | Name of the enumeration value of the attribute VERDAT |
| Datum Identifier | DTID |  | A() | Enumeration value of the attribute VERDAT |
| Datum Source | DTSR | {2} | b11 | {2} - Feature Catalogue |
| Datum Source Information | SCRI | omitted | A() |  |

    - Information Type Identifier field - IRID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {150} | b11 | {150} - Information Type |
| Record identification number | RCID |  | b14 | Range: 1 to 232-2 |
| Object code | OBJC |  | b12 | A valid information type code from the FC |
| Record version | RVER |  | b12 | RVER contains the serial number of the record edition |
| Record update instruction | RUIN | {1} | b11 | {1} - Insert |

    - Point Record Identifier field - PRID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {110} | b11 | {110} - Point |
| Record identification number | RCID |  | b14 | Range: 1 to 232-2 |
| Record version | RVER |  | b12 | RVER contains the serial number of the record edition |
| Record update instruction | RUIN | {1} | b11 | {1} – Insert |

### 2-D Integer Coordinate Tuple field structure – C2IT

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Coordinate in Y axis | *YCOO |  | b24 | Y-coordinate or latitude |
| Coordinate in X axis | XCOO |  | b24 | X-coordinate or longitude |

### 3-D Integer Coordinate Tuple field structure– C3IT

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Vertical CRS Id | VCID |  | b11 | Internal identifier of the Vertical CRS |
| Coordinate in Y axis | *YCOO |  | b24 | Y- coordinate or latitude |
| Coordinate in X axis | XCOO |  | b24 | X- coordinate or longitude |
| Coordinate in Z axis | ZCOO |  | b24 | Z - coordinate (depth) |

    - Multi Point Record Identifier field - MRID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {115} | b11 | {115} - Multi Point |
| Record identification number | RCID |  | b14 | Range: 1 to 232-2 |
| Record version | RVER |  | b12 | RVER contains the serial number of the record edition |
| Record update instruction | RUIN | {1} | b11 | {1} - Insert |

    - 2-D Integer Coordinate List field structure – C2IL

| Subfield name | Label | Value | Format | Subfield content and specification |
| --- | --- | --- | --- | --- |
| Coordinate in Y axis | *YCOO |  | b24 | Y-coordinate or latitude |
| Coordinate in X axis | XCOO |  | b24 | X-coordinate or longitude |

    - 3-D Integer Coordinate List field structure – C3IL

| Subfield name | Label | Format | Subfield content and specification |
| --- | --- | --- | --- |
| Vertical CRS Id | VCID | b11 | Internal identifier of the Vertical CRS |
| Coordinate in Y axis | *YCOO | b24 | Y- coordinate or latitude |
| Coordinate in X axis | XCOO | b24 | X- coordinate or longitude |
| Coordinate in Z axis | ZCOO | b24 | Z - coordinate (depth or height) |

    - Curve Record Identifier field - CRID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {120} | b11 | {120} - Curve |
| Record identification number | RCID |  | b14 | Range: 1 to 232-2 |
| Record version | RVER |  | b12 | RVER contains the serial number of the record edition |
| Record update instruction | RUIN | {1} | b11 | {1} - Insert |

    - Point Association field - PTAS

| Subfield name |  | Label |  | Value |  | Format |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Referenced Record name | *RRNM |  |  |  | b11 |  | Record name of the referenced record |  |
| Referenced Record identifier | RRID |  |  |  | b14 |  | Record identifier of the referenced record |  |
| Topology indicator |  | TOPI |  |  |  | b11 |  | {1} - Beginning point {2} - End point {3} - Beginning & End point |

    - Segment Header field - SEGH

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Interpolation | INTP | {4} | b11 | {4} - Loxodromic |

    - Composite Curve Record Identifier field - CCID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {125} | b11 | {125} - Composite Curve |
| Record identification number | RCID |  | b14 | Range: 1 to 232-2 |
| Record version | RVER |  | b12 | RVER contains the serial number of the record edition |
| Record update instruction | RUIN | {1} | b11 | {1} - Insert |

    - Curve Component field - CUCO

| Subfield name |  | Label |  | Value |  | Format |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Referenced Record name | *RRNM |  |  |  | b11 |  | Record name of the referenced record |  |
| Referenced Record identifier | RRID |  |  |  | b14 |  | Record identifier of the referenced record |  |
| Orientation |  | ORNT |  |  |  | b11 |  | {1} - Forward {2} - Reverse |

    - Surface Record Identifier field - SRID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {130} | b11 | {130} - Surface |
| Record identification number | RCID |  | b14 | Range: 1 to 232-2 |
| Record version | RVER |  | b12 | RVER contains the serial number of the record edition |
| Record update instruction | RUIN | {1} | b11 | {1} – Insert |

    - Ring Association field - RIAS

| Subfield name |  | Label |  | Value |  | Format |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Referenced Record name | *RRNM |  |  |  | b11 |  | Record name of the referenced record |  |
| Referenced Record identifier | RRID |  |  |  | b14 |  | Record identifier of the referenced record |  |
| Orientation |  | ORNT |  |  |  | b11 |  | {1} - Forward {2} - Reverse |
| Usage indicator |  | USAG |  |  |  | b11 |  | {1} - Exterior {2} - Interior |
| Ring Association update instruction |  | RAUI |  | {1} |  | b11 |  | {1} – Insert |

### Feature Type Record Identifier field - FRID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {100} | b11 | {100} - Feature type |
| Record identification number | RCID |  | b14 | Range: 1 to 232-2 |
| Object code | OBJC |  | b12 | A valid feature type code from the FC |
| Record version | RVER |  | b12 | RVER contains the serial number of the record edition |
| Record update instruction | RUIN | {1} | b11 | {1} - Insert |

### Spatial Association field - SPAS

| Subfield name |  | Label |  | Value |  | Format |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Referenced Record name | *RRNM |  |  |  | b11 |  | Record name of the referenced record |  |
| Referenced Record identifier | RRID |  |  |  | b14 |  | Record identifier of the referenced record |  |
| Orientation |  | ORNT |  |  |  | b11 |  | {1} Forward {2} Reverse {255} NULL (Not Applicable) |
| Scale Minimum |  | SMIN |  |  |  | b14 |  | Denominator of the largest scale for which the feature type can be depicted by the referenced spatial object. If the value is 0 it does not apply. |
| Scale Maximum |  | SMAX |  |  |  | b14 |  | Denominator of the smallest scale for which the feature type can be depicted by the referenced spatial object. If the value is 232-1 it does not apply. |
| Spatial Association Update Instruction |  | SAUI |  | {1} |  | b11 |  | {1} - Insert |

### Feature Association field – FASC

| Subfield name |  | Label |  | Value |  | Format |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Referenced Record name | *RRNM |  |  |  | b11 |  | Record name of the referenced record |  |
| Referenced Record identifier | RRID |  |  |  | b14 |  | Record identifier of the referenced record |  |
| Feature Association Code |  | ASCD |  |  |  | b12 |  | A valid code for the feature association |
| Role Code |  | RLCD |  |  |  | b12 |  | A valid code for the role |
| Feature Association Update Instruction |  | FAUI |  | {1} |  | b11 |  | {1} - Insert |
| Attribute label/code |  | *ATLB |  |  |  | b12 |  | A valid attribute code |
| Attribute index |  | ATIX |  |  |  | b12 |  | Index (position) of the attribute in the sequence of attributes with the same code and the same parent (starting with 1). |
| Parent index |  | PAIX |  |  |  | b12 |  | Index (position) of the parent complex attribute within this FASC field (starting with 1). If the attribute has no parent (top level attribute) the value is 0. |
| Attribute Instruction |  | ATIN |  |  |  | b11 |  | {1} - Insert {2} - Delete {3} - Modify |
| Attribute value |  | ATVL |  |  |  | A() |  | A string containing a valid value for the domain of the attribute specified by the subfields above. |

### Theme Association field - THAS

| Subfield name |  | Label |  | Value |  | Format |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Referenced Record name | *RRNM |  |  |  | b11 |  | Record name of the referenced record |  |
| Referenced Record identifier | RRID |  |  |  | b14 |  | Record identifier of the referenced record |  |
| Theme Association Update Instruction |  | TAUI |  | {1} |  | b11 |  | {1} - Insert |

### Masked Spatial Type field - MASK

| Subfield name |  | Label |  | Value |  | Format |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Referenced Record name | *RRNM |  |  |  | b11 |  | Record name of the referenced record |  |
| Referenced Record identifier | RRID |  |  |  | b14 |  | Record identifier of the referenced record |  |
| Mask Indicator |  | MIND |  | {1} or {2} |  | b11 |  | {1} – Truncated by the dataset limit {2} – Supress portrayal |
| Mask Update Instruction |  | MUIN |  | {1} |  | b11 |  | {1} - Insert |

## Dataset cancellation structure

Dataset cancelation file

|

|--<1>- Data Set General Information record

|

|--<1>-DSID (13\\*1): Data Set Identification field

### Field Content

### Data Set Identification field - DSID

| Subfield name | Label | Value | Format | Comment |
| --- | --- | --- | --- | --- |
| Record name | RCNM | {10} | b11 | {10} - Data Set Identification |
| Record identification number | RCID | {1} | b14 | Only one record |
| Encoding specification | ENSP | ‘S-100 Part 10a’ | A() | Encoding specification that defines the encoding |
| Encoding specification edition | ENED | “1.1” | A() | Edition of the encoding specification |
| Product identifier | PRSP | “INT.IHO.S-101.1.0” | A() | Unique identifier for the data product as specified in the product specification |
| Product edition | PRED | “1.0” | A() | Edition of the product specification |
| Application profile | PROF | “2” | A() | “2” – ER Profile |
| Dataset file identifier | DSNM |  | A() | The file name including the extension but excluding any path information |
| Dataset title | DSTL |  | A() | The title of the dataset |
| Dataset reference date | DSRD |  | A(8) | The reference date of the dataset Format: YYYYMMDD according to ISO 8601 |
| Dataset language | DSLG | “EN” | A() | The (primary) language used in this dataset |
| Dataset abstract | DSAB | omitted | A() | The abstract of the dataset |
| Dataset edition | DSED | “0” | A() | 0 - indicates the cancelation |
| Dataset topic category | *DSTC | {14}{18} | b11 | A set of topic categories |

# Annex C – Normative Implementation Guidance

# Annex D – Feature Catalogue

Annex F – Portrayal Catalogue